The Minister for Ecological Transition

HAVING REGARD TO	Article 17(3) of Law No 400 of 23 August 1988;

HAVING REGARD TO	Article 184b of Legislative Decree No 152 of 3 April 2006, and, in particular, paragraph 2, which provides that ‘the criteria referred to in paragraph 1 shall be adopted in accordance with the provisions of the EU rules or, in the absence of EU criteria, on a case-by-case basis for specific types of waste through one or more decrees of the Minister for the Environment and the Protection of Land and Sea, pursuant to Article 17(3) of Law No 400 of 23 August 1988’;

HAVING REGARD TO	Directive 2008/98/EC of the European Parliament and of the Council of 19 November 2008, and in particular Article 11(1) thereof, which provides, inter alia, that Member States shall take measures to promote selective demolition in order to enable the safe removal and treatment of hazardous substances and to facilitate high-quality reuse and recycling by selectively removing materials;

HAVING REGARD TO	Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93 and Commission Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC;

HAVING REGARD TO	Regulation (EC) No 1221/2009 of the European Parliament and of the Council of 25 November 2009 allowing voluntary participation by organisations in an EU eco-management and audit scheme (EMAS), repealing Regulation (EC) No 761/2001 and Commission Decisions 2001/681/EC and 2006/193/EC;

HAVING REGARD TO	Legislative Decree No 82 of 7 March 2005 containing the ‘Digital Administration Code’;

HAVING REGARD TO	Presidential Decree No 445 of 28 December 2000 entitled ‘Consolidated text on the legislative and regulatory provisions relating to administrative documentation’;

HAVING REGARD TO	the Decree of the Minister for the Environment of 5 February 1998 on the identification of non-hazardous waste subject to simplified recovery procedures pursuant to Articles 31 and 33 of Legislative Decree No 22 of 5 February 1997, published in Official Gazette No 88 of 16 April 1998;

HAVING CONSIDERED	that there is a market for the recovered aggregate due to the fact that it is commonly used for the construction of civil engineering works, replacing the natural raw material, and has an actual economic value, that there are specific purposes for which the substance can be used, in accordance with the criteria set out in this Regulation, and that it complies with the legislation and the existing standards applicable to the products;

HAVING CONSIDERED	that the investigation carried out showed that the recovered aggregate, which meets the criteria set out in this Regulation, does not have an overall adverse impact on human health or the environment;

HAVING OBTAINED	 the opinion of the Council of State, expressed by the Consultative Section for legislative acts in the meeting of…………………….;

HAVING REGARD TO	the communication to the President of the Council of Ministers, carried out with Note ……., pursuant to Law No 400 of 23 August 1988;

HAVING REGARD TO	the communication referred to in Article 5 of Directive (EU) 2015/1535 laying down a procedure for the provision of information in the field of technical regulations and rules on Information Society services with Note …..

Hereby adopts

the following regulation

# Article 1

# Purpose and objectives

- This Regulation lays down the specific criteria under which inert waste from construction and demolition activities and other inert waste of mineral origin, as defined pursuant to points (a) and (b) of Article 2(1) of this Regulation, subject to recovery operations, cease to be classified as  waste pursuant to and for the purposes of Article 184b of Legislative Decree No 152 of 3 April 2006.

# Article 2

# Definitions

- For the purposes of this regulation, the definitions as per Article 183 of Legislative Decree No 152 of 3 April 2006 are applied, in addition to the following:

a) ‘inert waste from construction and demolition activities’: waste from construction and demolition operations identified in Chapter 17 of the European Waste List referred to in Decision 2000/532/EC and listed in point 1 of Table 1 of Annex 1 to this Regulation;

b) ‘other inert waste of mineral origin’: waste not belonging to Chapter 17 of the European Waste List referred to in Decision 2000/532/EC and listed in point 2 of Table 1 of Annex 1 to this Regulation;

c) ‘inert waste’: solid waste from construction and demolition activities and other inert waste of mineral origin that undergoes no significant physical, chemical or biological transformation; inert waste shall not dissolve, burn or be subjected to any other physical or chemical reactions, shall not be biodegradable and, in the event of contact with other materials, shall not lead to harmful effects such as to cause environmental pollution or damage to human health;

d) ‘recovered aggregate’; waste referred to in points (a) and (b) which has ceased to be waste as a result of one or more recovery operations in compliance with the conditions laid down in Article 184b(1) of Legislative Decree No 152 of 3 April 2006 and the provisions of this Decree;

e) ‘recovered aggregate batch’: not more than 3,000 cubic metres of recovered aggregate;

f) ‘recovered aggregate producer’: the operator of the plant authorised for the production of recovered aggregate (hereinafter also only: producer);

g) ‘declaration of compliance’: declaration in lieu of the affidavit issued by the producer attesting to the characteristics of the recovered aggregate, as referred to in Article 5;

h) 'competent authority’: the authority issuing the authorisation pursuant to Title IIIa of Part II or to Title I, Chapter IV of Part IV of Legislative Decree No 152 of 3 April 2006, i.e. the authority receiving the communication referred to in Article 216 of said decree.

# Article 3

# Criteria for end-of-waste status

- For the purposes of Article 1 and pursuant to and for the purposes of Article 184b of Legislative Decree No 152 of 3 April 2006, inert waste from construction and demolition activities and other inert waste of mineral origin, as defined in points (a) and (b) of Article 2, shall cease to be classified as waste and shall be classified as a recovered aggregate if the same complies with the criteria set out in Annex 1.

# Article 4

# Specific uses

- The recovered aggregate can only be used for the specific purposes listed in Annex 2.

# Article 5

# Declaration of conformity and arrangements for retaining samples

- Compliance with the criteria referred to in Article 3 is certified by the recovered aggregate producer through a declaration in lieu of the affidavit, pursuant to Article 47 of Presidential Decree No 445 of 28 December 2000, prepared for each recovered aggregate batch produced. The declaration shall be drafted using the form in Annex 3 and sent to the competent authority and to the locally competent environmental protection agency by one of the methods referred to in Article 65 of Legislative Decree No 82 of 7 March 2005.

- The recovered aggregate producer shall keep a copy of the declaration referred to in paragraph 1 at the production plant or its registered office, including in electronic form, making it available to the control authorities who request it.

- In order to meet the criteria set out in Article 3, the recovered aggregate producer shall keep for five years, at the production facility or at its registered office, a sample of recovered aggregate taken, at the end of the production process of each recovered aggregate batch, in accordance with UNI 10802. The method for storing the sample shall be such as to ensure that the chemical and physical characteristics of the recovered aggregate are not altered and are suitable for repeating the analyses.

# Article 6

# Management system

- The recovered aggregate producer shall apply a quality management system according to the standard UNI EN ISO 9001 certified by an organisation accredited in accordance with the legislation in force, demonstrating compliance with the criteria set out in this Regulation. The quality manual shall include operational procedures for checking the characteristics of compliance with the criteria set out in Annex 1 of the sampling and self-monitoring plan.

- The provisions referred to in Article 5(3) shall not apply to registered companies pursuant to Regulation (EC) No 1221/2009 of the European Parliament and of the Council of 25 November 2009, or to companies in possession of the environmental certification UNI EN ISO 14001, issued by an accredited body pursuant to the applicable legislation.

# Article 7

# Transitional and final provisions

- In order to comply with the criteria laid down in this Regulation, the producer shall, within 180 days of the entry into force of this Regulation, submit to the competent authority an update of the communication made pursuant to Article 216 of Legislative Decree No 152 of 3 April 2006, indicating the maximum amount recoverable, or an application for updating the authorisation granted under Chapter IV, Title I, Part IV or Title IIIa of Part II of Legislative Decree No 152 of 3 April 2006. For simplified procedures, the quantitative limits laid down in the Decree of the Minister for the Environment of 5 February 1998 in Annex 4, the technical standards set out in Annex 5 and the emission limit values set out in Annex 1, sub-Annex 2, shall remain fixed.

- Pending the adjustment referred to in paragraph 1, materials resulting from already-authorised recovery procedures may be used for the specific purposes referred to in Article 4 if they have characteristics that comply with the criteria set out to in Article 3, certified by a declaration of compliance pursuant to Article 5.

- The Annexes constitute an integral part of this Regulation.

This Decree, bearing the State seal, shall be included in the official collection of legal acts of the Italian Republic. All interested parties shall be bound to observe and ensure observance of this Regulation.

Roberto Cingolani

# Annex 1 (Article 3)

a) eligible waste.

Only non-hazardous construction and demolition inert waste identified in Table 1, point 1, and non-hazardous inert waste of mineral origin listed in Table 1, point 2, may be used for the production of recovered aggregate.

Waste eligible for the production of recovered aggregates originates exclusively from products which have undergone prior decontamination/rehabilitation operations aimed at the removal of asbestos-containing materials, PCB-containing/contaminated equipment, bituminous sheaths, potentially hazardous coatings and insulation materials and other materials contaminated or containing hazardous substances, within the meaning of Decision 2000/532/EC and Directive 2008/98/EC. The waste allowed preferentially comes from selectively demolished products.

Waste from abandoned or buried construction and demolition activities shall not be eligible for the production of recovered aggregate.

1. Inert waste from construction and demolition activities (Chapter 17 of the European Waste List) 170101 Cement 170102 Bricks 170103 Tiles and ceramics 170107 Mixtures or slag of cement, bricks, tiles and ceramics, other than those mentioned in heading 170106 170302 Bituminous mixtures other than those mentioned in heading 170301 170504 Excavation earth and rocks, other than those mentioned in heading 170503 170508 Crushed stone for railway ballast, other than that mentioned in heading 170507 170904 Mixed construction and demolition waste, other than that mentioned in headings 170901, 170902 and 170903 2. Other inert waste of mineral origin (not belonging to Chapter 17 of the European Waste List) 010408 Gravel and crushed stone waste, other than that mentioned in heading 01 04 07 010409 Waste of sand and clay 010410 Dust and similar residues, other than those mentioned in heading 010407 010413 Waste from cutting and sawing stone, other than that mentioned in heading 010407 101201 Preparation mixture residues that have not been heat-treated 101206 Scrap moulds consisting exclusively of chips and waste from unfired glazed and fired ceramic products or of baked terracotta chips and expanded clay possibly covered with unfired glaze in concentration <10 % by weight 101208 Ceramic waste, bricks, tiles and building materials (heat-treated) 101311 Waste from the production of cement-based composite materials, other than those mentioned in headings 101309 and 101310 120117 Residues of blasting material, other than those mentioned in heading 120116 consisting solely of abrasive waste sands 191209 Minerals (e.g. sand, rocks)

Table 1 – Waste permitted for the production of recovered aggregate

b) Checks on incoming waste.

Waste permitted for the production of recovered aggregate shall be subject to an examination of the documentation accompanying the incoming waste, to a visual inspection and to additional checks, where necessary.

To this end, the producer of the recovered aggregate shall have in place a system for checking the acceptance of waste to verify that the waste corresponds to the characteristics laid down in this Regulation.

For undertakings registered in accordance with Regulation (EC) No 1221/2009 of the European Parliament and of the Council of 25 November 2009 and for undertakings holding the environmental certification UNI EN ISO 14001 issued by an organisation accredited under the current legislation, that system shall be integrated into the environmental management system.

The system shall at least ensure compliance with the following obligations and shall require the establishment of a procedure for the management, traceability and reporting of non-compliances detected:

- examination of the documentation accompanying the incoming waste load by personnel with an appropriate level of training;

- visual inspection of the incoming waste load;

- acceptance of such waste only where the examination of the accompanying documentation and the visual inspection are successful under the control of personnel with annual training and refresher courses to sort the waste, and remove and keep separate any foreign material;

- weighing and recording of incoming waste load data;

- separate storage of waste that does not comply with the criteria set out in this Regulation in a dedicated area;

- placing in reserve compliant waste, as set out in Table l of this Annex, in the area dedicated exclusively to it, which is structured in such a way as to prevent mixing, including accidental mixing, with other types of waste that is not allowed;

- handling of waste sent to the production of recovered aggregate by personnel with annual training and refresher courses in order to prevent contamination of the waste with other waste or foreign material;

- carrying out additional checks, including analytical checks, on a sample basis or whenever the analysis of the documentation and/or the visual inspection indicate this need.

c) Minimum manufacturing process and storage at the producer

The process of treatment and recovery of ‘inert waste from construction and demolition activities’ and ‘other inert waste of mineral origin’, as defined in points (a) and (b) of Article 2, for the production of the recovered aggregate, shall take place through mechanical and technologically interconnected steps, such as, merely by way of example:

- grinding,

- sifting,

- granulometric selection,

- separation of the metal fraction and unwanted fractions.

The recovery process, depending on the type of material, is carried out through the completion of all or only some of the steps referred to, or other mechanical processes that allow compliance with the criteria laid down in this Decree.

During the compliance verification step for the recovered aggregate, storage and handling at the producer shall be organised in such a way that the individual production batches are not mixed.

Pending transport to the site where it will be used, the recovered aggregate is stored and handled in the plant where it was produced and in the storage areas used for this purpose. This is without prejudice to all the provisions in force regarding safety and prevention in the workplace and the specific authorisation provisions.

- Quality requirements of the recovered aggregate

d.1) Checks on the recovered aggregate

For each batch of recovered aggregate output, compliance with the parameters set out in Table 2 shall be ensured.

| Parameters | Unit of measure | Limit concentrations |
| --- | --- | --- |
| Asbestos | mg/kg expressed as dry matter | 100(*) |
| (AROMATIC HYDROCARBONS) |  |  |
| Benzene | mg/kg expressed as dry matter | 0.1 |
| Ethylbenzene | mg/kg expressed as dry matter | 0.5 |
| Styrene | mg/kg expressed as dry matter | 0.5 |
| Toluene | mg/kg expressed as dry matter | 0.5 |
| Xylene | mg/kg expressed as dry matter | 0.5 |
| Organic aromatic additives (20 to 23) | mg/kg expressed as dry matter | 1 |
| (POLYCYCLIC AROMATIC HYDROCARBONS) |  |  |
| Benzo(a)anthracene | mg/kg expressed as dry matter | 0.5 |
| Benzo(a)pyrene | mg/kg expressed as dry matter | 0.1 |
| Benzo(b)fluoranthene | mg/kg expressed as dry matter | 0.5 |
| Benzo(k)fluoranthene | mg/kg expressed as dry matter | 0.5 |
| Benzo(ghi)perylene | mg/kg expressed as dry matter | 0.1 |
| Chrysene | mg/kg expressed as dry matter | 5 |
| Dibenzo(a,e)pyrene | mg/kg expressed as dry matter | 0.1 |
| Dibenzo(a,l)pyrene | mg/kg expressed as dry matter | 0.1 |
| Dibenzo(a,i)pyrene | mg/kg expressed as dry matter | 0.1 |
| Dibenzo(a,h)pyrene | mg/kg expressed as dry matter | 0.1 |
| Dibenzo(a,h)anthracene | mg/kg expressed as dry matter | 0.1 |
| Indenopyrene | mg/kg expressed as dry matter | 0.1 |
| Pyrene | mg/kg expressed as dry matter | 5 |
| Polycyclic aromatic additives (25 to 34) | mg/kg expressed as dry matter | 10 |
| Phenol | mg/kg expressed as dry matter | 1 |
| PCB | mg/kg expressed as dry matter | 0.06 |
| C>12 | mg/kg expressed as dry matter | 50 |
| Cr VI | mg/kg expressed as dry matter | 2 |
| Floating materials (**) | cm3/kg | <5 |
| Foreign fractions (**) | % in weight | <1% |

Table 2 - Parameters to be searched and limit values

(*) Corresponding to the detection limit of the analytical technique (microscopy and/or equivalent in terms of detection). In any case, the officially recognised methodology shall be used for the whole national territory which allows the detection of lower concentration values.

(**) Where not defined by applicable technical standards

d.2)	Release test on the recovered aggregate.

Each batch of recovered aggregate produced, with the exception of those intended for the packaging of concretes referred to in UNI EN 12620 with resistance class Rck/leq ≥ 15 MPa, must be subjected to the release test to assess compliance with the limit concentrations of the parameters identified in Table 3.

Appendix A to the UNI 10802 standard and the method provided by the UNI EN 12457-2 standard shall be applied to determine the release test.

Only in cases where the sample to be analysed has a very fine grain size, an ultracentrifuge (20000 G) should be used for at least 10 minutes without proceeding with the natural sedimentation step.

Only after this step can the subsequent filtration step be carried out in accordance with point 5.2.2 of the UNI EN 12457-2 step.

| Parameters | Unit of measure | Limit concentrations |
| --- | --- | --- |
| Nitrate | mg/l | 50 |
| Fluoride | mg/l | 1.5 |
| Cyanides | micrograms/l | 50 |
| Barium | mg/l | 1 |
| Copper | mg/l | 0.05 |
| Zinc | mg/l | 3 |
| Beryllium | micrograms/l | 10 |
| Cobalt | micrograms/l | 250 |
| Nickel | micrograms/l | 10 |
| Vanadium | micrograms/l | 250 |
| Arsenic | micrograms/l | 50 |
| Cadmium | micrograms/l | 5 |
| Total chromium | micrograms/l | 50 |
| Lead | micrograms/l | 50 |
| Selenium | micrograms/l | 10 |
| Mercury | micrograms/l | 1 |
| COD | mg/l | 30 |
| Sulphates | mg/l | 750 |
| Chlorides | mg/l | 750 |
| pH |  | 5.5 < > 12.0 |

Table 3 – Analytes to be researched and limit values.

e) Reference technical standards for the EC certification of the recovered aggregate.

Table 4 sets out the reference technical standards for attributing the CE marking to the recovered aggregate.

| Standard | Title |
| --- | --- |
| UNI EN 13242 | Aggregates for non-alloy materials and alloys with hydraulic binders for use in civil engineering and road construction |
| UNI EN 12620 | Aggregates for concrete |
| UNI EN 13139 | Aggregates for mortar |
| UNI EN 13043 | Aggregates for bituminous mixtures and surface treatments for roads, airfields and other trafficked areas |
| UNI EN 13055 | Light aggregates |
| UNI EN 13450 | Aggregates for railway ballast |
| UNI EN 13383-1 | Aggregates for protective works (armourstone) – Specifications |

Table 4 – Technical standards for EC certification

# Annex 2 (Article 4)

The recovered aggregate shall be used, in accordance with the technical use standards set out in Table 5, for:

- the construction of the body of embankments in civil engineering earthworks;

- the construction of road, rail, airport sub-bases and civil and industrial forecourts;

- the construction of foundations for transport infrastructure and civil and industrial forecourts;

- the realisation of environmental recoveries, filling and bridging;

- the creation of ancillary layers having capillary-breaking, antifreeze, draining, etc. functions;

- the packaging of concrete and mixtures alloyed with hydraulic binders (cement mixtures, concrete mixtures, etc.).

| Use | Compliance with European harmonised standards/performance | Technical capabilities |
| --- | --- | --- |
| Fillings, backfills, morphological restorations | UNI EN 13242 | UNI EN 11531-1 Schedule 4a |
| Embankment body | UNI EN 13242 | UNI 11531-1 Schedule 4a |
| Non-alloy mixtures, capillary-breaking layer, foundation, base | UNI EN 13242 UNI EN 13450 | UNI 11531-1 Schedule 4b |
| Production of mixtures alloyed with hydraulic binders (cement mixtures, concrete mixtures, etc.) | UNI EN 13242 | UNI EN 14227-1:2013 |
| Production of concrete | UNI EN 12620 | UNI 8520-1 Schedule 1 UNI 8520-2 Annex A UNI 11104 Schedule 4 UNI EN 206 Annex E Ministerial Decree of 17 Jan. 2018 NTC: Table 11.2.III |

Table 5- Technical use standards for the recovered aggregate

For all uses, except those referred to in point (d), the application of the CE marking as provided for in Regulation (EU) No 305/2011 of the European Parliament and of the Council of 9 March 2011 is required.

Soil uses shall not constitute a potential source of contamination for soil, subsoil and groundwater.

For the uses referred to in point 1(f), the limits set out in heading 47 of Annex XVII to Regulation (EC) No 1907/2006 for the presence of Cr VI in cement and cement-containing mixtures shall be complied with.

# Annex 3 Declaration of compliance (Article 5)

DECLARATION OF COMPLIANCE (DoC)

DECLARATION IN LIEU OF THE AFFIDAVID

PURSUANT TO AND IN ACCORDANCE WITH ARTICLE 5 OF THE DECREE OF THE MINISTER FOR ECOLOGICAL TRANSITION, NO [•] OF [•] [•] [202•] PUBLISHED IN [•]

(Articles 47 and 38 of Presidential Decree No 445 of 28 December 2000)

| Declaration number (Batch no.) | _________ |
| --- | --- |
| Year |  |
|  | (yyyy) |

(NOTE: record the declaration number incrementally)

| Details of the recovered aggregate producer pursuant to Article 2(1)(f) of Decree [•] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Company Name |  |  | Tax/VAT number |  |
| Registration in the companies register |  |  |  |  |
| Address |  |  |  | House Number |
| Postcode | City |  |  | Province |
| Production facility |  |  |  |  |
| Address |  |  |  | House Number |
| Postcode | City |  |  | Province |
| Authorisation / Issuing body |  |  |  | Issued on |

# The manufacturer indicated above declares that

- the recovered aggregate batch is represented by the following quantity in quantity in volume:

______________________________________________________________________

(NOTE: indicate cubic meters in figures and letters)

- the aforementioned batch of recovered aggregate complies with the criteria set out in Article 3 of Decree No [•] of the Minister for Ecological Transition of [•] [•] [202•] published in [•];

- the above batch of recovered aggregate has characteristics that are better detailed in the following Table 1.

Table 1

| Characteristics of the recovered aggregate |  |  |
| --- | --- | --- |
| Technical reference standards |  | Specific purposes (Annex 2) |
|  UNI EN 13242: Aggregates for non-alloy materials and alloys with hydraulic binders for use in civil engineering and road construction; |  | - a) - b) - c) - d) - e) - f) |
|  UNI EN 14227-1: Mixtures alloyed with hydraulic binders – Specifications – Part 1: Granular mixtures bound with cement for road bases and sub-bases; |  | - a) - b) - c) - d) - e) - f) |
|  UNI EN 12620: Aggregates for concrete; |  | - a) - b) - c) - d) - e) - f) |
|  UNI EN 13139: Aggregates per mortar; |  | - a) - b) - c) - d) - e) - f) |
|  UNI EN 13043: Aggregates for bituminous mixtures and surface treatments for roads, airfields and other trafficked areas; |  | - a) - b) - c) - d) - e) - f) |
|  UNI EN 13055: Light aggregates; |  | - a) - b) - c) - d) - e) - f) |
|  | UNI EN 13450: Aggregates for railway ballast; | - a) - b) - c) - d) - e) - f) |
|  | UNI EN 13383-1: Aggregates for protective works (armourstone) – Specifications. | - a) - b) - c) - d) - e) - f) |

Finally, the producer states that:

- they are aware of the criminal penalties for providing false or inaccurate statements in official documents and of the consequent loss of benefits pursuant to Articles 75 and 76 of

PRESIDENTIAL DECREE NO 445/2000;

- they have been informed that all personal data collected shall be electronically processed exclusively for the procedure for which the declaration is submitted (Article 13 of Regulation (EU) No 679/2016).

| _________ , on ______________________ (NOTE: indicate place and date) | _______________________________ NOTE: (Signature and stamp of the manufacturer) |
| --- | --- |

(exempt from stamp duty pursuant to Article 37 of Presidential Decree No 445/2000)

Annexes: photostat copy of the subscriber’s identity document and analysis report.