
Draft of 18 July 2016
– 39 –


1. ------IND- 2016 0374 PL- EN- ------ 20160810 --- --- PROJET

REGULATION

OF THE MINISTER OF INFRASTRUCTURE AND CONSTRUCTION
)

of … 2016

on the manner of declaring the performance properties of construction products and the manner of marking them with construction marking
)
Pursuant to Article 8(8) of the Construction Products Act of 16 April 2004 (Journal of Laws of 2014 (883), of 2015 (1165) and of 2016 (542)) the following is decreed:

§ 1. The Regulation defines:
1)
the manner of declaring the performance properties of construction products;
2)
national systems for the assessment and verification of the constancy of performance properties for construction products, hereinafter “national systems”;
3)
the groups of construction products with an obligation to draw up the national declaration on performance properties, hereinafter “national declaration” and, in terms of these groups, appropriate national systems for the assessment and verification of the constancy of performance properties;
4)
the model and content of the national declaration;
5)
the manner of making available and delivering the national declaration to the recipients of construction products;

6)
the manner of marking construction products with construction marking and the scope of information accompanying the marking.

§ 2. Whenever the Regulation refers to:
1)
Polish Standard – this shall be understood as the Polish Standard of the product, which does not hold the status of withdrawn standard;
2)
national technical specification – this shall be understood as the Polish Standard or a national technical assessment;
3)
factory production control – this shall be understood as a documented, ongoing and internal control of production at the production plant in keeping with the applicable national technical specifications;
4)
national certificate on the constancy of performance properties – this shall be understood as a document issued by a certification body, which is required for the drawing up of the national declaration;
5)
national certificate on the compliance of performance properties – this shall be understood as a document issued by a certification body, which is required for the drawing up of the national declaration;
6)
construction product type – this shall be understood as a set of representative levels or classes of performance properties in reference to specific characteristics and the intended application of the construction product manufactured by means of a combination of raw materials or other components in a given production process.
§ 3. 1. The producer shall declare the performance properties of the construction product in terms of level, class or descriptively, on the basis of assessment and verification of the constancy of performance properties, performed in keeping with the system corresponding to that product and its intended application.
§ 4. 1. National systems referred to under (2–6) define the activities of the producer involved in the assessment and verification of the constancy of construction product performance properties, as well as the scope of this assessment and verification, carried out on behalf of the producer by a certification body or research laboratory accredited in keeping with the Conformity Assessment and Market Surveillance Systems Act of 13 April 2016 (Journal of Laws (542)) which are not their own accredited bodies.
2. Under the 1+ national system:
1)
producer measures connected with assessment and verification, referred to in (1), shall cover the description of the type of construction product and the performance of:

a)
factory production control,
b)
tests on samples collected by the producer at the production plant, in keeping with the study plan determined by the product;
2)
assessment and verification performed by the certification body, referred to in (1), shall cover:

a)
an assessment of the performance properties of construction products on the basis of tests on samples collected by the certification body, calculations, tabular values or descriptive documentation belonging to the product;
b)
the performance of a preliminary inspection of the production plant and factory production control;
c)
issuance of the national certificate of the constancy of performance properties;
d)
continued supervision, assessment and evaluation or factory production control;
e)
the performance of inspection tests of samples collected by the certification body at the production plant or at producer storage facilities;
3. Under the 1 national system:
1)
producer measures connected with assessment and verification, referred to in (1), shall cover the description of the type of construction product and the performance of:

a)
factory production control,

b)
tests on samples collected by the producer at the production plant, in keeping with the study plan determined by the product;
2)
assessment and verification performed by the certification body, referred to in (1), shall cover:

a) 
an assessment of the performance properties of construction products on the basis of tests on samples collected by the certification body, calculations, tabular values or descriptive documentation belonging to the product;
b)
the performance of a preliminary inspection of the production plant and factory production control;
c)
issuance of the national certificate of the constancy of performance properties;
d)
continued supervision, assessment and evaluation or factory production control;
4. Under the 2+ national system:
1)
producer measures connected with assessment and verification, referred to in (1), shall cover the description of the type of construction product and the performance of:

a)
an assessment of the performance properties of the product on the basis of samples collected by the producer, calculations, tabular values or descriptive documentation belonging to the product;
b)
factory production control;
c)
tests on samples collected by the producer at the production plant, in keeping with the study plan determined by the product;
2)
assessment and verification performed by the certification body, referred to in (1), shall cover:

a)
the performance of a preliminary inspection of the production plant and factory production control;
b)
issuance of the national certificate of factory production control;
c) 
continued supervision, assessment and evaluation of factory production control;

5. Under the 3 national system:
1)
producer measures connected with assessment and verification, referred to in (1), shall cover the description of the type of construction product and the performance of factory production control:

2)
assessment and verification performed by the research laboratory, referred to in (1), covers the assessment of construction product performance properties on the basis of samples collected by the producer, calculations, tabular values or descriptive documentation belonging to the product;

6. Under the 4 national system:
1)
producer measures connected with assessment and verification, referred to in (1), shall cover the description of the type of construction product, assessment of the product performance properties on the basis of tests, calculations, tabular values or descriptive documentation belonging to the product and the performance of factory production control:

2)
the certification body or research laboratory does not participate in the assessment and verification, referred to in (1).
7. The national certificate on the constancy of performance properties under the 1+ and 1 national system may be limited, suspended or withdrawn by the certification body on the basis of the results of assessment and verification.
8. The national certificate on the compliance of factory production control under the 2+ may be limited, suspended or withdrawn by the certification body on the basis of the results of assessment and verification.
9. If a national technical assessment has been issued for a construction product, the assessment of the product performance of the product, referred to in (2)(2)(a), (3)(2)(a), (4)(1)(a), (5)(2) and (6)(1) constitutes the decisions for that national technical assessment in terms of product performance for the product.
§ 5. 1. The producer, in defining the type of construction product and assessing its performance properties, may replace the tests or calculations, required for the assessment of performance properties, by indicating in the technical documentation, referred to in Article 10a(1) of the Construction Products Act of 16 April 2004, that:

1)
the fundamental characteristics indicated in the applied technical specifications and referring to the intended use, attain a given performance property level or class without the need for tests or calculations, in keeping with the conditions defined in the applied national technical specifications or decisions of the European Commission in this respect;
2)
a construction product covered by the Polish Standard belongs to the same type of product, for which the assessment of performance properties was performed by another producer, who approved the use of test results performed under the assessment and shall be held responsible for their correctness.
2. A producer who is a micro-enterprise in the understanding of the provisions of the Freedom of Economic Activities Act of 2 July 2004 (Journal of Laws of 2015(584), as amended
)), and who applies national system 3 or 4, may replace in the Polish Standard the indicated methods of performing tests by means of other methods if the equivalence of these methods is provided in the technical documentation, referred to in (1).

§ 6. 1. The certification body that carries out activities under national systems 1+, 1 and 2+ shall hold accreditation for bodies which certify products for the performance of activities stemming from these systems in reference to the construction product or a group of construction products.
2. A research laboratory that performs activities under national system 3 shall hold accreditation for research laboratories with an accreditation range defined by reference to standards on test methods, research or calculation procedures, invoked under national technical specifications and referring to construction products.
§ 7. 1. Groups of construction products for which a national declaration must be drawn up and the required national systems for these products are defined in Annex 1 to the Regulation.

2. The producer, referred to in § 5(2), may apply national system 4 for assessing the performance properties of a construction product for which, in keeping with Annex 1, the corresponding national system is national system 3.

§ 8. The model and content of the national declaration is defined by Annex 2 to the Regulation.
§ 9. 1. The producer shall make available a copy of the national declaration on its website for 10 years starting from the day of placing the construction product on the market.
2. In making available the national declaration on performance properties on the website the producer shall:
1)
ensure that the content of the national declaration is not changed after being posted on the website;
2)
indicate, with the use of construction marking, in the accompanying information on the marking of the construction product, the website address on which the national declaration is made available.
3. If no copy of the national declaration is made available on the producer’s website, a copy of such declaration shall be sent to the recipient of the construction product by electronic means.
4. At the request of the recipient of the construction product, a copy of the national declaration shall be provided in paper form.
5. If the same construction products are being delivered to one recipient, these may be accompanied by one copy of the national declaration in paper or electronic form.
§ 10. 1. The producer shall mark the construction product with construction marking prior to placing it or making it available on the national market.
2. The construction mark shall be placed visibly, legibly and permanently, immediately on the production product or on the label attached to the product.
3. If the placing of the construction mark in the manner defined in (2) is not possible because of the size or character of the construction product, the construction mark shall be placed on the unit packaging or on the outside packaging of the construction product or on the documents accompanying the product.

§ 11. 1. The following information shall accompany the marking of the construction product:
1)
the two last digits of the year, in which the construction mark was placed for the first time on the construction product;
2)
the name and address of the registered office of the producer or an identification mark permitting the unequivocal indication of the producer’s name and address;
3)
the name and marking of the construction product;
4)
the reference number of the Polish Standard or the number and year of issue of the national technical assessment, in keeping with which the performance properties were declared;
5)
the number of the national declaration;
6)
the level or class of the declared performance properties;
7)
the name of the certification body, if such body participated in the assessment and verification of the constancy of construction product performance properties;
8)
the producer’s website address, if the national declaration is available there.
2. The information, referred to in (1), shall be placed or attached to the construction product, in keeping with the provisions of § 10(2) and (3).

§ 12. A construction product which is marked with construction marking may be provided with other marking, signs and inscriptions, providing they do not limit the visibility and legibility of the construction mark and the information, referred to in § 11 and their meaning and graphic form are not misleading.
§ 13. A certification body which holds accreditation permitting the performance of tasks under compliance assessment systems, referred to in § 4(3) of the Regulation of the Minister of Infrastructure of 11 August 2004 on the manner of declaring the compliance of construction products and the manner of marking them with construction marks (Journal of Laws, No 198(2041), as amended), which issued the national compliance certificate or the factory production control national compliance certificate prior to the day of entry into force of this Regulation may, from 1 January 2017 under the issued certificate, perform tasks stemming from these systems on the basis of the held accreditation.
§ 14. The producer of the construction product mentioned in Annex 1 which, in keeping with provisions binding until 31 December 2016, did not have an obligation to mark with the construction mark, shall not have an obligation until 30 June 2018 to draw up a national declaration when placing the construction product or making it available on the national market.

§ 15. The producer of a construction product who drew up a national declaration on performance properties, pursuant to Article 5(2) of the Act of 25 June 2015 on the Amendment of the Construction Products Act, Act – Construction law and the Act on the Amendment of the Construction Products Act and the System of Assessing Conformity Act (Journal of Laws (1165)) may use, until 30 June 2017, the manner of marking a construction product with a construction mark and the scope of information accompanying the product in keeping with the provisions binding until 31 December 2016.

§ 16. The Regulation comes into force on 1 January 2017.

Minister of Infrastructure and Construction

ANNEXES
to the Regulation of the Minister of 
Infrastructure and Construction
of … (…)
ANNEX 1
GROUPS OF CONSTRUCTION PRODUCTS WITH AN OBLIGATION TO DRAW UP THE NATIONAL DECLARATION ON PERFORMANCE PROPERTIES AND NATIONAL SYSTEMS FOR THE ASSESSMENT AND VERIFICATION OF THE CONSTANCY OF PERFORMANCE PROPERTIES REQUIRED FOR THESE GROUPS

	Item
	grOup OF CONSTRUCTION PRODUCTS
	intended application of the construction productS
	Classes
	NATIONAL SYSTEM FOR THE ASSESSMENT AND VERIFICATION OF THE CONSTANCY OF PERFORMANCE PROPERTIES 

	(1)
	(2)
	(3)
	(4)
	(5)

	1.
	Precast normal/lightweight/autoclaved cellular concrete products
)


	
	Precast normal/lightweight/autoclaved cellular concrete products 
	- to be used as structural elements 
)

	-
	2+

	
	
	- for remaining applications
	-
	4

	
	Prefabricated sets/beam and block systems (prefabricated beams and prefabricated blocks of concrete or blocks of other materials)
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E)***,F
	4

	
	
	- for all applications
	-
	2+

	2.
	Doors, windows, gates and corresponding building fittings and supplementary products

	
	Doors and gates (with or without fittings)
	- for use in fire divisions and along escape routes
	-
	1

	
	
	- for use in compliance with acoustics requirements, saving energy, tightness and safe operation
	-
	3

	
	
	- for remaining use in internal communication
	-
	4

	
	Windows (with or without fittings)
	- for use in fire divisions and along escape routes
	-
	1

	
	
	- for remaining applications
	-
	3

	
	Construction fittings for doors, gates and windows
	- for use in fire divisions and along escape routes
	-
	1

	
	
	- for remaining applications
	-
	3

	
	Shutters, blinds, external awnings, sunblinds
	- for external purposes
	-
	4

	
	Supplementary products: intake vents, strips, foam for mounting doors, windows and gates
	- for use in building structures
	-
	3

	
	Window sills
	- for use in building structures
	-
	4

	3.
	Membranes, including liquid applied to provide insulation against water or steam

	
	Products for damp proof/waterproof installation layers with the exception of sheet products
Products for layers regulating vapour penetration, vapour-proof or vapour permeable
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E)***,F
	4

	
	
	- for remaining applications
	-
	3

	
	Products for damp proof insulation in sheets
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E) ***, F
	4

	
	
	- for remaining applications
	-
	2+

	
	Products for waterproofing roofs
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E) ***,F
	4

	
	
	- to be used in compliance with the resistance to external fire
	BROOF(t1)
)
	3

	
	
	
	BROOF

	4

	
	
	- for remaining applications
	-
	2+

	
	Waterproofing underlays under roofing
Products/sets for the placing of roofing layers in liquid form-
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E) ***,F
	4

	
	
	- to be used in compliance with the resistance to external fire
	BROOF(t1)8) 
	3

	
	
	
	BROOF9)
	4

	
	
	- for remaining applications
	-
	3

	
	Products for waterproofing walls and floors of rooms exposed to humidity, including those applied in liquid form
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E) ***, F
	4

	
	
	- for remaining applications
	-
	2+

	
	Products/sets for the placing of waterproof bridge plates in liquid form
	- for use with bridge plates
	-
	2+

	4.
	Thermal insulation products. Composite insulating kits/systems

	
	Products for thermal insulation (factory products and products for preparation at the building site)
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E)**, F
	4

	
	
	- for remaining applications
	-
	3

	
	Composite systems/thermal insulating kits with plaster coating or other types of façade layers
	- for use on external walls subject to fire safety requirements
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E, 
(A1-E)**, F
	2+

	
	
	- for use on external walls not subject to fire safety requirements
	-
	2+

	
	Prefabricated insulation elements with ready surface quality
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	3

	
	Mesh reinforcement, glues for affixing thermal insulation materials to the base
	- to be used in compliance with requirements on fire resistance
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	2+

	5.
	Structural bearings. Pins for structural joints

	
	Structural bearings
	- for use in buildings and structures if the requirement to place a single bearing is decisive
)
	-
	1

	
	
	- for use in engineering buildings and structures if the requirement to place a single bearing is not decisive
)

	-
	3

	
	Mandrel joints for construction joints
	- for structural use7)
	-
	2+

	
	
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E)***, F
	4

	
	Expansion joints for road bridges
	- for use with road bridges 
	-
	1

	6.
	Chimneys, flues and specific products

	
	Prefabricated chimneys (elements one level in height), flues (elements or blocks), multi-coated chimneys (elements or blocks), single-coated chimney blocks, element sets of free-standing chimneys and extra chimneys
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E)***, F
	4

	
	
	- for remaining applications
	-
	2+

	
	Chimney nozzles and chimney termination elements
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B*, C**, D, E
	3

	
	
	
	(A1-E)***, F
	4

	
	
	- for remaining applications
	-
	4

	7.
	Gypsum products

	
	Gypsum-plasterboards, reinforced fibre gypsum boards, gypsum-particle boards, including those undergoing additional treatment (e.g. laminated boards)
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for reinforcing wooden framework structures or roofing 
	-
	3

	
	
	- for use in fire-protection systems for structural elements or for separating fire zones in buildings
	-
	3

	
	
	- for remaining applications
	-
	4

	
	Composite panels/gypsum-plasterboard panels, reinforced fibre gypsum boards, gypsum-particle boards
Gypsum blocks, fibre gypsum boards and gypsum-particle boards
Gypsum glues
Gypsum plaster casts, gypsum-based plaster, gypsum masonry mortar, gypsum plaster and mortar for special use

Ancillaries:

- mesh and strips for plastering,

- fillers and tape for grouting,

- framework structure elements for use with gypsum-plasterboard panels, fibre gypsum boards, fibre gypsum and products made of these boards,

- mechanical fasteners for attaching (including nails, screws and staples),

- corners and special profiles,

- prefabricated ledges
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for use in fire-protection systems for structural elements or for separating fire zones in buildings
	-
	3

	
	
	- for remaining applications
	-
	4

	8.
	Geotextiles, geomembranes and related products

	
	Geosynthetics (membranes and textiles), geocomposites, geomesh, geogrates
	- for use as a barrier to liquids and gases, protective layer for drainage and filtration, for reinforcement of the soil
	-
	2+

	
	
	- for use as a barrier to liquids and gases, protective layer for drainage and filtration, for reinforcement of the soil in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*-
	1

	
	
	
	Al**, A2**, B**, C**, D, E, (Al-E)***, F
	3

	
	
	
	(Al-E)***, F
	4

	9.
	Curtain walling/cladding/structural sealant glazing

	
	Sets for curtain walling
Sets for housing balconies and loggias
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E, (Al-E)***, F
	3

	
	
	- for remaining applications
	-
	3

	
	External wall cladding sets
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(Al-E)***, F
	4

	
	
	- for remaining applications
	-
	2+

	
	Types I and III structural glazing sealant systems
):
	- for use with walls and overlaps
	-
	2+

	
	Types II and IV structural glazing sealant systems
:
	- for use with walls and overlaps
	-
	1

	10.
	Fixed firefighting equipment (fire alarm/detection products, fixed firefighting, fire and smoke control and explosion suppression products, evacuation systems).

	
	Fire alarm systems– sets:
signal-alarm sets, fire-signalling sets, alarm systems, fire alarm transmission systems
Fire alarm signalling systems- composition: fire detectors, control devices and signalling devices, alarm transmission devices, short-circuit isolators, alarm devices, power supply, input/output devices, manual fire-warning points, operation panels for the fire brigade, remote signalling and service equipment, outlet mounting linear elements, response indicators
Autonomous fire detectors, autonomous carbon monoxide detectors, power supply
Fire suppression and extinguishing systems - sets: internal hydrants, dry and irrigated hydrant installations, sprinkler systems, water spray and water mist systems, extinguishing foam fire sets, powder fire extinguishing sets, gas extinguishing sets (including CO2 extinguishing systems), aerosol extinguishing sets, sets for extinguishing sparks, sets to reduce oxygen concentration
Fire suppression and extinguishing systems - composition:
external hydrants, water flow sensors/switches, pressure sensors/switches, fire hydrant valves, connections for firefighters, fire pumps and fire pump sets, nozzles/sprinklers/vapour distributors/outlet items, water control and emergency valve assemblies, air control and emergency valve assemblies, signalling valve assemblies, boosters, pressure tank valve assemblies and their triggering devices, directional valves and triggering devices, non-electric locking devices, flexible connectors, pressure gauges and pressure switches, mechanical weighing devices, return and one-way valves, control and signalling devices, power sources, foam dispensers, nozzle foam pumps, firefighting water installation pumps, flexible hose connectors, pipe grooved fittings, pipe system grooved fittings, plastic tube systems, inlet valves and bolts, handles and fixing pipes sets, pressure reduction and regulation valves, extinguishing agent tanks, intake and distribution collectors, filters, operating panels for firefighters
Explosion prevention systems – sets
Explosion prevention systems - composition:
sensors, silencing devices, explosion sensors, relief products, steering and signalling devices, power supply, fire brigade operation panels
Systems for controlling the spread of smoke and heat - sets: sets for channelling away smoke and heat, pressure differentiation sets, sets for steering firefighting shut-offs
Systems for controlling the spread of smoke and heat - composition: smoke curtains, fire dampers, ducts, mechanical ventilators, smoke vents, relief valves, steering control devices, operating panels for firefighters, manually operated desmoking devices, power supply, linear actuators, rotary actuators, dampers for air-flow compensation, pressure sensors
Evacuation systems – sets:
emergency evacuation lighting systems, management systems for opening exits on escape routes, evacuation sets
Evacuation systems – composition:
steering and signalling devices, power supply, actuators, controlled passage interfaces, luminaires for emergency lighting, operating panels for firefighters, emergency chutes
Fire-protection current disconnector -sets
Fire-protection current disconnector -composition:
start-up devices, signalling devices, actuators
	- for use in building structures
	-
	1

	11.
	Sanitary appliances

	
	Sinks, wash basins, bathtubs, whirlpool baths, shower trays; showers with equipment or without, shower and bath screens, toilet bowels, toilet bowels with tank (compact), bidets; squatting toilets, macerating toilets, flush tanks, installation systems for sanitation and sanitary fittings and moulded toilet modules
	- for use in building structures
	-
	4

	
	Modular public toilets and sets for prefabricated toilets
	- for use in building structures
	Al*, A2*,B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(Al-E)***, F
	4

	12.
	Circulation fixtures: road equipment

	
	Road marking products
Road signs, permanently installed devices for controlling and steering traffic for railways and pedestrians
Road lighting and traction poles
Systems for limiting vehicle movement
Systems for separating pedestrians from vehicular traffic, including footbridges with protected handrails
	- for use in road traffic areas
	-
	1

	
	Devices/sets for reducing traffic noise levels
Anti-glare screens
Speed bumps
Traffic separators, pedestrian islands, blocking posts, road mirrors and protective covers
	- for use in road traffic areas
	-
	3

	13.
	Structural timber products/elements and ancillaries

	
	Structural products of solid wood: bridge elements, truss elements, beam elements, interior wall elements, roof elements, sets of trusses, beams, interior walls, roofs, frames, tram sleepers and pales
	- for structural use in buildings and structures
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E, (Al-E)***, F
	2+

	
	Structural products of glued laminated timber: bridge elements, truss elements, beam elements, interior wall elements, roof elements, sets of trusses, beams, interior walls, roofs, frames and pales
	- for structural use in buildings and structures
	-
	1

	
	Timber posts
	- for use with overhead power lines
	-
	2+

	
	Plate connectors for timber structural products 
	- for use with timber structural products and timber structures
	-
	2+

	
	Remaining connector items for timber structural products
	- for use with timber structural products and timber structures
	-
	3

	14.
	Wood-based panels and elements

	
	Raw wood-based panels, veneered or coated
	- for structural purposes 
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E, (Al-E)***, F
	2+

	
	
	- for non-structural purposes
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E,
	3

	
	
	
	(A1 - E)***, F
	4

	
	Light composite wood-based beams, posts and beam-plate elements
	- for structural purposes
	-
	1

	
	Prefabricated laminated-ribbed panels made of wood-based materials
Light, self-supporting composite panels 
	- for structural purposes
	-
	1

	
	
	- to be used in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
A1*fl, A2*fl, B*fl, C*fl
	1

	
	
	
	A1**, A2**, B**, C**, D, E
A1**fl, A2**fl, B**fl, C**fl, Dfl, Efl
	3

	
	
	
	(A1 - E)***, F
(A1fl - Efl)***, Ffl
	4

	
	
	- to be used in compliance with the resistance to external fire
	BROOF(t1)8)
	3

	
	
	
	BROOF9)
	4

	
	
	- for remaining applications
	-
	3

	15.
	Cement, building limes and other hydraulic binders

	
	Common, special, alumina, masonry cements
	- for preparing concrete, mortar, grout and other mixtures for the performance of construction works and construction products
	-
	1+

	
	Building limes
	- for preparing mortar, grout and other mixtures for the performance of construction works and construction products
	-
	2+

	
	Hydraulic binders for mixtures used for road substructure and ground stabilisation
	- for preparing concrete, mortar, grout and other mixtures for road substructure and ground stabilisation
	-
	2+

	
	Other building binders
	- for preparing concrete, mortar, grout and other mixtures for the performance of construction works and construction products
	-
	2+

	16.
	Reinforcing and pre-stressing steel for concrete (and ancillaries). Post tensioning kits.

	
	Products for reinforcing concrete: bars, rods (rings), soldered/welded mesh, reinforcing mats, tape with indents, soldered/welded prefabricated reinforcements
	- for use with reinforcement 
	-
	1+

	
	Products for pre-stressing concrete:
Post tensioning kits for pre-stressing concrete structures
	- for use when compressing a concrete structure
	-
	1+

	
	Channels and covers for protecting or managing products for pre-stressing
	- for use for protecting or managing pre-stressing steel products
	-
	4

	17.
	Masonry and related products

	
	Category I masonry units
)

	- for all applications
	-
	2+

	
	Category II masonry units
	- for all applications
	-
	4

	
	Category I masonry units with integral thermal isolation
	- for applications subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1*, A2*, B*, C*, D*, E*
	3

	
	
	
	(A1*-E*), F*
	4

	
	
	- for remaining applications
	-
	2+

	
	Category II masonry units with integral thermal isolation
	- for applications subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1*, A2*, B*, C*, D*, E*
	3

	
	
	
	(A1*-E*), F*
	4

	
	
	- for remaining applications
	-
	4

	
	Factory made mortar/masonry mass with determined composition 
	- for all applications
	-
	4

	
	Factory made mortar/masonry mass with determined properties
	- for all applications
	-
	2+

	
	Factory made mortar, plaster mass and products relating to: mesh, corners, beads
	- for all applications
	-
	4

	
	Wall ties, tension straps, hangers and brackets, support brackets, reinforcements for meshwork, lintels
	- for all applications
	-
	3

	
	Products for protection and repairs: mortar and repair mass, products for stop and merge injection, products for drying wall structures
	- for all applications
	-
	2+

	18.
	Waste water engineering products

	
	Pipes, fittings, connectors and seals
Devices providing protection against back flow

Vent valve
	- for use in drainage installations and networks
	-
	4

	
	Pumping stations and devices (products and sets) for cleaning sewage at the site of origin (domestic wastewater treatment, septic tanks) 
	- for use in drainage installations
	-
	3

	
	Prefabricated dewatering canals
	- to wastewater from buildings and structures, including roads
	-
	3

	
	Sewage manholes and inspection chambers, canal manholes, steps, ladders and manhole railing, sewage inlets, siphons, bolts, tanks
Retention-filter boxes with fittings for rainwater
	- for use in wastewater installations and networks, including roadways, car parks, hard shoulders and outside buildings
	-
	4

	
	Separators
	- to wastewater from buildings and structures, including roads
	-
	4

	
	Covers and tops of manholes and inspection chambers and gully tops
	- for use on roads, car parks, pavements and level crossings 
	-
	1

	
	
	- for other applications
	-
	4

	19.
	Floorings

	
	Rigid flooring products:
Setts, paving slabs, curbs, blocks, lights in pavements, metal finishing panels, floor tiles and panels, including slate, terrazzotiles, quarry tiles, mosaics; floor coverings of metal net or grate; floor gratings, planks, composite and laminated elements
	- for external use and road finishing, for external covering of road traffic and pedestrian movement
	-
	4

	
	Rigid flooring products:
plates, mosaics, mesh and plate covering, floor gratings, parquet floor, planks, composite and laminated elements, wood-based products

Raised floors and cavity floors placed on the market as sets
	- for external use together with covered transport areas
	Al*fl, A2*fl, B*fl, C*fl
	1

	
	
	
	Al**fl, A2**fl, B**fl, C**fl, Dfl, Efl
	3

	
	
	
	(Aln – Efl)***, Ffl
	4

	
	Elastic and fibre floor products:
uniform and non-uniform elastic floor covering in the form of tiles, panels, sheets or rolls

Floor underlay products, including products for reinforcing anti-contraction floor underlay

Sets for finishing floors and surfaces in-situ
	- to be used in compliance with the requirements on reaction to fire
	Al*fl, A2*fl, B*fl, C*fl
	1

	
	
	
	Al**fl, A2**fl B**fl, C**fl, Dfl, Efl
	3

	
	
	
	(Aln – Efl)***, Ffl
	4

	
	
	- for internal use subject to the requirements on hazardous substances
	-
	3

	
	
	- for remaining applications
	-
	4

	20.
	Structural metallic products and ancillaries7)

	
	Metal sections and construction profiles, hot-rolled section, cold formed profiles or formed in another manner, with various shapes, flat products (plates, sheets, tape), bars, castings, forgings made of various metallic materials, protected or unprotected against corrosion by coating
	- for use in metal or combined metal-concrete constructions
	-
	2+

	
	Metal construction elements, including metal sections cut to size for specific purposes
Sets of construction elements. These products may be protected or unprotected against corrosion by coating
	- for use in constructions and foundations
	-
	2+

	
	Welding materials
	- for use in metal constructions
	-
	2+

	
	Structural connectors: Metallic rivets, bolts, screws (nuts and washers), high strength friction grip bolts, studs
	- for use in metal constructions
	-
	2+

	21.
	Internal & external wall and ceiling finishes, internal partition kits.

	
	Plates
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	A1**, A2**
B**, C**, D, E
	3

	
	
	
	(Al – E)***,F
	4

	
	
	- for remaining applications
	-
	3

	
	Plates
Covering in roll form

Floor coverings

Facing
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	A1**, A2**
B**, C**, D, E
	3

	
	
	
	(Al – E)***,F
	4

	
	
	- for use subject to the requirements on safe operation and/or the provisions on the requirements on hazardous substances
	-
	3

	
	
	- for remaining applications
	-
	4

	
	Suspended ceilings (sets)
	for applications subject to the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	A1**, A2**
B**, C**, D, E
	3

	
	
	
	(Al – E)***,F
	4

	
	
	- for remaining applications
	-
	3

	
	Special profiles, fastening frames
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	A1**, A2**
B**, C**, D, E
	3

	
	
	
	(Al – E)***,F
	4

	
	
	 - to be used subject to the requirements on safe operation 
	-
	3

	
	
	- for remaining applications
	-
	4

	
	Sets for preparing partition walls
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(Al – E)***, F
	4

	
	
	- for remaining applications
	-
	3

	22.
	Roof coverings, roof lights, roof windows and ancillary products.

	
	Flat and profiled sheet. Roof tiles, roof slates, roof shingles. Factory-bonded composite or sandwich panels
Rooflights

Self-supporting roof sets

Roof windows
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- to be used in compliance with the resistance to external fire
	BROOF(t1)8)
	3

	
	
	
	BROOF9)
	4

	
	
	 for remaining applications
	-
	3

	
	Roof access systems, walkways and footholds, roof safety hooks and anchorages
Asphalt roofing cements
	- to be used in compliance with the resistance to external fire
	BROOF(t1)8)
	3

	
	
	
	BROOF9)
	4

	
	
	- for remaining applications 
	-
	3

	
	Roof accessories
	- to be used in compliance with the resistance to external fire
	BROOF(t1)8)
	3

	
	
	
	BROOF9)
	4

	
	
	- for remaining applications
	-
	4

	
	Eaves and ceilings products
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for use subject to the requirements on hazardous substances
	-
	3

	
	
	- for remaining applications
	-
	4

	
	Fastenings/mechanical connectors for roofing
	- for all applications
	-
	2+

	
	Kits of mechanically fastened flexible roof waterproofing membranes, together with the system for fastening, connecting, edge processing with or without thermal insulation
	- for use as roof waterproofing protection
	-
	2+

	23.
	Road construction products

	
	Asphalt 
	- for road construction and reinforcing road surfaces
	-
	2+

	
	Mineral-asphalt mixtures
Products for reinforcing surfaces


	- for road construction and reinforcing road surfaces subject to the requirements on reaction to fire 
	A1*fl, A2*fl, B*fl, C*fl 
	1

	
	
	
	A1**fl, A2**fl, B**fl, C**fl, Dfl, Efl
	3

	
	
	
	(A1fl – Efl)***, Ffl
	4

	
	
	- for remaining purposes for road construction and reinforcing road surfaces
	-
	2+

	
	Water protection products and sets used for bridge surfaces
	- for bridge surfaces
	-
	2+

	
	Ancillaries
	- for concrete roads
	-
	4

	24.
	Aggregates

	
	Aggregates for:
- concrete, mortar, grout,

- mineral-asphalt mixtures and reinforcement of surfaces,
- mixtures hydraulically unbound and bound,

- concrete mortar and cement grout

Stone for engineering works and armourstone

Fillers for:

- concrete, mortar, grout,

- mineral-asphalt mixtures and reinforcement of surfaces,

- concrete mortar and cement grout
	- for works requiring a high level of safety
)

	-
	2+

	
	Aggregates for:
- concrete, mortar, grout,

- mineral-asphalt mixtures and reinforcement of surfaces,
- mixtures hydraulically unbound and bound,

- concrete mortar and cement grout

Stone for engineering works and armourstone

Fillers for:

- concrete, mortar, grout,

- mineral-asphalt mixtures and reinforcement of surfaces,

- concrete mortar and cement grout
	- for works not requiring a high level of safety
	-
	4

	
	Ballast
	- for rail surfaces 
	-
	2+

	25.
	Construction adhesives

	
	Structural building adhesives
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(Al – E)***, F
	4

	
	
	- for remaining applications 
	-
	2+

	
	Adhesives for tiles
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(Al – E)***, F
	4

	
	
	- for remaining applications
	-
	3

	
	Adhesives for floor products
Adhesives for internal wall, external wall and ceiling finishing 
	- to be used in compliance with the requirements on reaction to fire 
	Al*, A2*, B*, C*
	1

	
	
	
	Al**, A2**, B**, C**, D, E
	3

	
	
	
	(Al – E)***, F
	4

	
	
	- for use in buildings and other building structures
	-
	4

	26.
	Concrete and products related to concrete, mortar and grout

	
	Ready-mixed concrete
	- for structural use7) 
	-
	2+

	
	
	- for remaining applications
	-
	4

	
	Admixtures
	- for concrete, mortar and grout 
	-
	2+

	
	Type I additives
)

	- for concrete, mortar and grout 
	-
	2+

	
	Type II additives


	- for concrete, mortar and grout 
	-
	1+

	
	Fibres
	- for concrete, mortar and grout construction purposes
	-
	1

	
	
	- for other uses with concrete, mortar and grout 
	-
	3

	
	Products for surface protection and structural and non-structural repair of concrete 
	- for use with low performance requirements
	-
	4

	
	
	- for other uses in buildings and structures
	-
	2+

	
	
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(Al – E)***, F
	4

	27.
	Space heating appliances

	
	Space heating appliances:
- with no internal source of energy (with the exception of electric and gas devices);

- for solid and liquid fuels

Solar panels

Sets/systems for solar heating
	- for use in buildings
	-
	3

	
	
	- to be used in compliance with the requirements on reaction to fire
	Al*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 – E)***, F
	4

	28.
	Pipes-tanks and ancillaries not in contact with water intended for human consumption
)

	
	Gas and liquid fuel transport/distribution/storage products earmarked for powering heating/cooling systems in buildings, from an external storage tank or from a network end pressure reduction station to the inlet leading to devices for heating/cooling in a building: pipe sets, pipes, fittings, tanks, leakage alarm systems and devices preventing the overfilling of connector elements, binders, welds, weld sealing, gaskets, safety canals and ducts, pipe/duct supports, regulating and cut-off fittings, safety auxiliary equipment
	- for use in installations in places subject to the requirements on fire safety
	-
	1

	
	
	- for remaining applications
	-
	3

	
	Products for the transport/ removal/storage of water not earmarked for human consumption: pipe sets, pipes, fittings, tanks, leakage alarm systems and devices preventing the overfilling of connector elements, binders, welds, weld sealing, gaskets, safety canals and ducts, pipe/duct supports, regulating and cut-off fittings, safety auxiliary equipment
	- for use in installations in places subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- in installations and in heating networks
	-
	3

	
	
	- for remaining applications
	-
	4

	29.
	Construction products in contact with water intended for human consumption18) 
)

	
	Sets (pipe installations and storage systems)
Pipes

Tanks

Fittings, water supply fittings, safety devices

Binders, welds, connectors, weld sealings, gaskets
	- for use in installations and networks in places subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	4

	30.
	Flat glass, profiled glass and glass block products

	
	Flat glass and formed glass plates
Profiled glass

Insulating glass units
	- for use in compliance with the requirements on resistance to fire
	-
	1

	
	
	- to be used in compliance with the requirements on reaction to fire
	A1, A2, B, C, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- to be used in compliance with the resistance to external fire
	BROOF(t1)8)
	3

	
	
	
	BROOF9)
	4

	
	
	- for use as armoured and explosion-proof glazing
	-
	1

	
	
	- for remaining applications 
	-
	3

	
	Wall tiles of glass pavers
	- for use when marking out fire zones
	-
	1

	
	
	- to be used in compliance with the requirements on reaction to fire
	A1, A2, B, C, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for use subject to the requirements on safe operation and/or requirements on the saving of energy or reducing noise
	-
	3

	
	
	- for remaining applications
	-
	4

	
	Glass pavers
	- for use in compliance with the requirements on reaction to fire
	A1, A2, B, C, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for use as armoured and explosion-proof glazing
	-
	1

	
	
	- for use subject to the requirements on the saving of energy or reducing noise
	-
	3

	
	
	- for remaining applications
	-
	4

	31.
	Sealants for joints

	
	Sealants for joints, expansion joints, installations passages and work cracks in interior walls, foundations, beams and floors
	- for use subject to the provisions on reaction to fire
	A1(*), A2(*), B(*), C(*)
	1

	
	
	
	A1(**), A2(**), B(**), C(**), D, E
	3

	
	
	
	(A1 do E) (***), F
	4

	
	
	- for use on the exterior of buildings
	-
	3

	
	
	- for use on the interior of buildings
	-
	4

	
	Sealant (putty) for glazing, with the exception of construction sealants (putties)
Products for sealing welds in floors and interior walls

Products for sealing sanitary devices and other connections in sanitary space
	- for use in the construction industry subject to the provisions on reaction to fire
	A1(*), A2(*), B(*), C(*)
	1

	
	
	
	A1(**), A2(**), B(**), C(**), D, E
	3

	
	
	
	(A1 do E) (***), F
	4

	
	
	- for remaining purposes in the construction industry
	-
	4

	32.
	Fixings

	
	Anchors for use in concrete (heavy type)
)
Metal anchors affixed to wall structures
	- for structural use3)
	-
	1

	
	Anchors for use in concrete (heavy type)
)
Plastic anchors for use with concrete and wall structures
	- for non-structural purposes
	-
	2+

	
	Anchors for affixing steel and sheet elements
Anchors affixed dynamically

Anchors for composite panels
	- for non-structural purposes
	-
	2+

	33.
	 Building kits, units and prefabricated elements

	
	Sets/systems of non-load bearing shuttering to be filled with concrete
	- for the making of external and internal barriers, subject to the requirements on fire safety
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E, (A1 -E)***, F
	2+

	
	
	- for the making of external and internal barriers, not subject to the requirements on fire safety
	-
	2+

	
	Sets for making prefabricated stairs
	- for use in buildings subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining purposes in buildings
	-
	2+

	
	Banisters and railing providing protection against falling
	- for use in the construction industry subject to the provisions on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- to be used subject to the requirements on safe operation
	-
	3

	
	
	- for remaining applications 
	-
	4

	
	Drainpipe systems and their elements
	- for use in buildings
	-
	4

	
	Product sets for the construction of frame buildings with the use of timber and of buildings with the use of beams
	- for use in the construction industry
	-
	2+

	
	Product sets for the construction of frame buildings with the use of metal or concrete
	- for use in the construction industry
	-
	1

	
	Sets/systems of prefabricated items for structures or their parts
	- for use in the construction industry
	-
	1

	
	Sets for protection against crumbling rock
	- for use in engineering structures
	-
	1

	
	Sandwich panels
	- for structural use7)
	-
	1

	
	
	- for use in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	3

	
	Pales, anchors and soil nails
	- structural foundations, reinforcements and ground anchoring 
	-
	2+

	
	Steel mesh and baskets for gabions
	- for use in building structures
	-
	2+

	
	Flooring blocks other than prefabricated blocks of common concrete/light concrete/cellular concrete
	- for structural use7)
	-
	2+

	
	
	- for use in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	4

	
	Elastomer elements, including rubber and plastic6)
	- for rail surfaces
	-
	2+

	34.
	Fire stopping, fire sealing and fire protective products. | fire retardant products 

	
	Separating or sealing products in case of fire. Products used for fire protection (including coatings)
	- for use in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	1

	35.
	Products for protection against metal corrosion and biological corrosion

	
	Coating products/sets for protection against the corrosion of metal structures
Products/sets for pressure and non-pressure impregnation penetration of timber structures for protection against biological corrosion 
	- for use in compliance with the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications
	-
	3

	36.
	Ventilation and air-conditioning products 

	
	Products for distribution and division of air: ventilation ducts and pipes
	- for use in buildings subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications in buildings
	-
	3

	
	Products for distribution and division of air: dampers, bolts, intake vents and uptake ventilators, air intakes, expulsion devices, stays, locks
Air bricks
	- for use in buildings subject to the requirements on reaction to fire
	A1*, A2*, B*, C*
	1

	
	
	
	A1**, A2**, B**, C**, D, E
	3

	
	
	
	(A1 - E)***, F
	4

	
	
	- for remaining applications in buildings
	-
	4

	37.
	Supply, steering and communication cables 

	
	Supply, steering and communication cables
)

Covers for protecting supply, steering and communication cables22)
	- for use in compliance with the requirements on reaction to fire
	Aca, B1ca, B2ca, Cca
	1+

	
	
	
	Dca, Eca
	3

	
	
	
	Fca
	4

	
	
	- for use in compliance with the requirements on resistance to fire
	-
	1+

	
	Cable sets (supply cables, steering cables and communication cables and fixings) for systems supplying and steering devices used for fire protection
	- for use in compliance with the requirements on resistance to fire
	-
	1+


*Products or materials which, during an unambiguously identified stage of the production process, are subject to modification, and which by assumption improve classification in terms of reaction to fire (e.g. by adding flame retardants or reducing organic matter content).

**Products or materials which, during an unambiguously identified stage of the production process, are not subject to modification, and which by assumption improve classification in terms of reaction to fire (e.g. by adding flame retardants or reducing organic matter content).
***Products or materials which do not require tests in terms of reaction to fire (in keeping with the appropriate decisions and Commission delegated Regulations).

A1, A2, B, C, D, E, F – classes of reaction to fire performance for construction products excluding floorings and cables in keeping with Commission delegated Regulation (EU) 2016/364 of 1 July 2015 on the classification of the reaction to fire performance of construction products pursuant to Regulation (EU) No 305/2011 (OJ L. 68 of 15 March 2016, p. 4.
A1fl, A2fl, Bfl, Cfl, Dfl, Efl, Ffl – classes of reaction to fire performance for construction products for floorings in keeping with Commission delegated Regulation (EU) 2016/364 of 1 July 2015 on the classification of the reaction to fire performance of construction products pursuant to Regulation (EU) No 305/2011 (OJ L. 68 of 15 March 2016, p. 4).
Aca, B1ca, Bca, Cca, Dca, Eca, Fca – classes of reaction to fire performance for construction products for cables in keeping with Commission delegated Regulation (EU) 2016/364 of 1 July 2015 on the classification of the reaction to fire performance of construction products pursuant to Regulation (EU) No 305/2011 (OJ L. 68 of 15 March 2016, p. 4).
ANNEX 2

SAMPLE
NATIONAL DECLARATION ON PERFORMANCE PROPERTIES

No …………………….

1.
Name and trade name of the construction product:


2.
Marking of the construction product
): 


3.
Intended application or applications:


4.
Name and address of the producer’s registered address and product manufacturing site:


5.
Name and address of the registered office of the authorised representative, if applicable:


6.
National system for the assessment and verification of the constancy of performance properties: 


7.
National technical specifications:

7a. Product Polish Standard:


Name of the accredited certification body, accreditation number and national certificate number or the accredited name of the laboratory/laboratories and the accreditation number
):


7b. National technical assessment:


Technical assessment unit/National technical assessment unit:


Name of the accredited certification body and the certificate number2):


8. 
Declared performance properties:

	Essential characteristics of the construction product for the intended application or applications
	Declared performance properties
	Comments
)

	
	
	

	
	
	

	
	
	

	
	
	


9.
The performance properties of the above described product comply with all of the declared performance properties, referred to in (8). This national declaration of performance properties shall be issued in keeping with the Construction Products Act of 16 April 2004, on the exclusive responsibility of the producer.

Signed on behalf of the producer:
	


	(name and surname and position)

	

	


	(place and date of issue)
	(signature)


�)	The Minister of Infrastructure and Construction manages the government administration department on construction, planning, zoning and housing, pursuant to § 1(2)(1) of the Regulation of the Prime Minister of 17 November 2015 on the specific scope of activities of the Minister of Infrastructure and Construction (Journal of Laws, (1907) and (2094)).


�)	Notification of this Regulation was made to the European Commission on […], under number […], pursuant to § 4 of the Regulation of the Council of Ministers of 23 December 2002 concerning the manner in which the national notification system of standards and legal acts functions (Journal of Laws, No. 239(2039) and of 2004, No 65(597)), which implements Directive 2015/1535/EU of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on Information Society Services (uniform text) (OJ L 241 of 19 September 2015, p. 1).





�)	Amendments to the joint text of the referred to Act were notified in the Journal of Laws of 2015 (699), (875), (978), (1197), (1268), (1272), (1618), (1649), (1688), (1712) and (1844) and of 2016 (65).


�	This Regulation was preceded by the Regulation of the Minister of Infrastructure of 11 August 2004 on the manner of declaring the compliance of construction products and the manner of marking them with construction marks (Journal of Laws, No 198(2041), and of 2006, No 245 (1782)) which, on the basis of Article 4 of the Act of 13 June 2013 on the Amendment of the Construction Products Act and the System of Assessing Conformity Act (Journal of Laws (898) and of 2015 (1165)) shall lose force on the day of entry into force of this Regulation.


�	The obligation to draw up the national declaration on performance properties concerns construction products not covered by Article 5(1) and Article 5(3) of the Construction Products Act of 16 April 2004, which means that construction products covered by standards harmonised or compliant with the European technical assessments issued for them, which are placed on the market or made available on the national market, in keeping with Regulation (EU) No 305/2011 of the European Parliament and of the Council of 9 March 2011 laying down harmonised conditions for the marketing of construction products and repealing Council Directive 89/106/EEC (OJ L 88 of 4 April 2011, p. 5), hereinafter “Regulation No 305/2011”, and construction products placed on the market i.e. the so-called “mutual recognition clause”, are not covered by the scope of this Annex.


�)	Does not apply to construction products defined as structures in the Regulation of the Minister of Infrastructure and Development of 13 May 2014 on the authorisation for operation of given types of structures, devices and rail vehicles (Journal of Laws of 2014 (720)).


�)	The structural application concerns elements, products and their sets, which impact the fulfilment of basic requirements relating to load capacity and stability in relation to building structures in keeping with Regulation No 305/2011.


�)	BROOF (t1) – in accordance with Annex 3 (4.1)(1) of the Regulation of the Minister of Infrastructure of 12 April 2002 on the technical conditions of buildings and their location (Journal of Laws of 2015 (1422).


�)	BROOF – in accordance with Annex 3 (4.1)(2) of the Regulation of the Minister of Infrastructure of 12 April 2002 on the technical conditions of buildings and their location (Journal of Laws of 2015 (1422).


�)	“Decisive” means that failure to meet the requirements may cause the construction of the structure to exceed limits on usability and/or load capacity.


�)	“Not decisive” means that failure to meet the requirements cannot cause the construction of the structure to exceed limits on usability and/or load capacity.


�)	type I – with a mechanical connection to transfer its own sealing burden onto the support frame and from there to the structure. The structural sealant transfers all other impact. Used as a safety measure in the event of sealant disintegration.


type III – the structural sealant transfers all impact, including its own sealing weight, onto the support frame and from there to the structure. Used as a safety measure in the event of sealant disintegration.


� type II – with mechanical connection to transfer own sealing burden onto the support frame and from there to the structure. The structural sealant transfers all other impact. Nie stosuje się zabezpieczeń na wypadek rozerwania spoiny. No safety measures are used in the event of sealant disintegration.


type IV – the structural sealant transfers all impact, including its own sealing weight, onto the support frame and from there to the structure. No safety measures are used in the event of sealant disintegration.


�)	Category I masonry units – masonry units of compressive strength declared with probability of reduced strength of no more than 5 %.


�)	Application requiring a high level of safety connected with an expected level of certainty and reliability by the party applying the construction product in terms of the constancy of performance properties of the product, as declared by the producer.


�)	Additives for increasing the volume of grout.


� Additives which modify the properties of concrete admixture or concrete.


�)	Does not apply to products earmarked exclusively for application in industrial processes.


�)	Product performance properties relating to product sanitary properties (capable of coming into contact with water for human consumption) are defined under the Collective Supply of Water and Collective Sewage Disposal Act of 7 June 2001 (Journal of Laws of 2015 (139) and (1893)) and the Regulation of the Minister of Health of 13 November 2015 on the quality of water earmarked for consumption human consumption (Journal of Laws of 2015 (1989)).


�)	Anchors used for affixing or supporting structural elements or heavy elements, such as casings, suspended ceilings.


�)	Anchors used for affixing to concrete non-structural elements such as light suspended ceilings, installations.





�)	National systems for the assessment and verification of the constancy of performance properties mentioned in column 5 exclusively refer to the assessment and verification of the constancy of performance properties relating to reaction and resistance to fire.








�) In keeping with the national systems for the assessment and verification of the constancy of performance properties defined under § 4 of the Regulation of the Minister of Infrastructure and Construction of… on the manner of declaring the performance properties of construction products and the manner of marking them with construction marking (Journal of Laws ()), the producer shall define the type of construction product, for which he is drawing up the national declaration on performance properties. The manner of marking a construction product defined in this way in the national declaration on performance properties is determined by the producer. This marking should be connected with the product type, in other words with the set of levels or classes of performance properties and the intended application of the product, as defined in the national declaration. The marking should be unique in reference to types of construction products manufactured by the given producer.


�) Fill in if the certification body or laboratory/laboratories took part in the adopted national system of assessment and verification of construction product constancy of performance properties.


�) In the event of applying provision § 5(1)(1) and (2) and (2) of this Regulation, in column three it must be stated which of the above-mentioned provisions in reference to the fundamental product characteristics was applied.





