# Regulations relating to universal design of motor vehicles used for licensed transport etc.

The proposed amendments are highlighted in red

Section 1: Scope of application

These Regulations apply to passenger transport requiring a licence on regular services and transport requiring a licence where the motor vehicle is used for transport for persons with disabilities, cf. Sections 4, 6 and 9 of the Professional Transport Act. The Regulations do not apply to Class I buses, with the exception of the provision in the fifth paragraph of Section 4

Section 2 Definitions

The definitions in Regulations of 5 July 2012 No. 817 relating to Approval of Motor Vehicles and their Trailers (“bilforskriften”) shall apply where appropriate.

Otherwise the following definitions shall apply to these regulations:

| a) | Motor vehicle for transport of persons with disabilities: Motor vehicle in licensed transport that is specially designed or equipped for transport for disabled people. |
| --- | --- |

| b) | Passenger space: Space intended for the transport of passengers. |
| --- | --- |

| c) | Seat: Space intended for seated passengers in fixed or removable furniture. |
| --- | --- |

| d) | Wheelchair space: Space intended for passenger seated in a wheelchair. |
| --- | --- |

Section 3. Approval

Buses covered by these Regulations must be approved by the Norwegian Public Roads Administration pursuant to the requirements set out in the Regulations relating to Motor Vehicles and their Trailers, as well as the requirements in Section 4. Upon approval, documentation must be provided that the requirements in Section 4 are met.

Passenger cars covered by these Regulations must be approved by the Norwegian Public Roads Administration pursuant to the requirements set out in the Regulations relating to Motor Vehicles and their Trailers, as well as the the requirements in Section 6. Upon approval, documentation must be provided that the requirements in Section 6 are met.

Documentation as mentioned in the first and second paragraphs shall be issued by the manufacturer of the bodywork, chassis, safety equipment or the complete motor vehicle, or by a competent independent laboratory.

Section 4: Requirements applying to buses

Buses covered by these Regulations shall comply with the requirements of UN/ECE Regulation 107-05, Annex 8.

In addition, buses used for passenger transport on regular services requiring a licence shall have the following:

a) Automatic stop announcement with voice messages and an electronic message board inside the bus located so that it can be clearly seen from a reserved space.

b) External broadcasting system (audio beacon) announcing which bus is arriving at the stop.

The choice of boarding device must be based on the structure of the bus. There shall be at least one wheelchair space in each bus.

Class II buses shall have seat belts that meet the requirements of UN/ECE Regulation No. 16-06. Attachment points for safety belts shall meet the requirements of UN/ECE Regulation No. 14-07.

Class I, II and III buses shall from 1 January 2023 meet the requirements for frontal protection described in Section 5 of UN/ECE Regulation No. 29 when the collision test has been carried out in accordance with Annex 3 Test A where the impact value of the impactor shall be in accordance with Section 5.5.2.

Section 5. (Repealed)

Section 6. Requirements applying to passenger cars

Passenger cars shall satisfy the requirements of Annex XI, Appendix 3 “Wheel-chair Accessible Vehicles”, of Directive 2007/46/EC. In addition, the following shall apply:

| a) | Wheelchair spaces shall be in accordance with Annex 8 Section 3.6.1. of UN/ECE Regulation 107-05. |
| --- | --- |

| b) | Seats in the wheelchair area shall be in accordance with Annex 8 Section 3.7. of UN/ECE Regulation 107-05. |
| --- | --- |

| c) | Lighting shall be in accordance with Annex 3 Section 7.6.12. of UN/ECE Regulation 107-05. |
| --- | --- |

| d) | Lift, ramps and similar shall be in accordance with Annex 8 Section 3.11. of UN/ECE Regulation 107-05. |
| --- | --- |

| e) | Space for fire extinguishers and first-aid equipment shall be provided in accordance with Annex 3 Section 7.5.4. of UN/ECE Regulation 107-05. |
| --- | --- |

| f) | Work lights shall be in accordance with Section 28-5 (8) of the Regulations relating to the Technical Requirements and Approval of Vehicles. |
| --- | --- |

Section 7. Enforcement

Buses covered by these Regulations shall at all times comply with the requirements in Section 4 when carrying out licensed transport, cf. however Section 8.

Passenger cars covered by these Regulations shall at all times comply with the requirements in Section 6 when carrying out licensed transport, cf. however Section 8.

The Norwegian Public Roads Administration and the police carry out inspections to enforce these Regulations. In the event of inadequate equipment or use of materials that do not meet the requirements of the Regulations or perform adequately, Sections 40  and 41 of the Professional Transport Act shall apply accordingly.

Section 8. Exemptions

For vehicles covered by these Regulations, the Norwegian Public Roads Administration may in individual cases grant exemption from one or more of the requirements in Sections 4 and 6. No exemption may be made from the requirements for boarding aids, restraint systems (tie-down mechanisms) for wheelchairs, seat belts, fire extinguishing equipment, first aid equipment and work lights. For the requirements in Section 4, second paragraph, the licensing authority may also grant exemptions.

Section 9. Entry into force and transitional provisions

These Regulations shall enter into force on 1 January 2010.

Agreements on the purchase of new means of transport entered into after 1 January 2010 shall comply with the requirements of these Regulations. Means of transport that have been previously approved and that are altered or modified may be approved in accordance with Section 3.

Vehicles approved pursuant to Regulations of 4 January 1984 No.599 relating to licensed transport of disabled persons - technical vehicle requirements, will still be considered approved for licensed transport of disabled persons.

The Regulations of 4 January 1984 No. 599 relating to licensed transport of disabled persons - technical vehicle requirements, are repealed as of 1 January 2010.