1. ------IND- 2020 0473 F-- EN- ------ 20200729 --- --- PROJET

The XXXX 2020

Order of XXXX on the criteria, sub-criteria and scoring system for calculating and displaying the repairability index for televisions.

Reference number: XXXXX

The Minister for the Ecological Transition and the Minister for the Economy and Finance,

Having regard to Commission Regulation (EU) 2019/2021 of 1 October 2019 laying down ecodesign requirements for electronic displays pursuant to Directive 2009/125/EC of the European Parliament and of the Council, amending Commission Regulation (EC) No 1275/2008 and repealing Commission Regulation (EC) No 642/2009;

Having regard to the French Environmental Code, particularly Article L. 541-9-2 thereof;

Having regard to Decree No XXX of XXXX on the implementing rules for the repairability index for electrical and electronic products, its criteria and its calculation method;

Article 1

This Order applies to televisions falling within the scope of the aforementioned Regulation of 1 October 2019.

Article 2

The criteria, sub-criteria and scoring system applicable to the products defined in Article 1 and used to calculate the repairability index are specified below:

CRITERION No 1 - DOCUMENTATION

Sub-criterion 1.1 Commitment on the duration of the provision, free of charge, of the technical documentation, instructions for use and maintenance instructions

|  | Column A Producer |  |  |  | Column B Repairers |  |  |  | Column C Consumers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Years of availability |  |  |  | Years of availability |  |  |  | Years of availability |  |  |  |
|  | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more |
| Documentation type | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  |
| Unambiguous product identification | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Disassembly diagram or exploded view | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Wiring and connection diagrams | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Circuit board diagrams | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| List of required repair and test equipment | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Technical manual of repair instructions | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Error and diagnostic codes | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Component and diagnostic information | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Software instructions (including reset) | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Access to incidents reported and recorded in the equipment | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Technical bulletins | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Specific guidelines on self-repair (recommended operations, safety and repair instructions, possible repercussions on the warranty) |  |  |  |  |  |  |  |  | 0 | 7 | 9 | 11 |
| Information on access to professional repairers |  |  |  |  |  |  |  |  | 0 | 7 | 9 | 11 |
| Fault detection and required actions (general public approach) |  |  |  |  |  |  |  |  | 0 | 7 | 9 | 11 |
| Instructions for use and maintenance instructions |  |  |  |  |  |  |  |  | 0 | 7 | 9 | 11 |

The maximum number of points is 407. Score for this sub-criterion = (number of points obtained / 407) x 10

CRITERION No 2 - DISASSEMBLY AND ACCESS, TOOLS, FASTENINGS

Sub-criterion 2.1 Ease of disassembling the parts (list 2)

|  | Number of steps for individual access to the part |  |  |  |
| --- | --- | --- | --- | --- |
|  | ND / NA (1) or 4 or more | 3 | 2 | 1 |
| List 2 parts (external parts) | Number of points |  |  |  |
| Remote control (2) | 0 | 1 | 2 | 3 |
| External power sources |  |  |  |  |

|  | Number of steps for individual access to the part |  |  |  |
| --- | --- | --- | --- | --- |
|  | ND / NA (1) or 12 or more | 10 to 11 | 8 to 9 | 1 to 7 |
| List 2 parts (internal parts) | Number of points |  |  |  |
| Internal power sources (3) | 0 | 1 | 2 | 3 |
| Main board | 0 | 1 | 2 | 3 |
| Screen | 0 | 1 | 2 | 3 |

- ND / NA = not removable or not individually accessible

- Simply remove the battery or batteries

- To be greyed out in the case of external power sources

For internal power sources, the maximum number of points is 12. Score for this sub-criterion = (number of points obtained / 12) x 10

For external power sources, the maximum number of points is 9. Score for this sub-criterion = (number of points obtained / 9) x 10

Sub-criterion 2.2 Tools required to disassemble the parts (list 2)

|  | Type of tool |  |  |  |
| --- | --- | --- | --- | --- |
|  | ND / NA | Proprietary tools | Specific tools | Tool-free, common tools (4) |
| List 2 parts | Number of points (5) |  |  |  |
| Remote control | 0 | 1 | 2 | 4 |
| External power sources |  |  |  |  |
| Internal power sources (3) | 0 | 1 | 2 | 4 |
| Main board | 0 | 1 | 2 | 4 |
| Screen | 0 | 1 | 2 | 4 |

- To be greyed out in the case of external power sources

- or tool supplied with the spare part or with the product

- take the lowest score if several tools are involved

For internal power sources, the maximum number of points is 16. Score for this sub-criterion = (number of points obtained / 16) x 10

For external power sources, the maximum number of points is 12. Score for this sub-criterion = (number of points obtained / 12) x 10

Sub-criterion 2.3 Characteristics of the fasteners (for the assembly of list 1 and 2 parts)

|  | Type of fastener |  |  |
| --- | --- | --- | --- |
|  | Neither removable nor reusable | Removable, non-reusable | Removable and reusable (3) |
| List 1 or list 2 parts | Number of points (4) |  |  |
| Back cover | 0 | 1 | 2 |
| Wi-Fi module (5) | 0 | 1 | 2 |
| Bluetooth module (6) | 0 | 1 | 2 |
| Infrared receiver | 0 | 1 | 2 |
| Speakers | 0 | 1 | 2 |
| Connectors (7) | 0 | 1 | 2 |
| Remote control | 0 | 1 | 2 |
| Internal power sources (8) | 0 | 1 | 2 |
| External power sources (9) | 0 | 1 | 2 |
| Main board | 0 | 1 | 2 |
| Screen | 0 | 1 | 2 |

- or fastener supplied with the spare part

- take the lowest score if multiple fasteners are involved

- To be greyed out if not applicable

- To be greyed out if not applicable

- to connect external equipment (cable, antenna, USB, DVD and Blue-Ray)

- To be greyed out in the case of external power sources

- To be greyed out in case of internal power sources

The maximum number of points is 20. Score for this sub-criterion = (number of points obtained / 20) x 10

CRITERION No 3 - AVAILABILITY OF SPARE PARTS

Sub-criterion 3.1 Producer's commitment on the period of availability of the list 2 parts

|  | Column A Producer |  |  |  | Column B Spare parts distributors |  |  |  | Column C Repairers |  |  |  | Column D Consumers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Years of availability |  |  |  | Years of availability |  |  |  | Years of availability |  |  |  | Years of availability |  |  |  |
|  | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more |
| List 2 parts | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  |
| Remote control | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Internal power sources (1) | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| External power sources (2) | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Main board | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Screen | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |

- To be greyed out in the case of external power sources

- To be greyed out in case of internal power sources

The maximum number of points is 176. Score for this sub-criterion = (number of points obtained / 176) x 10

Sub-criterion 3.2 Producer's commitment on the period of availability of the list 1 parts

|  | Column A Producer |  |  |  | Column B Spare parts distributors |  |  |  | Column C Repairers |  |  |  | Column D Consumers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Years of availability |  |  |  | Years of availability |  |  |  | Years of availability |  |  |  | Years of availability |  |  |  |
|  | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more | 0 to 6 | 7 to 8 | 9 to 10 | 11 or more |
| List 1 parts | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  |
| Back cover | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Wi-Fi module (3) | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Bluetooth module (4) | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Infrared receiver | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Speakers | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |
| Connectors (5) | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 | 0 | 7 | 9 | 11 |

- To be greyed out if not applicable

- To be greyed out if not applicable

- to connect external equipment (cable, antenna, USB, DVD and Blue-Ray)

The maximum number of points is 264. Score for this sub-criterion = (number of points obtained / 264) x 10

Sub-criterion 3.3 Delivery time for list 2 parts

|  | Column A Producer |  |  |  | Column B Spare parts distributors |  |  |  | Column C Repairers |  |  |  |  | Column D Consumers |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delivery days (1) |  |  |  | Delivery days (1) |  |  |  | Delivery days (1) |  |  |  |  | Delivery days (1) |  |  |  |  |
|  | 11 or more | 6 to 10 | 4 to 5 | 1 to 3 | 11 or more | 6 to 10 | 4 to 5 | 1 to 3 | 11 or more | 6 to 10 | 4 to 5 | 1 to 3 | 11 or more |  | 6 to 10 | 4 to 5 | 1 to 3 |
| List 2 parts | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  |  | Number of points |  |  |  |  |
| Remote control | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Internal power sources (2) | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| External power sources (3) | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Main board | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Screen | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |

- working days from the day of the order

- To be greyed out in the case of external power sources

- To be greyed out in case of internal power sources

The maximum number of points is 48. Score for this sub-criterion = (number of points obtained / 48) x 10

Sub-criterion 3.4 Delivery time for list 1 parts

|  | Column A Producer |  |  |  | Column B Spare parts distributors |  |  |  | Column C Repairers |  |  |  |  | Column D Consumers |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delivery days (1) |  |  |  | Delivery days (1) |  |  |  | Delivery days (1) |  |  |  |  | Delivery days (1) |  |  |  |  |
|  | 11 or more | 6 to 10 | 4 to 5 | 1 to 3 | 11 or more | 6 to 10 | 4 to 5 | 1 to 3 | 11 or more | 6 to 10 | 4 to 5 | 1 to 3 | 11 or more |  | 6 to 10 | 4 to 5 | 1 to 3 |
| List 1 part | Number of points |  |  |  | Number of points |  |  |  | Number of points |  |  |  |  | Number of points |  |  |  |  |
| Back cover | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Wi-Fi module | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Bluetooth module | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Infrared receiver | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Speakers | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |
| Connectors (2) | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 |  | 1 | 2 | 3 |

- working days from the day of the order

- to connect external equipment (cable, antenna, USB, DVD and Blue-Ray)

The maximum number of points is 72. Score for this sub-criterion = (number of points obtained / 72) x 10

CRITERION No 4 - PRICE OF SPARE PARTS

Sub-criterion 4.1 Ratio of the price of list 2 parts compared with the price of the new product

Based on the ratio described in the Order of XXXX on the display methods, signage and general parameters for calculating the repairability index, the number of points obtained for this criterion is determined as follows:

- if the ratio is greater than 0.3 then the number of points is 0;

- if the ratio is less than 0.1 then the number of points is 100;

- if the ratio is between 0.1 and 0.3 then the number of points is determined according to the following correspondence table:

| Ratio | 0.1 | 0.11 | 0.12 | 0.13 | 0.14 | 0.15 | 0.16 | 0.17 | 0.18 | 0.19 | 0.2 | 0.21 | 0.22 | 0.23 | 0.24 | 0.25 | 0.26 | 0.27 | 0.28 | 0.29 | 0.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Points | 100 | 95 | 90 | 85 | 80 | 75 | 70 | 65 | 60 | 55 | 50 | 45 | 40 | 35 | 30 | 25 | 20 | 15 | 10 | 5 | 0 |

The rounding rule is as follows:

- If the number of the third decimal place is less than 5, round down to two decimal places;

- If the number of the third decimal place is greater than or equal to 5, round up to two decimal places.

The maximum number of points is 100. Score for this sub-criterion = (number of points obtained / 100) x 10

CRITERION No 5 - SPECIFIC CRITERION

For the products concerned by this Order, the coefficients of the sub-criteria of criterion 5 are defined as follows:

| Criterion | Sub-criterion | Sub-criterion score | Sub-criterion coefficient | Criterion score | Criterion coefficient |
| --- | --- | --- | --- | --- | --- |
| 5. Specific criterion | 5.1. Accessibility of the usage meter | ▀▀/10 | 1 | ▀▀/20 | 1 |
|  | 5.2. Free remote assistance | ▀▀/10 | 0.5 |  |  |
|  | 5.3. Possibility of resetting software | ▀▀/10 | 0.5 |  |  |

Sub-criterion 5.1 Accessibility of the usage meter

A usage meter is a display device for consumers that cumulatively records product use in numbers of a given unit. In this Order, the unit in question is the number of hours of operation of the screen.

|  | Column C Consumers |  |  |
| --- | --- | --- | --- |
| Accessibility of the usage meter | Situation |  |  |
|  | Absence | Difficult to access (1) | Visible or easily accessible (2) |
|  | Number of points |  |  |
|  | 0 | 1 | 2 |

- the consumer must make more than three clicks to view the usage meter value.

- the consumer must make three clicks or fewer to view the usage meter value.

The maximum number of points is 2. Score for this sub-criterion = (number of points obtained / 2) x 10

Sub-criterion 5.2 Free remote assistance

|  | Column B Repairers |  | Column C Consumers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of remote assistance | None | Up-to-date information on website | None | Remote information | Remote diagnostic assistance | Remote repair assistance |
| Number of points | 0 | 1 | 0 | 1 | 2 | 4 |

The maximum number of points is 5. Score for this sub-criterion = (number of points obtained / 5) x 10

Sub-criterion 5.3 Possibility of resetting software

|  | Column A Producer |  | Column B Repairers |  | Column C Consumers |  |
| --- | --- | --- | --- | --- | --- | --- |
| Possibility of resetting software free of charge and without restrictions on access to these services | Not possible | Possible | Not possible | Possible | Not possible | Possible |
|  | Number of points |  | Number of points |  | Number of points |  |
| Operating system reset (2) | 0 | 1 | 0 | 1 | 0 | 1 |
| Firmware reset (2) | 0 | 1 | 0 | 1 | 0 | 1 |

The maximum number of points is 6. Score for this sub-criterion = (number of points obtained / 6) x 10

The Minister for the Ecological Transition:

For and on behalf of the Minister, the Commissioner General for Sustainable Development,

T. Lesueur

The Minister for the Economy and Finance:

For and on behalf of the Minister, the Director-General for Competition Policy, Consumer Affairs and Fraud Control,

V. Beaumeunier