D R A F T

1. ------IND- 2020 0611 DK-- EN- ------ 20201008 --- --- PROJET
Order on the regulation of certain industrial greenhouse gases

The following is laid down by virtue of § 30; § 45(1); and § 59(4) of the Chemicals Act (see Consolidation Act No 115 of 26 January 2017):
Scope of the Order
§ 1. The Order covers perfluorocarbons (PFCs) and sulfur hexafluoride (SF6) and hydrofluorocarbons (HFCs) with the exception of hydrofluoroolefins (HFO-s), both the substances alone and in mixtures containing one or more of these substances.
Paragraph 2. The Order shall not cover the industrial use of the greenhouse gases referred to in paragraph 1, except for use as protective gas in light-metal foundries and for the production of flexible foam plastics. In this Order, “industrial use” is understood to mean use in the production of products, where greenhouses gas is not present in the end-product.
Restrictions of use, etc.
§ 2. The import, sale and use of new products containing the greenhouse gases mentioned in § 1(1) is prohibited.
Paragraph 2. Notwithstanding paragraph 1, the import, sale and use of new products listed in Annex 1 is permitted for the greenhouse gases stated therein.
§ 3. The import, sale and use of the greenhouse gases, new or recovered, mentioned in § 1(1) is prohibited.
Paragraph 2. Notwithstanding paragraph 1, the import, sale and uses listed in Annex 2 are permitted for the specified greenhouse gases.
Administrative provisions
§ 4. Oversight and monitoring of compliance with the rules of the notice, pursuant to the provisions, is conducted by the Environmental Protection Agency.
Paragraph 2. In exceptional cases, the Danish Environmental Protection Agency may permit derogation from the rules of the Order.
Paragraph 3. Appeals against the Danish Environmental Protection Agency’s decisions under Paragraph 2 may not be lodged with another administrative authority.
Penalties and entry into force
§ 5. Unless higher penalties are stipulated under other legislation, fines will be imposed on those who:
1) import, sell or use new products containing greenhouse gases in violation of § 2 or
2) import, sell or use greenhouse gases in contravention of § 3.
Paragraph 2. The penalty may be increased to imprisonment for up to two years if the violation was committed wilfully or through gross negligence, and if the violation resulted in or was intended to result in financial gain, including savings, for the perpetrators themselves or for others.
Paragraph 3. Entities, etc. (legal persons) may be rendered criminally liable in accordance with the provisions in Chapter 5 of the Penal Code [Straffeloven].
§ 6. This Order will enter into force on 1 July 2021.
Paragraph 2. Order No 1326 of 19 November 2018 on the regulation of certain industrial greenhouse gases is repealed.
Annex 1
The following products with the specified greenhouse gases are exempt from the prohibition in § 2
	Product
	Greenhouse gas

	1. 
	High voltage switchgear (voltages exceeding 1 kV). 
	SF6

	2. 
	Heat pumps with a charge of 50 kg or less and which have been fully factory-assembled as compact units that are primarily assembled with welds or soldering.
	HFCs

	3. 
	Cooling systems for heat pumps with a charge of 50 kg or less and which have been fully factory-assembled as compact units that are primarily assembled with welds or soldering.
	HFCs

	4. 
	Heat pumps not covered by point 2, refrigeration equipment, air conditioning systems (comfort cooling) and dehumidifiers with charges equal to, or less than, 5 tonnes of CO2 equivalent
.
	HFCs

	5.
	Cooling systems not covered by point 4 for heat pumps with a charge of between 0.15 kg and 10 kg and which have been fully factory-assembled as compact units that are primarily assembled with welds or soldering.
	HFCs



	6. 
	Vaccine chillers.
	HFCs

	7. 
	Mobile refrigeration equipment.
	HFCs

	8. 
	Air conditioning systems in vehicles and aircraft.
	HFCs

	9. 
	Low temperature freezers (temperatures below -50 °C).
	HFCs

	10. 
	Medical aerosols.
	HFCs

	11. 
	Laboratory equipment.
	All

	12. 
	Test facilities for the testing of cooling equipment.
	HFCs

	13. 
	Automatic control devices (thermostats, valves, etc.).
	All

	14. 
	Products for military use.
	All

	15. 
	Products for use on board ships.
	All


Annex 2
The following uses of the specified greenhouse gases are exempt from the prohibition in § 3
	Use
	Greenhouse gas

	1. 
	Servicing of refrigeration equipment, air conditioners, heat pumps and dehumidifiers.
	HFCs

	2. 
	Servicing of air conditioning in vehicles or aircraft.
	HFCs

	3. 
	Servicing of high voltage switchgear (voltages exceeding 1 kV).
	SF6

	4. 
	Servicing of laboratory equipment.
	All

	5. 
	Military use.
	All

	6. 
	Shipboard incineration.
	All


�A note on notification in accordance with the Information Procedure Directive is to be inserted here once such notification has been made.


�The quantity and CO2 equivalent shall be determined in accordance with the rules applicable to labelling in accordance with Article 12(3)(c) of Regulation (EU) No 517/2014 of the European Parliament and of the Council of 16 April 2014 on fluorinated greenhouse gases and repealing Regulation (EC) No 842/2006.





