1. ------IND- 2012 0543 SK- EN- ------ 20121016 --- --- PROJET

New version

ACT

of .................... 2012

on construction products

The National Council of the Slovak Republic has adopted this Act:

Section 1

Subject of the regulation

- This Act regulates the:

- appointment of a technical assessment body and notifies the procedures for its appointment and monitoring its activities to the European Commission (hereinafter referred to as the ‘Commission’), other Member States of the European Union and States which are party to the Treaty on the European Economic Area) (hereinafter referred to as a ‘Member State’) and

- notification of the legal entity (hereinafter referred to as a ‘notified person’) and notifies the procedure of notifying and monitoring its activities to the Commission and other Member States.)

- This Act further provides:

- the conditions for placing and making available construction products) (hereinafter referred to as the ‘product’) onto the market in the Slovak Republic (hereinafter referred to as the ‘domestic market’),

- for authorisation of a legal entity (hereinafter referred to as an ‘authorised person’) for the technical assessment and the assessment and verification of the consistency of the essential product characteristic parameters declared by the manufacturer (hereinafter referred to as the ‘parameter assessment’) which separate legislation) does not relate to and sets out the rights and obligations of authorised persons,

- for the rights and obligations of manufacturers,)

- the responsibilities of state administrative authorities.

Section 2

Placement of products on the domestic market

- Only products which fulfil the following may be placed on the domestic market:

- the harmonised conditions in accordance with separate legislation,) or

- the conditions in accordance with this Act.

- The conditions indicated in Paragraph 1 need not apply to products produced as follows:

- individually or processed in a non-serial manner according to the customer’s individual requirements on the basis of a specific order in which the builder responsible for safely integrating the product into the construction does so in accordance with the technical regulations and the manufacturer’s instructions and under the responsibility of the construction manager and the people who did the work necessary to process and integrate the product,

- directly at the construction site for the purpose of building them into a specific structure in accordance with the manufacturer’s technical specifications and instructions and under the responsibility of the construction manager and the people who did the work necessary to process and integrate the product,

- by traditional means using methods in accordance with the requirements of historical building conservation, or using production processes which are not industrial in nature, in particular for repairing buildings under historical conservation protection or buildings in a historical conservation zone, or due to the building’s architectural or historical value, if its use is in accordance with the technical and safety regulations.

- If the product parameters are based on a harmonised standard or European Technical Assessment,) then the Slovak technical standards) determined in accordance with Section 13 Paragraph 1 Letter c) to be suitable for assessing the product’s essential characteristics (hereinafter referred to as the ‘defined standard’) may not be used for procedures in accordance with this Act, nor may technical assessments issued in accordance with Section 3 (hereinafter referred to as ‘SK technical assessments’) be used.

- If a product manufactured outside the Slovak Republic is placed on the domestic market, then it will be necessary to enclose a translation into the national language) with the instructions on product use, product safety instructions, information on the health and safety risks which the product poses in ordinary use, as well as a declaration of the parameters under separate legislation).

Section 3

Technical assessment

- The technical assessment is the procedure for issuing an SK technical assessment, which is used if there is no harmonised standard, European Technical Assessment or defined standard for the product which the manufacturer intends to place on the domestic market.

- The technical assessment documents shall contain:

- an assessment of the essential product characteristic parameters declared by the manufacturer in terms of those parameters’ effect on at least one basic requirement for construction) and

- a positive evaluation of the parameters of the essential product characteristics which is relevant to the product’s suitability for its intended purpose.)

- Documented essential product characteristic parameters in SK technical assessments are expressed in numerical form in levels, by limit values in classes or by descriptions of them. The SK technical assessment is issued in the national language.

- The manufacturer shall assume the costs of the technical assessment.

Section 4

Parameter assessment

- The manufacturer must ensure that a parameter assessment is conducted before placing a product on the domestic market.

- The parameter assessment shall verify the consistency of the essential product characteristic parameters declared by the manufacturer.

- The parameter assessment shall use procedures in accordance with the parameter assessment systems which are being established for individual product groups by the generally applicable legislation issued to implement this Act in accordance with Section 13 Paragraph 2 (hereinafter referred to as the ‘implementing legislation’).

- The manufacturer shall assume the costs of the parameter assessment.

Section 5

Simplified parameter assessment

- When determining the product type, the manufacturer may replace type testing or type assessment with SK specific technical documentation which demonstrates that:

- the essential product characteristics achieve the determined parameter level or belong to the determined parameter class without the need for further testing or calculation, or

- the product corresponds to a type which has already been tested by another manufacturer and uses the test results obtained by that other manufacturer with his consent and with his vouching for their correctness.

- A manufacturer which is a micro-enterprise) may substitute product type determination based on type testing by the use of test methods which differ from the defined test methods pursuant to the defined standard if the use of the SK specific technical documentation demonstrates that the test methods used were equivalent to the test methods indicated in the defined standard.

- The SK specific technical documentation must demonstrate that the product has the parameters required for it to be considered as a product which achieves the level, class or threshold values under the defined standard.

Section 6

SK parameter declaration

- The manufacturer shall declare the parameters of the product’s essential characteristics in relation to the basic requirements for construction (hereinafter referred to as the ‘SK parameter declaration’), if established by the implementing legislation, for products placed on the domestic market. The SK parameter declaration shall be compiled in the national language.

- The SK parameter declaration shall contain at least:

- information on the manufacturer, including the commercial name, registered office and contact address, place of manufacture, company identification number (ID No), as well as the first name and surname of the authorised representative, if appointed,

- the product type, production lot number, serial number, or other element or information making it possible to identify the product,

- the designation of the type and name of the defined standard used or SK technical assessment,

- information on the product’s intended use in construction in accordance with the defined standard or SK technical assessment,

- the system(s) used to assess the product's parameters,

- information on the authorised person and on the SK certificates which that authorised person issued, or on protocols on tests or calculations conducted in accordance with the system used to assess the product’s parameters,

- information on the place of the technical assessment and designation of the SK technical assessment, if an SK technical assessment was used to assess the parameters,

- the parameters of the product’s essential characteristics declared by the manufacturer expressed as a level, class or description,

- the first name, surname and position of the person authorised to sign the SK parameter declaration on behalf of the manufacturer, that person’s signature and the manufacturer’s stamp.

- If the product in question contains a chemical substance which is hazardous to health or to the environment, then the manufacturer must include a safety information card with the SK parameter declaration.)

- The manufacturer shall provide the distributor or consumer with the written SK parameter declaration when the product is first delivered.

- The product’s instructions for use, the safety instructions and any information on the health and safety risks which the product poses in ordinary use must be in the national language and must be enclosed with the product when it is made available on the domestic market.

Section 7

Certification

- Certification is the procedure for assessing the parameters and verifying the conformity of the production management system. Certification is conducted by an authorised person in accordance with either the defined standard or the SK technical assessment. The procedure leads to the issue of the SK certificate.

- The following are SK certificates in accordance with this Act:

- certificate of consistency of the essential product characteristic parameters (hereinafter referred to as the ‘SK product certificate’), and

- certificate of the conformity of the manufacturer’s production management system (hereinafter referred to as the ‘SK management system certificate’).

Section 8

Product type and tests

- The tests for parameter assessment purposes are the product type test, planned test and control test. The results of the tests shall be documented in protocols on tests and on calculations.

- The product) type shall be determined before the product is placed on the domestic market by means of a type test on a product sample taken by the manufacturer, or shall be determined based on a type calculation, tabular values or descriptive documentation on the product under either the defined standard or the SK technical assessment. The authorised person or manufacturer shall conduct the product type determination for individual product groups to the extent indicated in the implementing legislation.

- The essential product characteristic parameters of the manufactured product shall be compared with the parameters of the product type, which have been ascertained in accordance with Paragraph 2, by means of a control test conducted at intervals under either the defined standard or the SK technical assessment. The authorised person shall conduct control tests (or ensure they are conducted) on samples of products which are being prepared to be made available on the domestic market.

- The essential product characteristic parameters shall be compared to the parameters ascertained using the type test by means of a test planned by the manufacturer. The manufacturer shall conduct the planned test (or ensure it is conducted) on samples of products which are being prepared to be made available on the domestic market.

- At the manufacturer’s request, or if justified by technical, economic or logistical reasons, the authorised person may conduct product tests at the manufacturer’s test laboratory or, with the manufacturer’s consent, at the test laboratory of a third person and using that laboratory’s testing equipment. Beforehand, the authorised person must verify whether the test method requirements are met and must assess whether the testing equipment is validly calibrated and whether the quality of the test results can be ensured.

Section 9

Collection of samples for the test

- Product samples needed to conduct the test shall be taken using the methods and procedures according to either the defined standard or the SK technical assessment.

- The product samples taken must represent the product prepared to be made available on the domestic market and must be marked so they cannot by mistaken for similar products from different places of manufacture or from different manufacturers. A sampling protocol shall be compiled on the process for taking the product samples.

- The marking of the sample taken must contain at least the manufacturer’s commercial name, a designation of the place of manufacture and the date and time when the sample was taken. If the marking of the sample taken is encoded, then it must be possible to ascertain this information from the sampling protocol.

- If the samples were taken by the authorised person, then the sampling protocol must contain at least:

- information on the manufacturer to the extent indicated in Section 6 Paragraph 2 Letter a) and information on the place of manufacture,

- information on the place at which the samples were taken in production,

- information on the quantity of identical products in storage or in the production lot from which the sample was taken, if that information is important for conducting the test,

- information on the number of samples taken,

- identification of the product under either the defined standard or the SK technical assessment,

- the product marking made by its manufacturer,

- the marking of the sample made by the authorised person,

- a marking of the essential product characteristics whose parameters constitute the subject of the test,

- the date on which the sample was taken,

- the names and surnames of the employees of both the authorised person and the manufacturer present during the sampling process as well as their signatures,

- the authorised person’s registration number.

Section 10

Test protocol

- A test protocol shall be compiled on every product test. The test protocol must contain at least the following:

- information on the manufacturer to the extent indicated in Section 6 Paragraph 2 Letter a) and information on the place of manufacture,

- information on the product tested and on the defined standard or SK technical assessment,

- information on the method, place and time of taking the product sample and information on the person who took the sample,

- the designation of the test laboratory and information on the people who tested the product,

- description of the method or procedure according to which the product test was conducted,

- date and designation of the place where the product test was performed,

- the result of the product test and an analysis of it, if the result of the test would not be sufficient,

- information on the date and place of the protocol’s delivery to the person who requested the product test to be conducted,

- the registration number of the authorised person who conducted the product test,

- signature of the head of the test laboratory.

- If information from previous type tests is to be admitted, then the test protocol will contain all the necessary information pertaining to the identity of the samples tested and the testing methods.

Section 11

Inspection

- Inspections for certification purposes include both an initial inspection of the place of manufacture and the production management system (hereinafter referred to as the ‘initial inspection’) and continuous monitoring of the production management system (hereinafter referred to as ‘continuous monitoring’). The initial inspection and continuous monitoring shall be conducted by the authorised person.

- The purpose of the initial inspection is to ascertain whether a production management system has been introduced at all of the manufacturer’s places of manufacture and whether the organisational, human resources and technical prerequisites for maintaining the quality of the product in the long term have been created in the manufacturer’s organisation structure. During the initial inspection, the product and conditions of conducting internal control in relation with the production of the specific product, all variants thereof and product groups which are manufactured using the same technology shall be assessed prior to the start of production or after a major change in production technology.

- The purpose of continuous monitoring is to monitor during the course of production whether the production management system implemented is in accordance with the requirements of the defined standard or SK technical assessment and with this Act and whether the manufacturer has fulfilled the conditions imposed by the authorised person during the initial inspection or during previous continuous monitoring.

- Planned continuous monitoring shall be conducted, on the basis of a written report, at dates after the initial inspection according to a plan compiled by the authorised person based on either the defined standard or on the SK technical assessment.

- Unplanned continuous monitoring shall be conducted

- if the manufacturer’s notifies a change in production technology or a change in the internal production control organisation,

- in order to check the rectification of shortcomings detected by a previous continuous monitoring or inspection conducted by the Slovak Trade Inspectorate (hereinafter referred to as ‘state inspection authority’)

- when resuming production after an interruption lasting longer than 12 months.

- The time between continuous monitoring inspections under Paragraphs 4 and 5 may not be longer than 12 months.

- The manufacturer is obliged to notify the authorised person without delay of any changes to production technology, raw materials, semi-finished goods and components used which affect, or may affect, the product’s characteristics, as well as any organisational and human resource changes which affect, or may affect, internal control.

Section 12

Production management system

- The production management system is the manufacturer’s constant internal control which ensures that the production process and the manufactured product conform to the requirements of this Act regarding the consistency of the essential product characteristic parameters in accordance with the defined standard or SK technical assessment.

- The production management system encompasses the

- issue of operational regulations governing control procedures,

- implementation of control procedures in accordance with the operational regulations,

- recording of controls conducted and their results,

- use of the results of controls and tests conducted to rectify detected discrepancies in essential product characteristic parameters from the requirements under the defined standard or the SK technical assessment and for modifying control procedures with the goal of eliminating the cause of those discrepancies.

- The control procedures shall encompass all or only some of the following activities, depending on the type and composition of the product, its production technology and the sensitivity of its essential product characteristic parameters to changes in production:

- description and control of the input raw materials, semi-finished goods, components or parts or ingredients thereof,

- inspection of the production equipment used in individual intermediate stages of production, in particular machine settings and furnishings.

- The manufacturer must organise the implementation of the production management system in such a manner that only products which have passed through all control procedures prescribed in the operational regulations leave production. If a product has left production and is shipped before the manufacturer has ascertained all results of the control procedures, he shall inform his consumer of the situation without delay.

- The manufacturer is obliged to compile reports on the results of the control procedures conducted and monitor whether the product passed through all control procedures under the operational regulations. The reports must clearly indicate that the product possesses all of the essential product characteristic parameters under the defined standard or the SK technical assessment. Statistical methods shall be used preferentially when evaluating the results of the control procedures.

- If control procedures in any phase or intermediate stage of production detect that a manufactured product does not possess the essential product characteristic parameters under the defined standard or the SK technical assessment, then the manufacturer shall be obliged to take all efforts to eliminate the causes of the detected discrepancy without delay. Once the discrepancy has been eliminated, the manufacturer shall repeat the control procedures prescribed by the operational regulations. The manufacturer shall immediately separate products which do not comply with the essential product characteristic parameters under the defined standard or the SK technical assessment from deliveries intended to be made available on the domestic market, and shall mark them suitably and store them separately from other products until a decision on their use has been reached.

- The manufacturer is obliged to appoint an employee to ensure the procedures of assessing the essential product characteristic parameters, to detect and record all cases of discrepancies from the defined standard and SK technical assessment and to establish measures for eliminating any discrepancies detected for each place of manufacture at which a product, part or ingredient thereof is manufactured and for each stage of production. The appointed employee must have the necessary knowledge of the production management system and of production technology.

- The manufacturer is obliged to ensure that only equipment which is operational, verified and calibrated is used for the control procedures.) The manufacturer must have control, testing and measurement equipment and other required furnishings for the control procedures. If the manufacturer does not have such equipment or furnishings, he shall be obliged to have the control procedures conducted by another person who does have the necessary equipment and furnishings and employees capable of using them.

- The manufacturer is obliged to keep documentation of the production management system recording the product description, the date of production, the control procedure used, the results ascertained and the criteria for conformity with the essential product characteristic parameters.

- The manufacturer is obliged store the complete documentation on the product and its individual lots, including production details and information on the essential product characteristic parameters, as well as records on the primary consumers to whom the product or lot was shipped over the entire time of production and for 10 years after production finishes. The product and its lots must be clearly identifiable and details on their production must be traceable.

Section 13

Responsibilities of the Ministry of Transport, Construction and Regional Development of the Slovak Republic

- The Ministry of Transport, Construction and Regional Development of the Slovak Republic (hereinafter referred to as the ‘Ministry’)

- cooperates with other central state administrative authorities in matters concerning harmonised technical specifications, European technical assessments, defined standards and SK technical assessments and in the process of ensuring the uniform enforcement of statutory regulations on products,

- cooperates with the Office of Standards, Metrology and Testing of the Slovak Republic in proposing and creating Slovak technical standards relating to products and when ensuring the repeal of defined standards at the end of the period of their simultaneous validity with the harmonised standards,

- determines which Slovak technical standards are defined standards, and repeals their determination as defined standards,

- ensures the Slovak Republic’s international cooperation with the Standing Committee on Construction and appoints the Slovak Republic’s representatives in European sector groups for construction, the organisation of technical assessment bodies and the group of notified persons,

- fulfils the tasks of the contact point for products pursuant to separate legislation,)

- is responsible for

1.	authorising legal entities and for changing and withdrawing authorisations,

2.	appointing, monitoring, evaluating and terminating the technical assessment body pursuant to separate legislation) and for informing the Commission and Member States of procedures which it implements,

- is the notifying authority) and informs the Commission and Member States of the procedure of notifying the authorised person) and notifies the Commission and Member States of the notified person,)

- is entitled to submit formal objections to harmonised standards and European evaluating documents,)

- pays authorised persons an annual remuneration for technical evaluations, for the work of the organisation of technical assessment bodies and remuneration for the Slovak representative's participation in said organisation,

- publishes the following on its website:

- announcements on the award, change and withdrawal of authorisations,

- announcements on the issue and cancellation of European evaluation documents and European technical assessments in the original language and in translation into the national language, as well as SK technical assessments in the national language,

- announcements of the period of simultaneous validity of defined standards and harmonised standards,

- a list of defined standards, which it shall also update,

- methodological guidelines concerning the activities of authorised persons and manufacturers,

- reports on measures issued in accordance with Paragraph 3,

- other information and data concerning technical specifications and activities of authorised persons,

- issues, at the manufacturer’s request, an opinion on the product’s classification in the parameter assessment system and on whether the product in question is a product, if there is any doubt on this matter in the activities of authorised persons or during monitoring of the domestic market,

- provides, at the authorised person’s request, an expert opinion on how to apply statutory regulations, harmonised standards, European technical assessments, defined standards and SK technical assessments.

- The Ministry issues generally applicable legislation establishing a list of the product groups which the parameter assessment, parameter assessment systems and template of the SK parameter assessment apply to.

- The Ministry issues measures which are used to define the essential characteristics for products made available on the domestic market whose parameters must be declared by the manufacturer in the SK parameter declaration in accordance with the individual harmonised standards, European technical assessments or defined standards.

Authorisation

Section 14

- The Ministry may only authorise legal entities with a registered office in the Slovak Republic which:

a)	employ the necessary number of employees with expert knowledge and sufficient experience to conduct parameter assessments,

b)	have compiled a description of the procedures according to which the parameter assessment is conducted and have ensured the transparency and reproducibility of those procedures,

c)	have introduced procedures which distinguish between the tasks of the authorised person and other activities,

d)	have the material means and human resources necessary for the expert and administrative tasks associated with the activities for which they are to be authorised, and have access to the necessary technical furnishings and measurement equipment,

e)	implement a management system ensuring the impartiality of the top management and the employees who conduct the parameter assessment,

f)	have introduced a system of rewarding the top management and the employees who conduct the tests and production control which is neither based on the number of tasks, nor on the results of them.

- The chairman or members of the executive board of a legal entity under Paragraph 1 must be of good repute. People who have been convicted for crimes of corruption, laundering the proceeds of crime, harming the financial interests of the European Community, unauthorised business activities or fraud are not deemed to be of good repute for the purposes of this Act. Good repute shall be demonstrated by means of an extract from the criminal record less than three months old.

- There is no legal entitlement to authorisation. The Ministry shall authorise legal entities in such number and composition in order for there to be sufficient capacity for technical assessments and parameter assessments.

- During authorisation, the Ministry may acknowledge decisions on accreditation) or other findings which attest that the requirements for authorisation under Paragraph 1 have been met.

- The Ministry may authorise a legal entity:

a)	for product certification,

b)	for production management certification,

c)	as a test laboratory,

d)	for technical assessment.

- The legal entity may be authorised for all activities pursuant to Paragraph 5, or only for some of them. A manufacturer may only be authorised as a test laboratory.

- An authorised person for product certification shall conduct the product type determination, initial inspection, continuous monitoring and control tests in accordance with the relevant parameter assessment system and issue SK product certificates.

- An authorised person for production management certification shall conduct the initial inspection and continuous monitoring in accordance with the relevant parameter assessment system and issue the SK management system certificate.

- An authorised test laboratory conducts the product type determination.

- An authorised person for technical assessment conducts technical assessments and issues SK technical assessments.

- Employees of an applicant for authorisation who will be participating in the activities for which the applicant is to be authorised must:

- have sufficient knowledge of the activities which the authorised person will conduct, including applicable defined standards and the creation of certificates, lists and protocols under this Act.

- maintain the confidentiality of the data and information which they gain during their activities; this does not apply in relation to notified persons and technical assessment bodies of other Member States.

- The authorised person may appoint another person to perform a certain task in his name within the scope of his authorisation and with the manufacturer’s consent. The test protocol and inspection report shall indicate which tasks were performed by an appointed person.

- The authorised person is obliged to notify the Ministry without delay of changes to the technical basis and to the requirements pursuant to Paragraph 1 which are significant for assessing the capacity to conduct the activity as an authorised person. Based on the notification, the Ministry shall decide on changing the decision on authorisation, if the change did not cause the authorised person to be organisationally or technically incapable.

- The authorised person is obliged to take out an insurance policy for liability for damages caused by the activities for which he is authorised as of the date of commencing activities, and is obliged to remain insured during the entire time those activities are performed.

Section 15

- The Ministry shall authorise a legal entity on the basis of an application. The application for authorisation shall contain:

- information on the applicant, including the commercial name, company identification number, registered office and contact address,

- information on the chairman or members of the applicant’s executive board and responsible representative, if appointed, including their first name, surname, titles, highest level of education achieved and previous experience in the field of product certification, production management certification, technical certification or technical assessment,

- information on the applicant’s technical basis and equipment,

- the required extent of authorisation in accordance with Section 14 Paragraph 5,

- the designation of the product group for which the authorisation is being applied,

- information and documents demonstrating fulfilment of the requirements under Section 14 Paragraph 1 and under Letters a) through c).

- If the applicant is in full compliance, the Ministry will only mark the decision on authorisation in the file and will only issue the applicant the certification of authorisation.

- The Ministry shall commence proceedings to withdraw the authorisation at its own initiative if it detects that any of the following have occurred, in spite of a warning from the state monitoring authority and the imposition of a fine:

- the authorised person has not been performing the activity for which it was authorised for more than 12 months,

- the authorised person is violating the procedures of parameter assessment or using unverified or uncalibrated measurement equipment,

- the authorised person has lost the ability to impartially and objectively assess parameters and their suitability for use for the intended purpose in construction due to a change in executive management or a change in the employees’ qualification structure,

- the authorised person’s technical basis and measurement equipment has become unusable.

- The deadline for the decision under Paragraphs 1 and 3 is 90 days from the date on which the proceedings commenced.

Section 16

Notification

- A notified person is an authorised person about whom the Ministry has informed the Commission and other Member States.

- The Ministry shall notify an authorised person who meets the requirements specified in Paragraphs 3 through 5 and in separate legislation.) These requirements must be met over the entire time the person is a notified person. If the notified person has branches or subcontractors, he must ensure that they also meet the requirements specified in Paragraphs 3 through 5 and in separate legislation.24)

- The Ministry may only notify an authorised person who is a member of a trade association or professional union of associated entrepreneurs engaged in the design, production, supply, assembly, use or maintenance of products whose parameters he is authorised to assess if it is demonstrated in the notification procedure that the authorised person in question does not depend on bodies of the trade association or professional union when assessing the parameters, that the authorised person in question is not a manufacturer, importer, distributor or authorised representative, nor connected personally or financially to a manufacturer, importer, distributor or authorised representative who is placing the product on the domestic market or the European Union internal market, or making it available on said markets, and that the authorised person in question is not in a conflict of interests, in particular by providing consulting services to a trade association, professional union, manufacturer, importer, distributor or authorised representative.

- The notified person, his top management and employees conducting the parameter assessment may not be project engineers, manufacturers, suppliers, fitters, purchasers, owners, users or maintenance personnel of the products whose parameters they are authorised to assess, nor may they be authorised representatives of any of the above; this does not apply in cases concerning the product's use in the scope of the authorised person’s activity, or the product’s use for the personal purposes of the top management and other employees.

- The notified person, his top management and employees performing tasks in the parameter assessment process may not be directly engaged in the design, production, construction execution, sale, assembly, use and maintenance of products whose parameters they are authorised to assess, nor may they be authorised to represent people engaged in these activities or provide those people with consultation services.

- The Ministry shall conduct notification upon application from the authorised person. The application for notification shall contain:

- information on the applicant, including the commercial name, company identification number, registered office and contact address,

- information on the chairman or members of the applicant’s executive board and responsible representative, if appointed, including their first names, surnames, titles, highest level of education achieved and previous experience in certification or testing,

- information on the applicant’s technical basis and equipment,

- information and documents demonstrating fulfilment of the requirements under Sections 3 through 5 and under Letters a) through c), and

- information pursuant to separate legislation.)

- The Ministry shall inform the Commission of the procedure for assessing the applicant for notification and of the notification of the authorised person and of the procedure of monitoring notified persons’ activities in the parameter assessment process and of changes to those procedures caused by electronic tools developed and administered by the Commission.

Section 17

Appointment of the technical assessment body

- A technical assessment body is an authorised person for technical assessment whom the Ministry has notified the Commission and other Member States of for one or more production fields in accordance with separate legislation.)

- The Ministry may appoint an authorised person for technical assessment who meets the requirements specified in separate legislation) as a technical assessment body. These requirements must be met over the entire time the person is a technical assessment body.

- The Ministry appoints a technical assessment body on the basis of an application from an authorised person for technical assessment. The application for appointment as a technical assessment body shall contain:

- information on the applicant, including the commercial name, company identification number, registered office and contact address,

- information on the chairman or members of the applicant’s executive board and responsible representative, if appointed, including their first names, surnames, titles, highest level of education achieved and previous experience in technical assessment or technical certification,

- information on the applicant’s technical basis and equipment,

- information and documents demonstrating fulfilment of the requirements under Letters a) through c), and

- information on the fulfilment of the requirements in accordance with separate legislation.27)

Section 18

State monitoring

- The Ministry shall conduct state monitoring of the activity of technical assessment bodies, notified persons and authorised persons. The Ministry

- monitors and evaluates the activities of technical assessment bodies, notified persons and authorised persons and executes measures to eliminate shortcomings detected in their activities,

- takes care to ensure that new legislation and amendments thereof do not restrict or impair the access of products with a CE mark on the domestic market and to ensure that parameter assessment systems for products whose parameters are based on harmonised technical specifications and European technical assessments are in accordance with that new legislation and amendments thereof,

- provides, at the Commission’s request, information in connection with the documents for the appointment of a technical assessment body and on the maintenance of a technical assessment body’s capacity, as well as information in connection with the documents for the notification and maintenance of capacity of a notified person,

- imposes fines for administrative offences under Section 27 Paragraphs 1 through 3.

- Employees charged with conducting state monitoring under Paragraph 1 are:

- authorised to enter the workplace of a technical assessment body, notified person and authorised person,

- authorised to inspect the documentation of a technical assessment body, notified person and authorised person,

- authorised to request employees of a technical assessment body, notified person and authorised person for explanations concerning their activities,

- obliged to maintain the confidentiality of facts which they gain knowledge of while conducting state monitoring, even after the end of their employment relationship; they may be released from this obligation by the person to whom the fact in question relates.

Section 19

Monitoring of the domestic market

- Monitoring of the domestic market and control of products placed and made available on the domestic market are conducted by the state inspection authority.

- The state inspection authority:

- conducts control and imposes measures and fines in accordance with separate legislation) and in accordance with this Act,

- takes measures concerning products which present a risk to health and safety,)

- takes measures in the detection of formal discrepancies and informs the Commission and Member States of the results of evaluations and adopted measures in accordance with separate legislation,)

- may propose an authorised person to cancel an SK certificate or SK technical assessment.

- If the state inspection authority detects that a product placed on the domestic market does not provide the sufficient level of safety required by the technical specifications and by this Act, either by means of a control or on the basis of evidence obtained from another Member State, it may refuse its provision on the domestic market and order it to be recalled or withdrawn from the domestic market. It may do this if:

- it has provided the manufacturer, importer, distributor or authorised representative with written information on the provisions of this Act which prevent the product from being made available on the domestic market and requested him to make a statement within a specified deadline and provide proof that the product has a sufficient level of safety,

- demonstrated that the product’s level of safety is insufficient based on the results of a control test on a sample taken in distribution and based on findings obtained on the product's essential characteristics, and

- taken the statement from the manufacturer, importer, distributor or authorised representative pursuant to Letter a) into account before making its final decision.

Proceedings

Section 20

Basic provisions

- The general legislation on administrative proceedings) shall apply to proceedings under this Act, unless this Act or the separate legislation) specifies otherwise.

- The general legislation on administrative proceedings does not apply to the:

- issue and cancellation of SK certificates,

- issue and cancellation of SK technical assessments,

- notification of authorised persons and cancellation of notification,

- appointment of authorised persons as technical assessment bodies and the cancellation of that appointment,

- determination of which Slovak technical standards are defined standards and the cancellation of their determination as defined standards,

- filing of objections against the technical assessment body, notified person and authorised person,

- review of the filing of objections by the Ministry.

Section 21

Proceedings for issuing an SK certificate

- The proceedings for issuing an SK certificate commence on the date upon which the authorised person for the certification of the relevant product or product group receives the written application from the manufacturer, his authorised representative or importer.

- The application for the issue of an SK certificate shall contain:

- information on the applicant, including the commercial name, company identification number and contact address,

- information on the manufacturer, including the commercial name, company identification number and contact address, if the applicant is not the manufacturer,

- the place where the product is manufactured and the production management system implemented,

- information on the product, in particular:

- its type designation and commercial name,

- the designation of all of its types and variants manufactured using the same technology,

- the purpose of its use in construction and any potential restrictions in its use,

- an indication of the defined standard or SK technical assessment which applies to the product,

- information on whether the application is for product certification or certification of a production management system.

- If the application does not contain the information under Paragraph 2 or has another shortcoming which prevents the proceedings from continuing, the authorised person may send the applicant a written request to complete the application or rectify any other shortcomings within it by a specified deadline. If the applicant fails to accomplish this within the specified deadline, the authorised person will not continue in the proceedings and will return the application to the applicant.

- The authorised person shall send the applicant a draft of a contract on the conditions of certification within ten business days of the date on which the application is complete. The provisions of the contract on the control activity) shall apply accordingly to the content and form of the contract. If the applicant rejects the draft of the contract, or if the contract is not concluded for other reasons, the authorised person will not continue in the proceedings and will return the application to the applicant.

- The authorised person will not continue in the proceedings before the contract is concluded and will return the application to the applicant if:

- the applicant withdraws the application,

- the authorised person finds out that:

- he is not competent for certification within the scope of the submitted application,

- his authorisation was withdrawn after the application was submitted,

- there is no defined standard or SK technical assessment according to which the certification can be conducted,

- the manufacturer ceased to exist without a legal successor after the proceedings had commenced,

- the product specified in the application ceases to be manufactured, or its production was prohibited or suspended, or

- certification for the same matter is underway with another authorised person.

- The applicant may only withdraw the application before the contract has been concluded.

- The applicant may withdraw from the contract if the authorised person fails to fulfil the contract or is inactive for at least two months.

- The authorised person may withdraw from the contract if the manufacturer fails to fulfil the contract, is inactive for at least two months and it is impossible to continue fulfilling the contract without his cooperation, or if a hindrance to the fulfilment of the contract under Paragraph 5 appears.

- If the parameter assessment systems demonstrate that the essential product characteristic parameters or production management system conform with the parameters under the defined standard or SK technical assessment, then the authorised person shall issue the SK certificate.

- The authorised person for product certification issues the SK product certificate on the basis of tests and inspections in the parameter assessment system which applies to the product group in accordance with the implementing legislation.

- The authorised person for production management system certification issues the SK management system certificate on the basis of the initial inspection and continuous monitoring within the scope pursuant to the implementing legislation.

- If the parameter assessment systems do not demonstrate that the essential product characteristic parameters or production management systems conform with the parameters pursuant to the defined standard or SK technical assessment, then the authorised person will not issue the SK certificate and will inform the applicant of this, stating the reason for which he did not issue the SK certificate.

- The SK product certificate contains:

- information on the authorised person, including the commercial name, registered office, company identification number and authorisation number,

- designation of the manufacturer, including the commercial name, company identification number and place of manufacture of the product to which the SK certificate relates,

- information on the product, including the type designation, commercial name and the designation of types and variants manufactured using the same technology,

- information on the purpose of use in construction in accordance with the defined standard or SK technical assessment, and information on any potential restrictions in its use,

- designation of the defined standard or SK technical assessment based on which the SK certificate was issued, and information on the essential product characteristic parameters which were assessed and whose consistency was verified,

- name and number of the SK certificate and date of its issue,

- information on the conditions of the SK certificate’s validity,

- first name, surname and position of the person authorised to sign the SK certificate, his signature and the stamp of the authorised person.

- The SK management system certificate contains:

- information on the authorised person, including the commercial name, registered office, company identification number and authorisation number,

- designation of the manufacturer, including the commercial name, company identification number and place of manufacture of the product to which the SK certificate relates,

- information on the product, including the type designation and commercial name,

- information on the purpose of use in construction in accordance with the defined standard or SK technical assessment, and information on any potential restrictions in its use,

- designation of the defined standard or SK technical assessment based on which the SK certificate was issued,

- the name and number of the SK certificate and date of its issue,

- information on the conditions of the SK certificate’s validity,

- first name, surname and position of the person authorised to sign the SK certificate, his signature and the stamp of the authorised person.

- The SK certificate is valid from the date which is specified on it as the date of validity. The SK certificate is issued with an indefinite period of validity. If the applicant requests, the authorised person shall restrict the validity of the SK certificate to the period time which the applicant specified in the application or asserted during the proceedings. The authorised person may also restrict validity of the SK certificate if:

- the manufacturer is preparing a change in production technology or a change in the production management system, in which case the SK certificate shall be valid until the change is made,

- the information on the product or production management system is valid for a limited period of time, in which case the SK certificate shall be valid for the time for which the information is valid,

- the rectification of an insubstantial deviation detected from the defined standard or SK technical assessment requires a systematic change in the production technology or change in the production management system, in which case the SK certificate shall be valid until it is implemented, albeit not longer than five years.

- The SK certificate shall be delivered to the applicant in person, and the authorised person shall publish an announcement of its issue on its website.

- The SK certificate may only be copied in whole. It may only be copied in part with the consent of the authorised person who issued it.

- The manufacturer may issue and publish an SK parameter declaration no earlier than the day after the SK certificate becomes valid.

- If the information which the SK certificate contains changes during the SK certificate’s validity, but the change does not constitute a reason to cancel the certificate, then the authorised person shall issue a new SK certificate containing the new information upon application, and the manufacturer shall issue a new SK parameter declaration based on the new certificate.

Section 22

Proceedings for cancelling an SK certificate

- The authorised person shall initiate proceedings to cancel an SK certificate if it detects that:

- the manufacturer failed to rectify a discrepancy with a product or production management system within the specified deadline,

- the essential product characteristic parameters according to the result of the control tests do not conform with the parameters in the SK parameter declaration or with the results of the product type test,

- the production management system fails to ensure the required level of product control on a long-term basis,

- the manufacturer has made a change to the production technology or to the raw materials, semi-finished goods or components used which has such a great effect on the essential product characteristic parameters that new certification is necessary,

- the manufacturer prevented the authorised person or subcontractor thereof from conducting continuous monitoring or control tests.

- The proceedings for cancelling an SK certificate shall commence when the person who was the applicant in the proceedings for issuing the SK certificate receives notification that the proceedings will commence.

- If any of the reasons under Paragraph 1 are demonstrated in the proceedings for cancelling the SK certificate, then the authorised person shall cancel the SK certificate upon receiving a written statement from the manufacturer on the reason pursuant to Paragraph 1. If the manufacturer requests the SK certificate to be cancelled, then the authorised person shall cancel the SK certificate immediately without further investigation.

- Notification that the SK certificate has been cancelled shall be delivered to the person who was the applicant for the issue of the SK certificate as well as to the state inspection authority, and shall be published on the authorised person’s website. If the SK certificate was issued on the basis of an SK technical assessment, then notification that the SK certificate has been cancelled shall also be delivered to the authorised person for technical assessment.

- The SK parameter declaration which was issued on the basis of the SK certificate shall cease to be valid on the date upon which the notification that the SK certificate has been cancelled is received by the applicant.

Section 23

Proceedings for issuing an SK technical assessment

- Proceedings for issuing an SK technical assessment shall commence on the date upon which the competent authorised person for technical assessment receives the written application from the manufacturer, their authorised representative or the importer.

- The application for the issue of an SK technical assessment shall contain:

- information on the manufacturer, including the commercial name, company identification number, registered office and contact address,

- information on the product, in particular its designation, classification in a product group, detailed technical description and technical drawings,

- the results of the tests conducted, calculations and other tasks which are significant for issuing the SK technical assessment,

- information on the scope of the product’s suitability for use in construction,

- information on the manufacturing of the product and places at which the product is manufactured,

- information on the production management system, in particular on its organisation, human resources and technical equipment,

- an indication of any technical regulations or technical standards which are not defined standards, but whose contents should be used as a basis for the technical assessment.

- If the application does not contain the information pursuant to Paragraph 2, then the authorised person shall send the applicant a written request to complete the application by a specified deadline. If the applicant fails to accomplish this within the specified deadline, the authorised person will discontinue the proceedings and return the application to the applicant.

- The authorised person shall send the applicant a draft of a contract on the conditions of the technical assessment within 30 business days of the date when the application is completed. The provisions of the contract on the control activity33) shall apply accordingly to the content and form of the contract. If the applicant rejects the draft of the contract, or if the contract is not concluded for other reasons, the authorised person will discontinue the proceedings and return the application to the applicant.

- The manufacturer is obliged to grant employees of the authorised person access to all premises where the product is manufactured or stored. Failure to fulfil this obligation is grounds for withdrawal from the contract. The authorised person may also withdraw from the contract if the applicant has failed to pay the contractually agreed sum for the technical assessment tasks or advance payment on said sum by the specified deadline, even after a repeated request.

- Before issuing the SK technical assessment, the authorised person shall verify or ensure the verification of the parameters of the relevant essential product characteristics in relation to the declared use and shall assess or ensure the assessment of the production management system at the places at which the product is manufactured, and shall compare the results ascertained with the regulation or standard pursuant to Paragraph 2 Letter g). If the applicant did not specify any regulation or standard pursuant to Paragraph 2 Letter g) in the application, or if the specified regulation or standard does not indicate the essential product characteristic parameters, the authorised person shall propose them to the applicant and, if the applicant consents to them, the authorised person shall verify or ensure the verification of the essential product characteristic parameters agreed upon, and shall evaluate or ensure the evaluation of the production management system at the places where the product is manufactured.

- If the tasks pursuant to Paragraph 6 demonstrate a positive evaluation of the essential product characteristic parameters in relation to the manufacturer’s declared use in construction, then the authorised person shall issue the SK technical assessment. If the authorised person does not issue the SK technical assessment, he shall notify the applicant of this and state the reasons why it was not issued.

- A new SK technical assessment can only be issued once a new technical assessment has been conducted on the basis of a new contract. In the case of a change to an SK technical assessment, the application may only be submitted to the authorised person who issued the SK technical assessment which is to be changed.

- The SK technical assessment shall contain:

- information on the authorised person, including the commercial name, registered office, company identification number and authorisation number,

- designation of the manufacturer, including the commercial name, company identification number and place of manufacture of the product to which the SK technical assessment relates,

- information on the product, including the commercial name, type designation and the designation of types and variants manufactured using the same technology,

- information on the product’s purpose and method of use in construction in accordance with the SK technical assessment, and information on any potential restrictions in its use in construction,

- information on the conditions of the SK technical assessment’s validity,

- the name and number of the SK technical assessment and its date of issue,

- first name, surname and position of the person authorised to sign the SK technical assessment, his signature and the stamp of the authorised person,

- the designation of the prescribed parameter assessment system and a list of the product tests and inspections of the production management system which are to be conducted accordance to said system,

- requirements for accompanying information,

- appendices

- a detailed technical description of the product, generally supplemented by diagrams,

- a description of the relevant essential product characteristic parameters detected,

- a list of the regulations and standards specified in Paragraph 2 Letter g), if applied,

- a list of further documents used when issuing the SK technical assessment.

- The SK technical assessment is valid from the date specified on it for an indefinite period of time.

- The SK technical assessment shall be delivered to the applicant in person, and the authorised person shall publish and announcement of its issue on its website.

- The SK technical assessment may only be copied in its entirety. It may only be copied in part with the consent of the authorised person who issued it.

Section 24

Proceedings for cancelling an SK technical assessment

- The authorised person shall initiate proceedings to cancel an SK technical assessment if it detects that:

- a European technical assessment has already been issued for the same product from the same manufacturer,

- a harmonised standard applies to the same product, or

- the generally applicable legislation stipulates that the product, component or part thereof may not be made available on the domestic market or used in a construction, or that its use is not in accordance with the requirements for personal safety during construction or maintenance work.

- The proceedings for cancelling an SK technical assessment shall commence on the date upon which the person who was the applicant in the proceedings for issuing the SK technical assessment receives notification that the proceedings will commence.

- If any of the reasons pursuant to pursuant to Paragraph 1 are demonstrated in the proceedings for cancelling the SK technical assessment, then the authorised person shall cancel the SK technical assessment upon receiving a written statement from the applicant on the reason pursuant to Paragraph 1. If the manufacturer requests the SK technical assessment be cancelled, then the authorised person shall cancel the SK technical assessment immediately without further investigation.

- Notification that the SK technical assessment has been cancelled shall be delivered to the person who was the applicant for the issue of the SK technical assessment as well as to the Ministry, and shall be published on the authorised person’s website.

- The SK certificates and SK parameter declaration which were issued on the basis of the SK technical assessment shall cease to be valid on the date upon which the notification that the SK technical assessment has been cancelled is received by the manufacturer.

Section 25

Proceedings on objections

- The applicant has the right to file objections to the procedure and to individual actions of the technical assessment body or authorised person in proceedings pursuant to this Act. Objections shall be submitted to the person whose procedure or action is being contested within 10 business days of the date upon which the procedure or action was conducted, unless a longer term was agreed upon in the contract. Objections do not have a suspensive effect.

- The person to whom the objections were submitted is obliged to address the content of the objections and investigate the procedure or action objected to without delay. If the procedure or action concerns a subcontractor or other appointed person, then the authorised person shall request a statement and written report on the inspection, test or measurement conducted from that subcontractor or appointed person.

- If the investigation finds the objections to be justified, then the technical assessment body, notified person or authorised person shall ensure rectification at the expense of the person who caused the shortcomings which were objected to, in particular a repeated control test and correction of the test protocol, repeated measurement or other effective redress.

- If the investigation finds that the objections are not justified, then the notified person or authorised person shall send written notification of the examination’s result to the person who filed the objections within three business days of the end of the examination of the objections’ content; this notification shall specify which control measures were conducted and what their results were.

Section 26

Proceedings for reviewing objections filed

- An applicant who was notified that his objections were unjustified by the technical assessment body, notified person or authorised person may request the Ministry to review the objections filed if he believes that the examination of the objections were not proceeded with in accordance with this Act, or that all necessary control measures were not taken to properly verify the content of the objections. A review of the objections filed may be requested within 15 business days of the date upon which the notification of the objections filed was received.

- The Ministry shall review the completeness and correctness of the objections filed. If it determines that their content was not properly verified or correctly evaluated, it will order the necessary actions to be taken to properly review the procedure or action objected to, otherwise it will deny the request.

Section 27

Administrative offences

- The Ministry shall impose a fine upon the technical assessment body, notified person or authorised person

- from EUR 1,000 to EUR 5,000 if said body or person:

- violates the conditions of appointment, notification or authorisation despite having been warned,

- does not conduct the activities for which it was appointed, notified or authorised for more than 12 months without good cause,

- failed to rectify shortcomings in its activity which were detected during the performance of state monitoring of the technical assessment or parameter assessment by the specified deadline,

- failed to notify the Ministry of a change in information which requires the changing or cancellation of the appointment, notification or authorisation nor requested the appointment, notification or authorisation to be changed, even though it was obliged to do so,

- fails to keep the documentation resulting from the technical assessment and parameter assessment on file for the specified period of time.

- from EUR 2,000 to EUR 10,000 if said body or person:

1.	issued an SK technical assessment or SK certificate outside of the scope of its authorisation or without performing all necessary actions implicit in the specified parameter assessment system,

2.	fails to proceed in accordance with this Act when verifying the content of objections, in particular if it fails to conduct all necessary control measures to properly verify the content of the objections,

3.	has not concluded an insurance policy for liability for damages caused by its own activity.

- The Ministry shall impose a fine from EUR 200 to EUR 2,000 upon anyone who:

- duplicates part of an SK technical assessment or SK certificate without the authorised person’s consent,

- fails, as a manufacturer, to keep documentation concerning the SK parameter declaration, including the associated documentary papers and complete documentation in accordance with this Act on file for the specified period of time.

- The Ministry shall impose a fine from EUR 5,000 to EUR 30,000 upon anyone who has

- illegitimately acted as a technical assessment body, notified person or authorised person, or as a contractual subcontractor of any of the above,

- forged or altered a European technical assessment, SK technical assessment, certificate or SK certificate,

- prohibited or prevented access on the domestic market to a product with a CE mark or the use of such a product if the product parameters declared by the manufacturer meet the basic requirements for construction, the general technical requirements pursuant to construction regulations and are suitable for the intended use.)

- The state inspection authority shall impose a fine from EUR 5,000 to EUR 50,000 to anyone who has:

- placed or made available a product without a CE mark on the domestic market in spite of having to mark it in accordance with separate legislation) or used a CE mark in a form other than that stipulated by the separate legislation,35)

- illegitimately made an SK parameter declaration or failed to make an SK parameter declaration, in spite of having been obliged to do so,

- illegitimately used a CE mark or marked a product with information or a marking which could be mistaken for such a mark,

- failed to stop access to a product on the domestic market, failed to pull a product off the domestic market or failed to recall a product which is not suitable for the intended use, in spite of being obliged to do so on the basis of a decision from the state inspection authority,

- made a product available on the domestic market without instructions on use, safety instructions, information on the health and safety risks which the product poses in ordinary use, or declaration on the parameters pursuant to separate legislation9), or in a language other than the national language.

- The fines pursuant to Paragraphs 1 through 4 may be increased by up to two times the maximum limit if the administrative offence caused an accident with consequences on human life or health, or with significant damage to property or parts of the environment.

- The means, duration and consequences of the unlawful situation shall be taken into consideration when determining the sum of the fine.

- Proceedings for imposing a fine may commence within six months of the date upon which the administrative authority gained knowledge of the administrative offence and on the person who committed it, but not later than three years after the date on which the administrative offence took place.

- The proceeds from fines shall be state budgetary revenue.

Section 28

Mutual recognition clause

- The conditions of placing products on the domestic market in accordance with this Act also apply to products manufactured in non-member states; this does not apply to products pursuant to Paragraph 2.

- The conditions of placing products on the domestic market in accordance with this Act do not apply to products which were manufactured or placed on the market in any Member State, the Swiss Confederation) or Turkey, or which were manufactured in accordance with the law of any state of the European Free Trade Association which is party to the Agreement on the European Economic Area. The provisions of Section 2 Paragraph 4 apply to these products as well.

Section 29

Common provisions

- This Act has been adopted in accordance with the legally applicable act of the European Union in the field of technical standards and technical regulations.)

- The obligations which manufacturers have in accordance with this Act apply accordingly to importers with a registered office in a non-member state and authorised representatives thereof with a registered office in the Slovak Republic.

Section 30

Transitional provisions

- Products which were placed on the domestic market in accordance with previous legislation shall be deemed to be products which meet the requirements of this Act for placing products on the domestic market until 30 June 2013.

- Certificates issued before 30 June 2013 shall remain valid until their validity periods expire or until they have been replaced by SK certificates in accordance with this Act.

- Technical certifications issued before 30 June 2013 shall be deemed to be SK technical assessments in accordance with this Act until their validity periods expire.

- The manufacturer may compile an SK parameter declaration on the basis of a certificate or declaration of conformity which was issued before 30 June 2013.

- Authorised persons in accordance with previous legislation are authorised persons in accordance with this Act within the scope of the authorisation granted.

- Certification locations in accordance with previous locations perform the activities of the authorised person for technical assessment within the scope of the authorisation granted; they must request authorisation for technical assessment in accordance with this Act by 30 September 2013, otherwise their authorisation shall expire.

Section 31

Annulling clause

The following shall be rescinded:

- Act No 90/1998 Coll. on construction products, as amended by Act No 264/1999 Coll., Act No 413/2000 Coll., Act No 134/2004 Coll. and Act No 173/2008 Coll.,

- Regulation of the Ministry of Construction and Regional Development of the Slovak Republic No 558/2009 Coll., establishing a list of construction products that must be labelled, systems of conformity attestation, and details on the use of conformity markings, as amended by Regulation No 451/2011 Coll.

Section 32

Effective date

This Act enters into legal effect on 1 July 2013.

) Article 29 of Regulation (EU) of the European Parliament and Council No 305/2011 of 9 March 2011 laying down harmonised conditions for the marketing of construction products and repealing Council Directive 89/106/EEC (OJ EU L 88, 4.4.2011).

) Articles 39 through 42 of Regulation (EU) No 305/2011.

) Article 2 Paragraph 1 of Regulation (EU) No 305/2011.

) Regulation (EU) No 305/2011.

) Article 2 Paragraph 19 of Regulation (EU) No 305/2011.

) Articles 4 through 6 of Regulation (EU) No 305/2011.

)  Article 2 Paragraphs 11 and 13 of Regulation (EU) No 305/2011.

)  Section 5 Paragraph 2 of Act No 264/1999 Coll. on technical requirements on products and conformity assessments and on the amendment and supplementation of certain laws.

) Article 7 Paragraph 4 of Regulation (EU) No 305/2011.

)  Articles 4 through 7 of Regulation (EU) No 305/2011.

) Section 43d of Act No 50/1976 Coll. on zone planning and construction order (the Construction Act) as amended by Act No .../2012 Coll.

) Section 43f of Act No 50/1976 Coll. as amended by Act No 237/2000 Coll.

)  Article 2 Paragraph 27 of Regulation (EU) No 305/2011.

) Article 31 and Annex II of Regulation (EC) No 1907/2006 of the European Parliament and Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93and Commission Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC (OJ EU L 396, 30.12.2006).

) Article 2 Paragraph 9 of Regulation (EU) No 305/2011.

)  Section 19 of Act No 142/2000 Coll. on metrology and on the amendment and supplementation of certain laws, as amended by Act No 431/2004 Coll.

) Articles 10 and 11 of Regulation (EC) of the European Parliament and Council No 764/2008 of 9 July 2008, laying down procedures relating to the application of certain national technical rules to products lawfully marketed in another Member State and repealing Decision No 3052/95/EC (OJ EU, L 218, 13. 8. 2008), Article 10 of Regulation (EU) No 305/2011.

) Articles 29 and 30 of Regulation (EU) No 305/2011.

)  Article 40 of Regulation (EU) No 305/2011.

)  Article 42 of Regulation (EU) No 305/2011.

) Article 39 of Regulation (EU) No 305/2011.

)  Articles 18 and 25 of Regulation (EU) No 305/2011.

) Section 5 of Act No 505/2009 Coll. on the accreditation of conformity assessment bodies and on the amendment and supplementation of certain acts.

)  Article 43 of Regulation (EU) No 305/2011.

)  Article 47 of Regulation (EU) No 305/2011.

)  Annex IV Table 1 of Regulation (EU) No 305/2011.

)  Article 30 and Annex IV Table 2 of Regulation (EU) No 305/2011.

) Sections 6 through 9 of Act No 128/2002 Coll. on state control of the internal market in matters of consumer protection, amending and supplementing other Acts, as amended.

) Section 20 of Act No 250/2007 Coll. on consumer protection, amending Slovak National Council Act No 372/1990 Coll. on minor offences, as amended.

) Articles 56 through 59 of Regulation (EU) No 305/2011.

) Act No. 71/1967 Coll. on administrative proceedings (Administrative Procedure Code), as amended.

) Articles 47 and 48 of Regulation (EU) No 305/2011.

) Sections 591 through 600 of the Commercial Code.

)  Article 8 Paragraph 4 of Regulation (EU) No 305/2011.

)  Chapter IV and Annex II of Regulation (EC) No 765/2008 of 9 July 2008 setting out the requirements for accreditation and market surveillance relating to the marketing of products and repealing Regulation (EEC) No 339/93 (OJ EU L 218, 13.8.2008).

) Agreement between the  European Community and the Swiss Confederation on mutual recognition in relation to conformity assessment (Special edition of the EC Official Journal, Chap. 11, Vol. 41). Decision No 1/2008 of 12 March 2008 of the Committee established under the Agreement between the European Community and the Swiss Confederation on Mutual Recognition in relation to Conformity Assessment on the inclusion in Annex 1 of a new Chapter 16 on construction products (2008/813/ES) (OJ EU L 282, 25.10.2008).

) European Parliament and Council Directive 98/34/EC of 22 June 1998, establishing a procedure for the provision of information in the field of technical standards and regulations and of rules on Information Society services (Special edition of the EU Official Journal, chap. 13/vol. 20), as amended.