[draft]Statutory Rules of Northern Ireland

2023 No.

road traffic and vehicles

# The Motor Vehicles (Construction and Use) (Amendment) Regulations (Northern Ireland) 2023

Made	-	-	-	-	***

Coming into operation	-	***

The Department for Infrastructure(), makes the following Regulations in exercise of the powers conferred by Articles 55(1), (2)(a) and (6) and 110(2) of the Road Traffic (Northern Ireland) Order 1995() and now vested in it().

Citation and commencement

These Regulations may be cited as the Motor Vehicles (Construction and Use) (Amendment) Regulations (Northern Ireland) 2023 and shall come into operation on xx xxxxx 2023.

Amendment of the Construction and Use Regulations

— The Motor Vehicles (Construction and Use) Regulations (Northern Ireland) 1999() are amended in accordance with paragraphs (2) to (5).

In regulation 2(2) (interpretation), in the definition of “ECE Regulation, followed by a number”, for “more than two digits” substitute “a full-stop followed by another number”.

In regulation 30 (tyre loads and speed ratings)—

in paragraph (7), for “but this requirement shall not apply to a retreaded tyre” substitute “or, in the case of a retreaded tyre, with the requirements of ECE Regulation 108 or ECE Regulation 109.”; and

in paragraph (12), in the definition of “load-capacity index”, for “2.29” to the end substitute “2 of ECE Regulation 30.02 or ECE Regulation 54, or as “load index” in paragraph 2 of ECE Regulation 108 or ECE Regulation 109;”.

In regulation 32 (condition and maintenance of tyres)—

in paragraph (1)—

for “Subject to paragraphs (2) to (4)” substitute “Subject to paragraph (1A) and paragraphs (2) to (4)”;

in sub-paragraph (g)(ii) omit “or” at the end;

after sub-paragraph (h) insert—

(i)	the tyre is not a retreaded tyre and—

(i)	the week of manufacture marked on its sidewall in accordance with ECE Regulation 30 or 54 falls more than 10 years before the date on which the motor vehicle is used on the road; or

(ii)	it does not have a week of manufacture marking which complies with ECE Regulation 30 or 54 (whether because it was manufactured before the ECE Regulation applied or otherwise);

(j)	the tyre is a retreaded tyre and—

(i)	the week of retreading marked on its sidewall in accordance with ECE Regulation 108 or 109 falls more than 10 years before the date on which the motor vehicle is used on the road; or

(ii)	it does not have a week of retreading marking which complies with ECE Regulation 108 or 109 (whether because it was retreaded before the ECE Regulations applied or otherwise); or

(k)	a date marked on the tyre sidewall in accordance with ECE Regulation 30. 54, 108 or 109 is illegible.;

after paragraph (1) insert—

(1A) Subject to paragraph (4)(a) and (ca), and notwithstanding anything in regulation 30—

(a)	paragraph (1)(i) and (j) only applies to tyres fitted—

(i)	to a front axle of a bus other than a minibus;

(ii)	in single configuration on any axle of a minibus; or

(iii)	to a front axle of a goods vehicle with a maximum gross weight exceeding 3,500 kg;

(b)	in sub-paragraph (a) —

(i)	“front axle” means any axle, forward of the mid-point of the chassis on which the wheels are controlled by the steering system; and

(ii)	“single configuration” means where no more than one wheel and tyre assembly is mounted at the end of the axle concerned; and

(c)	paragraph (1)(k) only applies to —

(i)	buses (including minibuses); and

(ii)	goods vehicles with a maximum gross weight exceeding 3,500 kg.;

in paragraph (4)—

in sub-paragraph (a), after “(1)(a) to (g)” insert “and (i) to (k)”;

after sub-paragraph (c) insert—

(ca)	Nothing in paragraph (1)(i) to (k) applies to a vehicle of historical interest used for non-commercial purposes, and for this purpose “vehicle of historical interest” means a vehicle which the Department considers to be of historical interest to Northern Ireland and which—

(i)	was manufactured or registered for the first time at least 40 years previously;

(ii)	is of a type no longer in production; and

(iii)	is historically preserved or maintained in its original state and has not undergone substantial changes in the technical characteristics of its main components,

where “type” in relation to the vehicle has the same meaning as “type of vehicle” in Article 3(32) of Regulation (EU) 2018/858 of the European Parliament and of the Council of 30 May 2018 on the approval and market surveillance of motor vehicles and their trailers, and of systems, components and separate technical units intended for such vehicles();; and

in paragraph (6)(a), in the definition of “original tread pattern” for “re-treaded”, in both places that it appears, substitute “retreaded”.

In Schedule 1, in Table 2 (ECE Regulations), after the entry relating to item 44 insert—

| 45 | 108 | 108 | 23.6.98 | Retread tyres for passenger cars and their trailers | – | – |
| --- | --- | --- | --- | --- | --- | --- |
| 46 | 109 | 109 | 23.6.98 | Retread tyres for commercial vehicles and their trailers | – | – |

Sealed with the Official Seal of the Department for Infrastructure on xx xxxxx 2023

Chris Hughes

A senior officer of the Department for Infrastructure

EXPLANATORY NOTE

(This note is not part of the Regulations)

These Regulations amend the Motor Vehicles (Construction and Use) Regulations (Northern Ireland) 1999 in order to make provision regarding the age of tyres (including retreaded tyres), and the legibility of date markings on tyres, which are permitted for use on buses (including minibuses) and goods vehicles with a maximum gross weight exceeding 3,500 kg. The Regulations also amend Table 2 of Schedule 1 to the 1999 Regulations to include provisions relating to retreaded tyres in ECE Regulations 108 and 109. These Regulations will provide for an exemption for vehicles of historic interest provided they are not used for commercial purposes.

These Regulations were notified to the European Commission pursuant to Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 (O.J. No. L241, 17.9.2015, p 1) under notification number xxxxxxxx. No comments were made during the three month standstill period. The Directive is available online at http://eur-lex.europa.eu

The ECE Regulations are issued by the United Nations Economic Commission for Europe. Copies of the ECE Regulations referred to in these Regulations can be obtained from the UNECE website: http://www.unece.org/trans/main/wp29/wp29regs.html. Copies can also be obtained from Vehicle Policy Branch, Safe and Accessible Travel Division, Department for Infrastructure, Clarence Court, 10-18 Adelaide Street, Town Parks, Belfast BT2 8GB (e-mail: vehicle.standards@infrastructure-ni.gov.uk)

A Regulatory Impact Assessment and Explanatory Memorandum have been produced and are available from Vehicle Policy Branch at the above address or online alongside this Statutory Rule at http://www.legislation.gov.uk/nisr

()	Formerly the Department for Regional Development; see section 1(6) and (11) of, and Schedule 1 to, the Departments Act (Northern Ireland) 2016 (2016 c. 5 (N.I.)). Pursuant to section 1(9) of that Act, the Department of the Environment is dissolved

()	S.I. 1995/2994 (N.I. 18); Article 55 was amended by Article 42(1) of the Road Traffic (Northern Ireland) Order 2007 (S.I. 2007/916 (N.I. 10))

()	See Article 8(1)(b) of, and Part 2 of Schedule 5 to, the Departments (Transfer of Functions) Order (Northern Ireland) 2016 (S.R. 2016 No. 76)

()	S.R. 1999 No. 454; relevant amending Regulations are S.R. 2011 No. 20, S.R. 2011 No. 303 and S.R. 2016 No. 160

()	O.J. No. L 151, 14.6.2018 p, 1