1. ------IND- 2018 0211 LV- EN- ------ 20180524 --- --- PROJET

# DRAFT

CABINET OF MINISTERS OF THE REPUBLIC OF LATVIA

[date] [month] 2018	 Regulation No

Riga 	(Minutes No § [TBC])

Procedure for terminating the waste status of rubber materials obtained from end-of-life tyres

Issued pursuant to 
Article 6 (1)1
of the Waste Management Law

- The regulation lays down the procedure for terminating the waste status of rubber materials obtained from end-of-life tyres.

- Within the meaning of this regulation, secondary raw materials are considered to be rubber materials obtained from end-of-life tyres through breaking, crushing, shredding, cutting or pelletising if, after the processing of the tyres, these materials are intended for sale on the market for further use, with or without binding agents, and they meet all the criteria mentioned in Annex 1 to this regulation.

- The following secondary raw materials may be obtained from end-of-life tyres:

  - rubber powder: a rubber material obtained from crushing end-of-life tyres into particles of up to 0.8 mm;

  - rubber pellets: a rubber material obtained from pelletising end-of-life tyres into particles sized 0.9–20 mm;

  - rubber mulch: a rubber material obtained from crumbling, crushing or shredding end-of-life tyres into irregularly shaped particles sized 10–50 mm. An admixture of textiles is allowed;

  - crushed rubber: a rubber material obtained from crumbling, crushing or shredding end-of-life tyres into irregularly shaped particles sized mainly between 50 and 300 mm. An admixture of metal wires and textiles is allowed;

  - rubber cuttings: a rubber material obtained from crumbling, crushing or shredding end-of-life tyres into irregularly shaped particles sized mainly between 300 to 500 mm. An admixture of metal wires and textiles is allowed.

- Rubber materials shall not be considered as secondary raw materials and shall be considered as waste if they are used for the following purposes:

  - incineration with or without energy recovery;

  - pyrolysis, plasmolysis, gasification and similar technological processes where the physical or chemical properties of the rubber materials are modified;

  - depositing in landfills or storage exceeding 1 year.

- The processor of end-of-life tyres shall ensure the following:

  - for each batch of secondary raw materials, a declaration is filled out showing that the secondary raw materials meet the end-of-waste criteria set out in Annex 1 to this regulation pursuant to Annex 2 thereto (hereinafter referred to as the ‘declaration of compliance’);

  - the record-keeping of recyclable end-of-life tyres is carried out according to the applicable rules and regulations on the official forms of environmental statistics;

  - during transportation, each batch of secondary raw materials is accompanied by a hard copy of the declaration of compliance, enabling the State Environmental Service of the Republic of Latvia to require that the processor of end-of-life tyres produce the original copy of the declaration of compliance.

- The processor of end-of-life tyres shall keep the declaration of compliance for 5 years after the date of issue thereof and produce it at the request of the competent authorities responsible for waste management within 10 working days.

- The declaration of compliance shall be prepared electronically pursuant to the rules and regulations on electronic documents.

- The processor of end-of-life tyres shall attach to each batch of secondary raw materials the technical specification of the product.

- The person responsible for importing the secondary raw materials into Latvia shall ensure that the processor of the end-of-life tyres attaches to each batch of secondary raw materials a statement with information about the physical and chemical properties of the shipment and details of the processor of end-of-life tyres.

- The person responsible for importing secondary raw materials into Latvia shall keep the statement mentioned in paragraph 9 of this regulation for 5 years after the date it was made and shall produce it at the request of the competent authorities responsible for waste management within 10 working days.

- The processor of end-of-life tyres shall introduce a quality management system to ensure the traceability of waste tyre processing.The processor of end-of-life tyres shall observe the requirements of the buyer of secondary raw materials with respect to the quality and physical and chemical properties of the secondary raw materials.

- The quality management system shall include a detailed description of the processing of end-of-life tyres as follows:

  - description of the quality monitoring of the processing of end-of-life tyres pursuant to Annex 1 to this regulation;

  - the sampling methods used, physical and chemical tests of samples, labelling of secondary raw materials, description of packaging and storage processes;

  - compliance and rejection criteria for end-of-life tyres, types of control measures to assess end-of-life tyres for compliance and the ways in which the outcomes of control measures are documented;

  - description of the entire processing cycle of end-of-life tyres, including the subsequent management and storage of the resultant waste, and information about the potential sale of the secondary raw materials;

  - quality compliance criteria of secondary raw materials and self-check criteria pursuant to Annex 1 to this regulation;

  - a list of the waste tyre processor's staff who are responsible for each processing stage of the end-of-life tyres;

  - potential volume of end-of-life tyre processing.

- The processor of end-of-life tyres shall keep the information related to the processing of end-of-life tyres specified in paragraph 12 of this regulation for 5 years after accepting the relevant batch of secondary raw materials.

- The processor of end-of-life tyres shall carry out a revision of the quality management system each year and also when significant changes are made to the technological processes of the secondary raw materials or when the physical or chemical properties of the secondary raw materials are modified.

- At the written request of the competent authorities in charge of waste management, the processor of end-of-life tyres or the person responsible for the import of secondary raw materials into Latvia shall provide access to all areas, premises and documents associated with the processing and storage of end-of-life tyres, in order to assess them for compliance with the requirements stipulated in this regulation.

- The processor of end-of-life tyres shall inform the buyer of secondary raw materials that end-of-life tyres are processed under a quality management system.

- If, upon importing rubber materials obtained from end-of-life tyres from other countries, the competent authorities of the dispatch country and the destination country cannot agree on the classification of the rubber material, Article 28(1) Regulation (EC) No 1013/2006 of the European Parliament and of the Council of 14 June 2006 on shipments of waste shall apply.

Annex 1
to Cabinet 
Regulation No .....
of [date] [month] 2018

End-of-waste criteria for rubber materials obtained from end-of-life tyres

| End-of-waste criteria | Self-check criteria |
| --- | --- |
| - Quality requirements for rubber materials obtained from end-of-life tyres through mechanical processing: - they do not have hazardous properties mentioned in Commission Regulation (EU) No 1357/2014 of 18 December 2014 replacing Annex III to Directive 2008/98/EC of the European Parliament and of the Council on waste and repealing certain Directives, and they do not exceed the concentration limits specified in Annex IV to Regulation (EC) No 850/2004 of the European Parliament and of the Council of 29 April 2004 on persistent organic pollutants and amending Directive 79/117/EEC; - they meet the restrictions of Article 50 of Annex XVII to Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93 and Commission on Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC; - they do not contain noticeable amounts of oil and lubricants; - separation and quantitative determination is carried out according to type/size. | The quality of rubber materials shall be assessed: - by visual inspection, - physical and chemical properties: through laboratory test reports, including those covered by the technical specifications of the buyers of secondary raw materials. At the request of the buyer of secondary raw materials, other laboratory tests may be carried out in accordance with the additional specifications of the buyer of secondary raw materials. For each type of secondary raw material obtained from end-of-life tyres, a testing of representative samples shall be carried out. The representative samples shall be obtained according to the sampling procedure covered by and described in detail as part of the quality management system (sampling methods used, sampling frequency, size, sample types and number, statistical analysis, etc.). The physical and chemical properties shall be tested in a laboratory. The compliance criteria of secondary raw materials specified in paragraph 1 of the annex shall be introduced and described within the operated quality management system. |
| - Requirements for waste to be recycled into secondary raw materials: - only end-of-life tyres corresponding to the waste classification and properties of hazardous waste as specified in rules and regulations may be used; - end-of-life tyres contaminated with hazardous substances or hazardous waste may not be used. | The quality of rubber materials shall be assessed by visual inspection. The quantities of received and processed end-of-life tyres shall be recorded pursuant to the applicable rules and regulations on the official forms of environmental statistics. |
| - End-of-life tyres shall be processed as follows: 3.1. foreign objects such as stones, pieces of metal and debris shall be removed; 3.2. methods and processes used shall not have a detrimental impact on the environment; 3.3. the rules and regulations governing the management and handling of waste shall be followed. | Tyres shall be cleaned mechanically or manually. |

Annex 2
to Cabinet 
Regulation No ....
of [date] [month] 2018

Declaration stating that the secondary raw materials meet the end-of-waste criteria

| 1. | Details of the processor of end-of-life tyres: |
| --- | --- |
|  | Name of legal person: Registration No: |
|  | Actual address: |
|  | Registered office: |
|  | Contact: |
|  | Telephone: |
|  | E-mail: |
| 2. | The technical requirements specified in the technical specification of the buyer of secondary raw materials, including composition, size, admixtures, physical and chemical properties, etc. (please specify the technical requirements): The secondary raw materials obtained from end-of-life tyres meet the requirements set out in the technical specifications. |
| 3. | Shipment size in kg: |
| 4. | The secondary raw materials obtained from end-of-life tyres meet the end-of-waste criteria.* |
| 5. | The processor of end-of-life tyres operates in accordance with the quality management system. |
| 6. | The secondary raw materials included in the shipment are intended only for direct use (please specify the intended use): __________________ |
| 7. | Statement of the processor of end-of-life tyres: I hereby confirm that the information in the declaration is complete and correct. |
| 8. | Other information: |
| 9. | Representative of the economic operator - Full name:__________________________ - Position, signature:_________________________ |
|  | Date: |
|  | Signature: |

* The criteria are defined in Annex 1 to Cabinet Regulation No ____ of [date] [month] 2018 on the procedure for terminating the waste status of rubber materials obtained from end-of-life tyres and are part of the processor’s quality management system for end-of-life tyres.