The Swedish Energy Agency’s Code of Statutes

Published by: Rikard Janson (Chief Legal Officer) ISSN 1650-7703

Regulations amending the Swedish Energy Agency regulations (STEMFS 2017:3) on reporting and calculating in accordance with the Fuel Act;

adopted on 20 January 2021.

By virtue of § 13 of the Fuel Ordinance (2011:346), the Swedish Energy Agency lays down1 that the Agency's regulations on reporting and calculating in accordance with the Fuel Act shall be amended and worded as follows:

a new section – § 12a – shall be introduced and six sections – §§ 2, 4-7, 12, 13, and 16 – shall be amended and worded as follows.

Preliminary provisions and definitions

§ 2 Terms and expressions in these regulations are used in the same way as they are used in the Fuel Act (2011:319) and the Fuel Ordinance (2011:346). The following terms are used in the way specified here:

bio-component: fuel component produced from biomass; fossil component: fuel component of mineral-based origin; fuel type: a fuel that meets a certain classification in accordance with

§§ 3–15 of the Fuel Act (2011:319) or which meets certain national or international standards for fuels, any other fuels that do not meet classifications or standards, and electricity used as fuel;

product: marketed product of the respective fuel type. In cases where the supplier markets several different fuel products that meet the same specification or standard, the different products shall be reported separately under the respective product names;

upstream emission reduction: measure to reduce greenhouse gas emissions that takes place before a raw material is processed at a refinery or other facility for the production of a fossil fuel component.

STEMFS 2021:2

Published on 2 February 2021

1 See Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on Information Society services.

1

STEMFS 2021:2

Reporting

- § The emissions report shall include all fuel products that the fuel supplier has delivered and for which tax has become chargeable under Chapter 5 of the Energy Tax Act (1994:1776) during the year to which the report relates, stating

- amount;

- net calorific value

- the energy source of delivered electricity in cases where the electricity has an origin marking.

- § The emissions report shall include all fossil components that are included in delivered fuel products in accordance with § 4, stating:

- the products in which the components are included;

- types of fossil components broken down by raw material source and process in accordance with Annex 1;

- amount;

- net calorific value; and

- country of origin of the raw material.

- § The emissions report shall include all bio-components that are included in delivered fuel products in accordance with § 4, stating:

- the products in which the bio-components are included;

- type of bio-component;

- amount;

- net calorific value

- raw materials;

- country of origin of the raw material;

- whether the bio-components meet the sustainability criteria set out in the Act (2010:598) on sustainability criteria for biofuels and bioliquids; and

- whether the bio-components have been used in aviation fuel.

- § If the country of origin is outside the European Union or the EEA, but there is no information on which country is the country of origin, the emissions report may state that there is no information on the country of origin.

2

Calculation of greenhouse gas intensity

12 § In calculating a fuel supplier's reduction of greenhouse gas emissions for the purpose stated in § 21 of the Fuel Act (2011:319), the greenhouse gas intensity calculation shall be done with the following formula:

∑ x (GHGix×AF×MJx) – UER

Greenhouse gas intensity#  =  ––––––––––––––––––––––––

∑ x MJx

| # | supplier’s identification |
| --- | --- |
| x | fuels that have been delivered and which shall be reported |
| GHGix | is the sum of greenhouse gas emissions expressed as g CO2eq/MJ from all bio-components and fossil components included in the fuels that have been delivered |
| MJx | energy included in fossil components and bio-components included in the fuels that have been delivered, expressed as megajoules |
| AF | the correction factors for drive system efficiency according to the table below |
| UER | Upstream emission reduction measured as g CO2eq |

Emissions shall be corrected for drive system efficiency and upstream emissions.

| Main area of use | Efficiency factor |
| --- | --- |
| Internal combustion engines | 1 |
| Battery-powered electrical systems | 0.4 |
| Hydrogen fuel cell electric drive system | 0.4 |

§ 12a As a basis for environmental information, a fuel supplier shall calculate the greenhouse gas intensity for marketed products according to the following formula:

∑ x (GHGix × MJx) GHGia = ––––––––––––––––

∑ x MJx

STEMFS 2021:2

Where:

| a | fuel products that have been delivered and which shall be reported |
| --- | --- |
| x | components of the product to be reported |
| GHGix | the greenhouse gas intensity for respective components included in product a, expressed as g CO2eq/MJ |
| MJx | the energy included in components that are included in product a, expressed as megajoules |
| GHGia | greenhouse gas emissions expressed as g CO2eq/MJ from all bio-components and fossil components included in the product that has been delivered. |

3

STEMFS 2021:2

§ 13 In calculations under § 12, a fuel supplier shall calculate greenhouse gas emissions from fossil components in accordance with weighted normal values in Annex 1.

In calculations under § 12a, a fuel supplier shall calculate greenhouse gas emissions from fossil components in accordance with normal values for the specific raw material source in Annex 1.

Upstream emission reduction

§ 16 n order for a fuel supplier to be credited with an upstream emission reduction in calculating greenhouse gas intensity under § 12, the fuel supplier shall have reported the following to the Swedish Energy Agency:

- the start date of the project, which must be later than 1 January 2011;

- annual emission reductions as g CO2eq;

- the period during which the claimed reduction took place;

- the geographical location of the project closest to the emissions source, as latitude and longitude coordinates in degrees to the fourth decimal;

- the minimum standard for annual emissions prior to the installation of reduction measures and annual emissions after the reduction measures have been implemented as CO2eq/MJ for the raw material produced;

- a non-reusable certificate number that uniquely identifies the system and

the claimed greenhouse gas reductions;

- a non-reusable number that uniquely identifies the calculation method and

the associated system.

Entry-into-force provisions

These regulations shall enter into force on 24 February 2021. On behalf of The Swedish Energy Agency

Robert Andrén

Ulf Jonson

4

Annex 1

Average life cycle normal greenhouse gas intensity for fossil fuel components

STEMFS 2021:2

| Raw material source and process | Fuel placed on the market | Life cycle greenhouse gas intensity (gCO2eq/MJ) | Weighted life cycle greenhouse gas intensity (gCO2eq/MJ) |
| --- | --- | --- | --- |
| Conventional crude oil1 | Petrol | 93.2 | 93.3 |
| Liquefied natural gas |  | 94.3 |  |
| Synthetic oil obtained from coal |  | 172 |  |
| Natural bitumen2 |  | 107 |  |
| Oil shale3 |  | 131.3 |  |
| Conventional crude oil1 | Diesel or gas oil | 95 | 95.1 |
| Liquefied natural gas |  | 94.3 |  |
| Synthetic oil obtained from coal |  | 172 |  |
| Natural bitumen2 |  | 108.5 |  |
| Oil shale3 |  | 133.7 |  |
| All fossil sources | LPG in a spark ignition engine | 73.6 | 73.6 |
| Natural gas, EU energy mix | Compressed natural gas in a spark ignition engine | 69.3 | 69.3 |
| Natural gas, EU energy mix | Liquefied natural gas (LNG) in a spark ignition engine | 74.5 | 74.5 |
| Sabatier reaction of hydrogen by electrolysis with non-biological renewable energy | Compressed synthetic methane in a spark ignition engine | 3.3 | 3.3 |
| Natural gas through steam reforming | Compressed hydrogen in a fuel cell | 104.3 | 104.3 |
| Electrolysis fully powered by renewable non-biological energy | Compressed hydrogen in a fuel cell | 9.1 | 9.1 |
| Coal | Compressed hydrogen in a fuel cell | 234.4 | 234.4 |
| Coal with carbon capture and storage of process emissions | Compressed hydrogen in a fuel cell | 52.7 | 52.7 |
| Plastic waste extracted from fossil raw materials | Petrol, diesel or gas oil | 86 | 86 |

1 Conventional crude oil means any refinery raw material exhibiting an API (American Petroleum Institute) gravity that is higher than 10 degrees when situated in a reservoir formation at its place of origin as measured per testing method ASTM D287, and not falling within the definition for CN code 2714

as set out in Regulation (EEC) No 2658/87.	5

STEMFS 2021:2

2 Natural bitumen means any refinery raw material that a) has an American Petroleum Institute (API) gravity of 10 degrees or less when situated in a reservoir formation at the place of extraction as defined pursuant to the testing method of the American Society for Testing and Materials (ASTM) D287; b) has an annual average viscosity at reservoir temperature greater than that calculated by the equation Viscosity (Centipoise) = 518.98e-0.038T, where T is the temperature in Celsius; c) falls within the definition of tar sands under combined nomenclature (CN) code 2714 as outlined in Council Regulation (EEC) No 2658/87; and d) where the mobilisation of the source of the raw material is achieved by mining extraction or thermally enhanced gravity drainage where the thermal energy is mainly derived from sources other than the feedstock source itself.

3 Oil shale means any source of refinery raw material as situated in a rock formation containing solid kerogen and falling within the definition for oil shale under CN code 2714 as outlined in Regulation (EEC) No 2658/87. Mobilisation of the source of the raw material is achieved by mining extraction or thermally enhanced gravity drainage.

6

Arkitektkopia AB, 2021