| THE FRENCH REPUBLIC |  |  |
| --- | --- | --- |
| Department of Ecological Transition Housing |  |  |

Order of […]

On the environmental declaration of products intended for use in building works and the environmental declaration of the products used for calculating the environmental performance of buildings

NOR: LOGL2113185A

Audiences affected: registrants affected by construction and decoration products as well as electrical, electronic and climate engineering equipment used to calculate the environmental performance of buildings.

Purpose: to specify the application of articles R. 412-49 to R. 412-57 of the Consumer Code and of Articles R. 111-20-24 to R. 111-20-30 of the Code on Construction and Housing.

Entry into force: the text enters into force on 1 January 2022.

Notice: the purpose of the Order is to specify the application of articles R. 412-49 to R. 412-57 of the Consumer Code and of Articles R. 111-20-24 to R. 111-20-30 of the Code on Construction and Housing.

The Order establishes:

— the content of environmental declarations of construction and decorating products and of electrical, electronic and climate engineering equipment intended for use in building works and environmental declarations of construction and decoration products, as well as electrical, electronic and climate engineering equipment used to calculate the environmental performance of buildings;

— the methodology for assessing and calculating the information contained in the environmental declaration;

— all the supporting evidence of information contained in the environmental declaration to be made available to the environmental declaration programme and to the control authorities or their representatives;

— the address of the database(s) where the environmental declarations referred to in R. 111-20-25 of the Code on Construction and Housing are made available to the public;

— the conditions for drawing up collective environmental declarations relating to construction or decorating products or equipment by several registrants;

— the conditions for drawing up assignable declarations relating to construction or decorating products or equipment.

References: texts created or modified by this Order can be consulted, in their wording resulting from this change, on the Légifrance website (http://www.legifrance.gouv.fr).

The Minister for Ecological Transition, the Associate Minister responsible for housing, attached to the Minister for Ecological Transition,

Having regard to Directive (EU) 2015/1535 of the European Parliament and of the Council of 9 September 2015 laying down a procedure for the provision of information in the field of technical regulations and of rules on information society services (codified text), and in particular Notification No year/XXX/F;

Having regard to the Consumer Code, in particular Articles L. 412-1 and R. 412-49 to R. 412-57;

Having regard to the Code on Construction and Housing, in particular Articles L. 111-9-2 and R. 111-20-24 to R. 111-20-30;

Having regard to the comments made during the public consultation carried out from [...] to [...], pursuant to Article L. 120-1 of the Environmental Code;

Having regard to the opinion of the High Council on Construction and Energy Efficiency, dated 13 April 2021;

Hereby order the following:

Article 1

(Definitions)

For the purposes of this Order, the following definitions shall apply:

“Indicator”: quantifiable value related to environmental aspects;

The word “registrant” is understood as meaning “responsible for placing on the market” in Article R. 412-49 of the Consumer Code or as meaning “registrant” in Article R. 111-20-24 of the Code on Construction and Housing;

‘Functional unit’: quantified performance of a system of products intended for use as a unit of reference in a life cycle analysis;

‘Declared unit’: quantity of equipment, if any, of a construction or decorating product, intended for use as a reference unit in a life cycle analysis in addition to the functional unit;

‘Reference life of the construction or decorating product or equipment’”: serviceable life that can be expected for a construction or decorating product or equipment according to a set of reference conditions for use and which can be used as a basis for estimating life under other conditions of use;

‘Supplementary product’: a general term for any product that must necessarily be associated with the construction, decoration or primary equipment during each of the implementation stages and life in practice;

‘Assignable declaration’: environmental declaration with rules for adapting the information mentioned therein to correspond to a particular product and a computer tool to apply those rules;

In the following articles, the term ‘product’ is understood to mean ‘construction products’, ‘decorating products’ and ‘electrical, electronic and climate engineering equipment’ as defined in Article R. 111-20-24 of the Code on Construction and Housing.

# Article 2

(Scope)

The provisions of this Order apply to the products mentioned in Article R. 412-50 of the Consumer Code and in Article R. 111-20-24 of the Code on Construction and Housing.

# Article 3

(Contents of the environmental declaration)

The environmental declaration referred to in Article R. 412-51 of the Consumer Code and in Article R. 111-20-25 of the Code on Construction and Housing contains the following information:

1. The values, for the production stage, the stage of the construction process, the usage stage, the end-of-life stage and the sum of these stages, of the following indicators:

- describing the environmental impacts:

- global warming (greenhouse gas emissions);

- depletion of the ozone layer;

- acidification of soil and water;

- eutrophication;

- photochemical ozone formation;

- depletion of abiotic resources – elements;

- exhaustion of abiotic resources – fossil fuels;

    - describing the use of resources:

- usage of renewable primary energy, excluding renewable primary energy resources used as raw materials;

- use of renewable primary energy resources used as raw materials;

- total usage of renewable primary energy resources (primary energy and primary energy resources used as raw materials);

- usage of non-renewable primary energy, excluding non-renewable primary energy resources used as raw materials;

- use of non-renewable primary energy resources used as raw materials;

- total usage of non-renewable primary energy resources (primary energy and primary energy resources used as raw materials);

- use of secondary material;

- use of renewable secondary fuels;

- use of non-renewable secondary fuels;

- net use of fresh water.

    - describing the categories of waste:

- hazardous waste disposed of;

- non-hazardous waste disposed of;

- radioactive waste disposed of;

    - describing outgoing flows:

- components intended for reuse;

- materials intended for recycling;

- materials intended for energy recovery;

- energy supplied outdoors.

2. From 1 January 2022:

  - The use of materials from renewable resources incorporated into the product expressed through an indicator of the amount of carbon from the atmosphere stored in the construction or decorating product;

  - The values of the indicators listed in 1. for each of the sub-stages of the usage stage, i.e. the following sub-stages:

  - use or application, excluding the energy and water requirements of the building’s operating phase;

  - maintenance;

  - repair;

  - replacement;

  - rehabilitation;

  - use of energy during use;

  - use of water during use.

    - In the event of an environmental declaration affected by the requirements of Article 9 of this Order, the ranges of variation (maximum and minimum values) of the control indicators defined in Article 9.

3. For environmental declarations verified from 1 October 2022:

  - The use of materials from renewable resources incorporated into the product expressed through an indicator of the amount of carbon from the atmosphere stored in the equipment;

  - The values of the indicators on the benefits and expenses relating to end-of-life valuation.

4. From 1 January 2025:

  - The values of the indicators on the benefits and expenses relating to end-of-life valuation.

5. The functional unit or declared unit of the product;

6. The reference life of the product;

7. Description of the products constituting the functional unit or declared unit (quantity of the main product, quantity of packaging, quantity of complementary products relating to the implementation or life in practice);

8. The product’s scope;

9. The following information on the product covered by the environmental declaration: group, description(s) or commercial designation(s), name(s) or designation of the registrant(s);

10. The date of the environmental declaration;

11. The verification certificate and the contact details of the auditor who carried out the independent third-party verification referred to in Article R. 412-55 of the Consumer Code and in Article R. 111-20-27 of the Code on Construction and Housing;

12. The registrant’s details;

13. The address of the website where this information is available free of charge.

# Article 4

(Provision of supporting evidence by the registrant)

The registrant shall keep at the disposal of the environmental declaration programme and the administrative authorities responsible for the checks or their representatives, all the items, or the contact details of the natural or legal persons holding these items, to justify the information contained in the environmental declaration, in particular:

- the origin of the raw materials, materials and components of the product;

- identification of inputs not included in the life cycle inventory in accordance with the cut-off rule;

- the total mass of inputs not included in the life cycle inventory due to the cut-off rule;

- the results of inventory calculations;

- supporting evidence for the reference life of the product;

- in the event generic data have to be used that originate from public or private databases, documentation relating to how technologically, geographically and temporally representative the generic data used are, references of the databases from which they originate and references of the data modules used;

- the scenarios upon which the life cycle inventory is based;

- the production site(s) covered by the environmental declaration;

- the production of each site expressed with the unit of quantity defined in the functional unit;

- where a sampling method is used, the evidence that the sample used is representative, in particular from a geographical, temporal and technological point of view, of the production of the product;

- the constituent elements of the framework of validity for environmental declarations affected by the requirements of Article 9;

- The constituent elements of the assignable declaration referred to in Article 11.

Article 5

(Units)

The functional unit formats for each category of construction or decorating products are defined in Annex I.

The formats of functional units or units declared for equipment are defined in Annex II.

# Article 6

(Evaluation and calculation methods)

The methods for evaluating and calculating the information referred to in Article 3, and in particular indicators, shall be subject to all the following conditions:

  - They do not result in omitting processes representing a total of more than 5% of the value of each of the items of information referred to;

  - They do not result in a product being granted that is subject to the environmental declaration of benefits assigned to other products when calculating the items of information referred to, with the exception of indicators relating to profits and expenses associated with the end-of-life valuation;

  - They comply with the NF EN 15804 + A1 standard: 2014-04 or the XP C08-100-1 standard: 2016-12 or the EN 50693 standard: 2019-08, or any equivalent standard;

  - They shall comply with the method of calculating the assessment of profits and expenses relating to end-of-life valuation as defined in Annex III.

In addition, for environmental declarations verified from 1 October 2022, where the environmental declaration covers different ways of laying the product covered by it, the methods of assessing and calculating the information referred to shall comply with the following rules:

- The following control indicators are assessed for each of these installation modes: total climate warming, total non-renewable primary energy use, non-hazardous waste disposed of;

- For each of these control indicators, the maximum value obtained for the sum of the steps referred to in Article 3 shall be compared with the average of the values obtained (i.e. the average impact of the different installation modes). If the maximum value for one of these indicators is greater than 1.1 times the mean value, it is therefore the maximum value (i.e. the maximum impact among the different laying modes) that has to be reported for each of the indicators listed in the environmental declaration. Otherwise, the average value can be reported for each indicator.

# Article 7

(Filing in database(s))

The registrant shall provide the verified environmental declaration in the database(s) indicated in the agreement signed by the Minister responsible for construction and the legal entity representing the environmental declaration programme.

The address of the database or the addresses of the databases are available on the website of the Minister responsible for construction.

# Article 8

(Updated Environmental Declaration)

The environmental declaration shall be updated with each significant change to the product, when this is necessary to reflect technological developments or other circumstances that may affect the content or accuracy of the environmental declaration and at least every five years.

# Article 9

(Collective environmental data)

The environmental declaration may be based on a collective environmental declaration of similar products from several registrants.

1. This collective environmental declaration meets the following conditions:

- It concerns a ‘standard product’;

- It shall be established under the conditions referred to in the preceding Articles;

- The homogeneity of parameters that significantly influence the value of each indicator must be ensured; to this end, the collective environmental declaration shall contain a validity framework, as defined in Annex IV, which gathers the following information:

- The identification of the influential parameters, specifying whether they are completed with generic or specific data;

- The periods of validity of these parameters.

2. The registrant who submitted the collective environmental declaration is responsible for the information it contains, including a list of the products concerned and the conditions under which products may be linked to it, if necessary.

3. The registrants of the products covered by this collective environmental declaration shall meet the following conditions:

- They are responsible for the information they transmit to the above-mentioned registrant;

- They shall respect the validity framework of the collective environmental declaration;

- They shall provide a certificate indicating compliance with the validity framework of the collective environmental declaration;

- They shall keep available for the inspection authorities the supporting evidence of compliance with the validity framework.

4. The independent third party verification referred to in Article R. 412-55 of the Consumer Code and referred to in Article R. 111-20-27 of the Code on Construction and Housing for the products attached to the collective environmental declaration consists of an independent third-party verification of the collective environmental declaration.

- 5. The control indicators to be considered shall be as a minimum:

- total global warming;

- total non-renewable primary energy use;

non-hazardous waste disposed of.

6. Collective environmental declarations must meet the following requirements:

- where the maximum value of each control indicator, obtained for the sum of the steps referred to in Article 3 and through the calculation of the variability described in Annex IV, is less than or equal to the average value of the indicator multiplied by a limit value, then the values of the indicators may be calculated with the mean values of the sensitive parameters. Otherwise, the upper threshold must be declared (maximum value the indicator can achieve).

- For collective environmental declarations made from 1 October 2022, the above-mentioned limit value is “1.4” and the calculation of the variability mentioned above shall at least cover the sensitive parameters relating to:

  - the composition of the product: quantity and type of materials;

  - the quantities of packaging;

  - manufacturing processes apart from the extraction and processing of raw materials (stages A2 and A3);

- For collective environmental declarations made from 1 October 2022, the above-mentioned limit value is “1.3” and the calculation of the variability referred to above shall at least cover the sensitive parameters relating to:

  - The composition of the product: quantity and type of materials;

  - The quantities of packaging;

  - Manufacturing processes apart from the extraction and processing of raw materials (stages A2 and A3);

  - Transport to the site (stage A4).

# Article 10

(Environmental data range)

The environmental declaration may be based on similar products from the same registrant. These environmental declarations must meet the requirements for collective environmental declarations set out in Article 9.

# Article 11

(Assignable environmental data)

The environmental declaration may be based on an assignable declaration.

This assignable declaration meets the following conditions:

- It concerns a ‘standard product’;

- It shall be established under the conditions referred to in the preceding Articles;

- It shall contain a set of calculation rules for adapting, on the basis of changeable parameters, the information mentioned therein to correspond to a product similar to the standard product but the dimensions, composition or certain life-cycle processes of which differ;

- The value of the changeable parameters referred to above must be able to be verifiable upon delivery of a building;

- Ιt contains a validity framework defining the values that may be taken by the changeable parameters referred to above and the characteristics to be met by the similar product referred to above.

Any assignable declaration verified from 1 October 2022 shall be accompanied by a computer calculation tool for applying the above-mentioned calculation rules and for exporting the information required by Article 3, together with the list of the above-mentioned changeable parameters and their values. All this information is exported as a computer file according to the format defined on the website of the Ministry in charge of construction.

The independent third-party verification referred to in Article R. 412-55 of the Consumer Code and referred to in Article R. 111-20-27 of the Code on Construction and Housing shall also, in the case of an assignable declaration, cover the calculation rules referred to in this article and the computer calculation tool referred to in the preceding paragraph.

# Article 12

(Certification)

The certification of environmental characteristics referred to in Article R. 412-53 of the Consumer Code is presumed to conform if it takes into account all the requirements of standard NF EN ISO 14024: 2018-03. This certification of construction or decorating products or equipment is issued by an organisation accredited for this purpose by the French Accreditation Committee (COFRAC) or by another accreditation body signatory to multilateral mutual recognition agreements.

This section does not apply to environmental declarations used in the calculation of the environmental performance of buildings.

# Article 13

The Order of 23 December 2013 concerning the environmental declaration of construction and decoration products for use in building works is repealed.

The Order of 31 August 2015 on the environmental declaration of electrical, electronic and climate engineering equipment for use in building works is repealed.

# Article 13

The Director of Housing, Urban Development and Landscapes shall be responsible for implementing this Order which shall be published in the Official Journal of the French Republic.

The Associate Minister for Housing, attached to the Minister

for Ecological Transition

For and on behalf of the Minister:

The Director of Housing, Urban Development and Landscapes,

François Adam

The Minister for Ecological Transition,

For and on behalf of the Minister:

The Director of Housing, Urban Development and Landscapes,

François Adam

Annexes

Annex I

Functional unit formats for each category of construction or decorating products.

| No. | FUNCTION | CATEGORY | UNIT FORMAT functional in units of the international system |  |
| --- | --- | --- | --- | --- |
| 1 | Roads, miscellaneous networks (including internal networks) | Tanks and reservoirs | A unit of a given volume |  |
|  |  | Drainage and sanitation networks (rainwater and waste water) | m medium |  |
|  |  | Water supply networks (interior and exterior) | m medium |  |
|  |  | Gas supply networks | m medium |  |
|  |  | Drainage system | m medium |  |
|  |  | Roads and external surfacing | m² of surfacing for a given use (external terrace, low traffic roads, parking, etc.) |  |
|  |  | Fencing | m of fencing |  |
|  |  | Sheaths/ducts | m |  |
|  |  | Miscellaneous |  |  |
| 2 | Structure, masonry, major work, frame | Flue blocks and chimney flue pipes | m of given inner section duct |  |
|  |  | Screeds/floating screeds | m² for a given thickness |  |
|  |  | Frames | m² covered area or m3 |  |
|  |  | Braces | m² of bracing |  |
|  |  | Slabs and pre-cast slabs | m² |  |
|  |  | Horizontal bearing elements (beams, joists, insulation, hollow-core slabs, voided slabs, lintels) | m of load-bearing element (specify scope and section) or m² (for surfacing element) |  |
|  |  | Vertical load-bearing elements (posts/columns/pillars) | m of element (specify section) |  |
|  |  | Stairs (internal and external) | A unit for a given height elevation |  |
|  |  | Foundations | m³ |  |
|  |  | Small masonry components | m² (specify thermal resistance) |  |
|  |  | Floors | m² of flooring (specify the load supported) |  |
|  |  | Walls (architectural elements) | m² (specify thickness) |  |
|  |  | Miscellaneous |  |  |
| 3 | Façades | Cladding (dressing, weatherboarding, cladding) | m² of façade |  |
|  |  | Curtain walls and skylights | m² of façade (specify percentage of opaque surface) |  |
|  |  | External surfacing of façades (including composite external thermal insulation systems) | m² of façade (specify the thermal resistance if relevant) |  |
|  |  | Miscellaneous |  |  |
| 4 | Roofing/sealing | Large roofing elements | m² of coverage |  |
|  |  | Small roofing elements | m² of coverage |  |
|  |  | Waterproofing and sealing products for buried walls | m² of coverage |  |
|  |  | Products for roofing sealing | m² of coverage |  |
|  |  | Green roofing elements | m² of coverage (specify the type of vegetation supported and the thermal characteristics if relevant) |  |
|  |  | Miscellaneous |  |  |
| 5 | Internal and external carpentry, closures | Windows, French windows | m² of joinery (open surface area before fitting) (specify U-value, solar factor, air water wind rating and light transmission) |  |
|  |  | Guardrails | m of guardrails |  |
|  |  | Doors (internal, external, portal, etc.) | m² (open surface area before installation) |  |
|  |  | Shutters, roller blinds, louvres, sun-breaks | m² of opaque/protected surface area |  |
|  |  | Skylights | m² of joinery (open surface area before fitting) (specify U-value, solar factor, air water wind rating and light transmission) |  |
|  |  | Miscellaneous |  |  |
| 6 | Insulation | Chevroned boxes, roof panels | m² of insulation (specify thermal resistance) |  |
|  |  | Lining systems | m² of insulation (specify thermal resistance) |  |
|  |  | Additional insulation | m² of insulation (specify thermal resistance) |  |
|  |  | Hollow-core slabs, insulating panels | m² of insulation (specify thermal resistance) |  |
|  |  | Thermal and acoustic wall insulators (ITI) | m² of insulation (specify heat and sound resistance) |  |
|  |  | Thermal and acoustic insulators for attics | m² of insulation (specify heat and sound resistance) |  |
|  |  | Heat and sound insulation, in bulk | m² of insulation (specify heat and sound resistance) |  |
|  |  | Heat and sound insulators for roofing terraces | m² of insulation (specify heat and sound resistance) |  |
|  |  | Under-screed heat and sound insulation | m² of insulation (specify heat and sound resistance) |  |
|  |  | Non-load-bearing distributed insulation | m² of insulation (specify heat and sound resistance if relevant) |  |
|  |  | Heat and sound insulators for walls (ITE) | m² of insulation (specify heat and sound resistance) |  |
|  |  | Sound insulators for partitions | m² of insulation (specify heat and sound resistance) |  |
|  |  | Heat and sound insulation under slabs | m² of insulation (specify heat and sound resistance) |  |
|  |  | Insulation under paving and ground slabs | m² of insulation (specify heat and sound resistance) |  |
|  |  | Miscellaneous |  |  |
| 7 | Partitioning, suspended ceilings | Partitioning | m² of partitioning (specify an acoustic performance if relevant) |  |
|  |  | Suspended ceilings, stretch ceilings | m² of suspended ceiling (specify acoustic performance) |  |
|  |  | Miscellaneous |  |  |
| 8 | Flooring and wall coverings, paints, decorating products | Paints, stains and varnishes, paint renders (without saturators) | m² of covered area |  |
|  |  | Skirting boards | m of skirtings |  |
|  |  | Wall and ceiling coatings | m² of coating |  |
|  |  | Hard floor coverings | m² of coating (specify the UPEC class of use or classification of resistance to wear and punching, water behaviour and resistance to chemical agents, or equivalent) |  |
|  |  | Soft floor coverings | m² of coating (specify the UPEC class of use or classification of resistance to wear and punching, water behaviour and resistance to chemical agents, or equivalent) |  |
|  |  | Acoustic products | m² of coating (specify the UPEC class of use or classification of resistance to wear and punching, water behaviour and resistance to chemical agents, or equivalent) |  |
|  |  | Saturators | m² of covered area |  |
|  |  | Miscellaneous |  |  |
| 9 | Preparation and implementation products | Carpentry adhesives | kg (specify density) |  |
|  |  | Repairs and assembly | kg (specify density) |  |
|  |  | Ground | m² of adhesive to the ground |  |
|  |  | Masonry mortars | kg (specify density) |  |
|  |  | Tiling | m² tiled |  |
|  |  | Decoration | kg (specify density) |  |
|  |  | Mastics and polyurethane foams | m (specify linear mass) |  |
|  |  | Building chemistry | kg (specify density) |  |
|  |  | Products for concrete | kg (specify density) |  |
|  |  | Synthetic resins | m² (specify density) |  |
|  |  | Miscellaneous |  |  |
| 10 | Sanitary and bathroom equipment | Taps and shower columns | Unit (specify type) |  |
|  |  | Bathtub | Unit (specify volume of capacity in litre and shape) |  |
|  |  | Shower tray | Unit (specify dimensions: width and length) |  |
|  |  | Sink | Unit (specify dimensions: width, length and number of bins) |  |
|  |  | Wash basin | Unit (specify dimensions: width, length and number of bins) |  |
|  |  | WC – Toilet – Bowl | Unit (specify dimensions: width and length) |  |
|  |  | Miscellaneous |  |  |
| 00 | Other | Miscellaneous |  |  |

Annex II

Formats of functional units or units declared for the facilities.

| No. | Function | Category | Functional unit | Declared unit | Group | Solution |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Installation facility for the electricity and communication networks ≤ 63 Amp. | Wall fittings | Unit | Unit | Automated systems | Motion sensors |  |
|  |  |  |  |  |  | Thermostats |  |
|  |  |  |  |  |  | Dimmers |  |
|  |  |  |  |  | Equipment boxes | For drywalling |  |
|  |  |  |  |  |  | For masonry |  |
|  |  |  |  |  | Controls | Switches |  |
|  |  |  |  |  |  | Push buttons |  |
|  |  |  |  |  |  | Others: Ducted ventilation, roller blind, etc. |  |
|  |  |  |  |  | Plates and supports | Wall equipment sub-assembly |  |
|  |  |  |  |  | Sockets | Low current outlets |  |
|  |  |  |  |  |  | High current sockets |  |
|  |  |  |  |  |  | Various sockets (TV, loudspeaker, IT, etc.) |  |
|  |  |  |  |  | Signalling | Audio output: loudspeaker |  |
|  |  |  |  |  |  | Audio output: bells and chimes |  |
|  |  |  |  |  |  | Indicator lights |  |
|  |  |  |  |  | Accessories | Remote controls, shutters, clamps |  |
|  |  | Modular equipment for enclosures | Unit | Unit | General protective equipment | Connections |  |
|  |  |  |  |  |  | Circuit breakers |  |
|  |  |  |  |  |  | Mains connection circuit breakers |  |
|  |  |  |  |  |  | Switches/isolators |  |
|  |  |  |  |  |  | Source inverters |  |
|  |  |  |  |  |  | Differential relays |  |
|  |  |  |  |  | Protection against lightning | Lightning rods |  |
|  |  |  |  |  |  | Telecommunications lightning rods |  |
|  |  |  |  |  | Line protection | Circuit breakers |  |
|  |  |  |  |  |  | Divisional circuit breakers |  |
|  |  |  |  |  |  | Motor circuit breakers |  |
|  |  |  |  |  |  | Fuses |  |
|  |  |  |  |  | Earth leakage protection | Differential blocks |  |
|  |  |  |  |  |  | RCD circuit breakers |  |
|  |  |  |  |  |  | RCD switches |  |
|  |  |  |  |  | Lighting management | Photocell switches |  |
|  |  |  |  |  |  | Solar management |  |
|  |  |  |  |  |  | Dimmers |  |
|  |  |  |  |  | Energy management | Load shedders |  |
|  |  |  |  |  |  | Energy managers |  |
|  |  |  |  |  | Blind and shutter management | Blind and shutter management |  |
|  |  |  |  |  | Time management | Intersecting schedules |  |
|  |  |  |  |  |  | Timers |  |
|  |  |  |  |  |  | Timer relays |  |
|  |  |  |  |  | Thermal management (heating, ventilation) | Thermostats |  |
|  |  |  |  |  | Measurement | Energy meters |  |
|  |  |  |  |  |  | Rain, sun sensors, etc. |  |
|  |  |  |  |  |  | Voltmeters, ammeters |  |
|  |  |  |  |  | Control devices | Push buttons |  |
|  |  |  |  |  |  | Contactors |  |
|  |  |  |  |  |  | Switches, inverters, commutators, etc. |  |
|  |  |  |  |  |  | Remote impulse switches |  |
|  |  |  |  |  | Signalling devices | Chimes and ringtones |  |
|  |  |  |  |  |  | Indicator lights |  |
|  |  | Home automation and communication systems | Unit | Unit | Multimedia networks and VDI (Voice Data Image) | Modular equipment for home automation |  |
|  |  |  |  |  |  | Home automation terminal equipment |  |
|  |  |  |  |  |  | Signalling/detection distributors |  |
|  |  |  |  |  |  | Wi-Fi access points |  |
|  |  | Enclosures | Unit | Unit | Cabinets | Polyester cabinets |  |
|  |  |  |  |  |  | Metal cabinets |  |
|  |  |  |  |  |  | Connections |  |
|  |  |  |  |  |  | Cabinet equipment |  |
|  |  |  |  |  | Boxes | Terminal covers |  |
|  |  |  |  |  |  | Flush-fitting boxes |  |
|  |  |  |  |  |  | Watertight boxes |  |
|  |  |  |  |  |  | Protruding boxes |  |
|  |  |  |  |  |  | VDI boxes |  |
|  |  |  |  |  |  | Junction boxes |  |
|  |  |  |  |  | Other | Desks, heat management of enclosures |  |
|  |  | Cable routing solutions | Metre | Unit | Cable routing systems and cable racks | Wire cable paths |  |
|  |  |  |  |  |  | Tiled cable trays |  |
|  |  |  |  |  |  | Cable racks |  |
|  |  |  |  |  | Trunking systems and ducts | Ducts |  |
|  |  |  |  |  |  | Profiled ducts |  |
|  |  |  |  |  |  | Rigid tubes and ducts |  |
|  |  |  |  |  |  | Supply ducts |  |
|  |  |  |  |  |  | Floor ducts |  |
|  |  |  |  |  |  | Installation ducts |  |
|  |  |  |  |  |  | Mouldings and skirting boards |  |
|  |  |  | Unit | Unit | Other point products for cable management | Technical installation housing (GTL) |  |
|  |  |  |  |  |  | Floor ducts |  |
|  |  |  |  |  |  | Splitter boxes and vertical installation columns |  |
|  |  |  |  |  |  | Cable trunking for cabinets |  |
|  |  |  |  |  |  | Other and accessories |  |
|  |  | Other |  |  | Connection/junction (outside enclosure) | Junction blocks |  |
|  |  |  |  |  |  | Connectors |  |
|  |  |  |  |  |  | Industrial sockets |  |
|  |  |  |  |  | Power supplies | Transformers |  |
|  |  |  |  |  |  | Central sources |  |
|  |  |  |  |  | Mobile electrical equipment | Inspection light |  |
|  |  |  |  |  |  | Multi-socket adaptor blocks, telephone splitter sockets, other |  |
|  |  |  |  |  | Flap opening motor | Motorisation of doors, gates, etc. |  |
|  |  |  |  |  |  | Motorisation of roller blinds |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 2 | Wires and cables | Electrical networks | km |  | Medium voltage cables |  |  |
|  |  |  |  |  | Low voltage cables <1 kV |  |  |
|  |  |  |  |  | Electrical network accessories |  |  |
|  |  | Electricity and data transfer inside the building | km |  | Energy wires and cables |  |  |
|  |  |  | m |  | Copper and optical fibre communication cables |  |  |
|  |  |  |  |  | Accessories inside the building |  |  |
|  |  | Other wires and cables | m |  | Uninsulated |  |  |
|  |  |  |  |  | Heating cables |  |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 3 | Safety of individuals and access control | Access detection and control | Unit | Unit | Intruder detection system | Intruder detection equipment |  |
|  |  |  |  |  |  | Control equipment (keyboard, remote control, etc.) |  |
|  |  |  |  |  |  | Alarm consoles |  |
|  |  |  |  |  |  | Signalling equipment (siren, transmitter, etc.) |  |
|  |  |  |  |  | Access control system | Audio door entry |  |
|  |  |  |  |  |  | Video door entry |  |
|  |  | Video surveillance | Unit | Unit | Video surveillance system | Image capture equipment (cameras) |  |
|  |  |  |  |  |  | Image management equipment |  |
|  |  |  |  |  |  | Interconnections |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 4 | Security of the building | Security lighting system | Unit | Unit | Surroundings | Self-contained emergency lighting units (B.A.E.S) |  |
|  |  |  |  |  |  | Light fitting for central source (L.S.C) |  |
|  |  |  |  |  |  | Floodlight units |  |
|  |  |  |  |  | Evacuation | Self-contained emergency lighting units (B.A.E.S) |  |
|  |  |  |  |  |  | Independent residential lighting units (B.A.E.H) |  |
|  |  |  |  |  |  | Dual function unit (B.A.E.S + B.A.E.H) |  |
|  |  |  |  |  |  | Light fitting for central source (L.S.C) |  |
|  |  |  |  |  |  | B.A.E.S + D.L (lighting device) |  |
|  |  |  |  |  |  | B.A.E.S + D.B.R (enhanced identification device) |  |
|  |  |  |  |  |  | L.S.C + D.B.R |  |
|  |  |  |  |  | Emergency lighting | Independent residential lighting units (B.A.E.H) |  |
|  |  |  |  |  |  | Dual function unit (B.A.E.S + B.A.E.H) |  |
|  |  |  |  |  | Intervention | Portable stand-alone intervention units (B.A.P.I) |  |
|  |  |  |  |  | Supply | Central source for emergency light fitting |  |
|  |  | Fire safety | Unit | Unit | Fire safety and technical alarm | Independent smoke detectors/alarms |  |
|  |  |  |  |  |  | Technical alarms and detectors (flooding, gas, etc.) |  |
|  |  |  |  |  |  | Self-contained carbon monoxide detectors |  |
|  |  |  |  |  |  | Technical alarms and detectors |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 5 | HVAC equipment | Heating and/or cooling and/or domestic hot water production | kW | unit | Boiler (heating alone) | Gas boiler |  |
|  |  |  |  |  |  | Fuel oil boiler |  |
|  |  |  |  |  |  | Hybrid boiler |  |
|  |  |  |  |  |  | Biomass boiler |  |
|  |  |  | kW | unit | Mixed boiler (heating and domestic hot water) | Gas boiler |  |
|  |  |  |  |  |  | Fuel oil boiler |  |
|  |  |  |  | unit |  | Independent biomass appliances |  |
|  |  |  | kW | unit | Heat pump | Electric compression heat pump |  |
|  |  |  |  |  |  | Hybrid heat pump |  |
|  |  |  | kW | unit | Water circulation pump | Radiator |  |
|  |  |  |  |  |  | Heated towel rail |  |
|  |  |  | kW | unit | Direct electrical heating via a visible fixed unit | Convector |  |
|  |  |  |  |  |  | Radiant |  |
|  |  |  |  |  |  | Radiator |  |
|  |  |  |  |  |  | Equipment with a secondary function as a heated towel rail |  |
|  |  | Domestic hot water production | Litre | unit | Individual storage water heater | Electric water heater |  |
|  |  |  |  |  |  | Thermodynamic water heater |  |
|  |  |  |  |  |  | Gas water heater |  |
|  |  |  |  |  |  | Individual solar water heater |  |
|  |  | End comfort units | kW | unit | Fan heaters | Fan heaters |  |
|  |  |  | m3 | unit | Climate beams | Passive climate beams (EN 15518) |  |
|  |  |  |  |  |  | Active climate beams (EN 15116) |  |
|  |  |  | kW | unit | Dynamic radiators | Dynamic radiators |  |
|  |  | Ducts and network accessories | ml of duct |  |  | Flexible ducts |  |
|  |  |  |  |  |  | Rigid ducts |  |
|  |  |  |  |  |  | Bends and accessories |  |
|  |  | Air diffusion | m3/hr or dm2 | unit |  | Passive terminals |  |
|  |  |  |  |  |  | Diffusers |  |
|  |  |  |  |  |  | Air inlets |  |
|  |  |  |  |  |  | Extractor and blower openings |  |
|  |  | Air treatment | m3/hr | unit |  | Air treatment plant |  |
|  |  |  |  |  |  | Double flow unit |  |
|  |  |  |  |  |  | Air filters |  |
|  |  | Ventilation box | m3/hr | unit |  | Single flow mechanically controlled ventilation (VMC) |  |
|  |  |  |  |  |  | Double flow mechanically controlled ventilation (VMC) |  |
|  |  |  |  |  |  | Ventilation box |  |
|  |  | Refrigeration | kW | unit |  | Ice water production unit |  |
|  |  |  |  |  |  | Cooling tower |  |
|  |  |  |  |  |  | Air coolers |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 6 | Local electricity production | Electricity generator | kWh | unit | Photovoltaic modules | Crystalline silicon (monocrystalline or polycrystalline) |  |
|  |  |  |  |  |  | Thin layers |  |
|  |  |  |  |  | Wind turbines | Horizontal |  |
|  |  |  |  |  |  | Vertical |  |
|  |  | Conversion and regulation | kWh | unit | Conversion | Synchronous inverters |  |
|  |  |  |  |  |  | Autonomous inverters |  |
|  |  |  |  |  |  | DC/DC converter |  |
|  |  |  |  |  |  | AC/DC converter |  |
|  |  |  |  |  | Regulation | Load controller |  |
|  |  | Storage | kWh | unit | Electrochemical battery | Lead - acid |  |
|  |  |  |  |  |  | Lithium - ion |  |
|  |  |  |  |  |  | Other |  |
|  |  |  |  |  | Miscellaneous |  |  |
|  |  | Monitoring |  |  | Monitoring | Monitoring equipment |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 7 | Lighting equipment | Equipment for indoor lighting and power supplies | Lumen | unit | Recessed lighting | Linear interior flush-fitting lighting for office lighting |  |
|  |  |  |  |  |  | Non-linear interior recessed lighting for office lighting |  |
|  |  |  |  |  |  | Indoor recessed accent lighting (spots) |  |
|  |  |  |  |  | Surface-mounted lights | Ceiling lights, linear internal suspended fittings for office or industrial lighting |  |
|  |  |  |  |  |  | Functional internal linear surface-mounted wall lighting |  |
|  |  |  |  |  |  | Ceiling lights, surface-mounted wall lighting, decorative internal suspended fittings |  |
|  |  |  |  |  |  | Internal portholes |  |
|  |  |  |  |  |  | Interior lighting strips (with no optical devices, in contrast to linear products) |  |
|  |  |  |  |  |  | Internal spotlights, wall or rail-mounted or suspended, for accent lighting |  |
|  |  |  |  |  |  | Water-resistant internal light fittings |  |
|  |  |  |  |  |  | LED flexible bars and strips (including power supply) |  |
|  |  |  |  |  | Suspended fittings | Industrial fittings |  |
|  |  |  |  |  |  | Continuous on-line structures for industrial or commercial lighting |  |
|  |  |  | unit | unit | Mountings | Spotlight supply rails |  |
|  |  | Equipment for outdoor lighting and power supplies | Lumen | unit | Spotlights | Spotlights for sports and large areas |  |
|  |  |  |  |  |  | Outdoor spotlights for decorative lighting |  |
|  |  |  |  |  | Light fittings for outdoor lighting, other than spotlights | Light fittings for decorative lighting |  |
|  |  |  |  |  |  | Light fittings for functional lighting |  |
|  |  |  |  |  |  | Period lighting |  |
|  |  |  |  |  | Recessed spotlights | Outdoor spotlights to be recessed in flooring or walls |  |
|  |  |  |  |  | Recessed lighting for orientation | Outdoor orientation lighting fixtures to be recessed in the floor or walls |  |
|  |  |  |  |  | Posts and columns | Decorative external terminals |  |
|  |  |  |  |  |  | Decorative external light columns |  |
|  |  |  |  |  | Water-resistant light fittings IP > 54 | Surface-mounted wall lights and waterproof portholes |  |
|  |  |  |  |  |  | Submersible recessed lighting for swimming pools or fountains |  |
|  |  |  |  |  |  | Submersible spotlights for swimming pools or fountains |  |
|  |  |  | unit | unit | Masts and mountings | Masts |  |
|  |  |  |  |  |  | Butts |  |
|  |  |  |  |  |  | Various mountings for outdoor light fittings |  |
|  |  | Miscellaneous |  |  |  |  |  |
| 00 | Other | Miscellaneous |  |  |  |  |  |

Annex III

Calculation of profits and expenses relating to end-of-life valuation.

The display of a positive value results in a net charge and a negative value results in a net profit.

1. In the event of recycling, for a given flow of recycled waste (and thus a given lambda material), the following formula applies:

ChargeNetRecycl = MSval (ISval – IVval) – MS (IS – IV),

where

MSval = mass (kg) of secondary material actually recovered from the mass of stock collected (lambda component of material in the ‘materials destined for recycling’ flow);

IVval = production inventory (per kg) of material (or mixture) for which the recycled lambda secondary material is substituted in the downstream system using it (i.e. not the production inventory of the downstream product, but the production inventory material that makes up this downstream product);

ISval = inventory (per kg) of production of ready-to-use secondary material in the downstream system from the stock;

MS = mass (kg) of secondary material incorporated into the product to which the declaration applies at manufacture (lambda component of material in the ‘use of secondary materials’ flow);

IV = production inventory (per kg) of virgin material used to produce the product to which the declaration applies, in the absence of any recycling of the material;

IS = inventory (per kg) of production of the ready-to-use secondary material used to produce the product to which the declaration applies.

2. In the case of energy recovery, for a given flow of recycled waste, the following formula applies:

ChargeNetValoNRJ = MSval (ISval – PCISval x REval x IVval),

where

MSval = mass (in kg) of secondary material actually recovered from the collected mass of stock (lambda component(s) of “energy recovery materials” flow material);

IVval = production inventory (per kg) of the energy for which the energy recovered by energy recycling of the secondary materials is substituted in the downstream system (i.e. not the production inventory of the downstream system to which the secondary material will contribute, but the production inventory of energy consumed by the downstream system, expressed in MJ delivered);

ISval = inventory (per kg) of energy recycling of the secondary material from stock, for which the reference flow is the quantity of recycled secondary material;

PCISval = IBD (MJ/kg) of secondary material recovered;

REval = energy output of the recovered secondary material energy recycling installation.

The overall benefit for the product declared is then obtained as the sum of all the collective benefits calculated for each of the materials/flows.

Annex IV

Framework for the validity of collective environmental declarations for products

1. Homogeneity

A life cycle analysis (LCA) involves a large amount of data for numerous scenarios. Certain data are collected on-site, some are calculated and some are estimated. The data supplying an LCA are therefore data with a certain level of uncertainty. Thus the result of an LCA is fairly sensitive to variations in input data.

Article 9 of the Order requires that the environmental impacts of registrants using the same collective environmental declaration be homogeneous. Thus, a collective environmental declaration will cover environmentally homogeneous products if it is possible to ensure that all products covered declare environmental impacts below a limit value if they comply with the validity framework associated with this collective environmental declaration.

It is accepted that this homogeneity should only be demonstrated for certain control indicators.

For LCA indicators, the demonstration of homogeneity is carried out through a sensitivity analysis of uncertain parameters and parameters varying from one registrant to another. It is strongly recommended that this analysis be carried out early in the LCA process and that the collective environmental declaration be built on the basis of this analysis.

2. Sensitivity analysis

An LCA sensitivity analysis must involve the following stages:

a) Selection of control indicators.

b) Identification of sensitive parameters: study of contributors to identify input parameters that contribute most to explaining the value of control indicators (results of the LCA).

c) Determining the ranges of variation in the sensitive parameters (limit parameters and any statistical distribution).

d) Parameterised simulations based on stages b and c.

The sensitivity analysis may be iterative. The result of the sensitivity analysis must be:

a) A list of factors that influence the results of LCA (sensitive parameters) and their authorised fields of variation (this list is the validity field).

b) for each control indicator, a probable variation interval (95% confidence interval) of the indicator values obtained by the parameter simulations.

Stage 1: selection of control indicators

The selection of control indicators must be reasonable. They may be chosen on a case-by-case basis from those referred to in Article 3 of the Order. Their selection must be justified. The following indicators should be studied as a minimum:

- global warming;

- use of non-renewable primary energy, excluding non-renewable primary energy resources used as raw materials;

- non-hazardous waste disposed of.

Stage 2: identification of sensitive parameters

An analysis of sensitive parameters should be carried out for each control indicator. To this end, it is proposed that, as an initial approach, the impacts be divided according to processes included in the life cycle. This analysis should be based on the reference scenario described in the sensitivity analysis.

It is recommended that consideration be given to all processes with a contribution to the appearance of more than 5% in the baseline scenario. The parameters of the LCA model (entering or leaving the process, internal process modelling parameter, etc.) that change this contribution should then be identified. These are the parameters considered sensitive.

At the end of this analysis, the registrant will have a list of the processes that contribute most to the control indicators throughout the entire life cycle and a list of sensitive parameters.

Stage 3: definition of ranges of variation of sensitive parameters

This stage consists of defining the range of variation of each sensitive parameter. This range should then be proposed as a minimum in the form of intervals. If known, a law may be provided for the distribution of the parameter in the interval defined above.
Following this stage, each sensitive parameter is associated with a range of variation.

Stage 4: Calculating the variability of indicators

Based on Stages 2 and 3, this stage involves using an appropriate mathematical method to determine the ranges of variation of the indicators (maximum values, mean values and minimum values) when subjecting the LCA model to variations in sensitive parameters.

The result of stage 4 is a set of ranges of values taken by each of the indicators covered by the sensitivity analysis.

The calculation of the variability of indicators shall at least cover sensitive parameters relating to the elements referred to in Article 9 of this Order.

Stages 3 and 4 can be performed by iteration to adapt the range of sensitive parameters to the homogeneity conditions required.

Sensitivity analysis report

The report shall contain the items corresponding to the four stages of the sensitivity analysis and the final outcome of the sensitivity analysis, including the scope of validity of the collective environmental declaration and the ranges of variation of the control indicators.

3. Sensitivity analysis and environmental values declared

- The results of the sensitivity analysis condition the environmental values reported in the collective environmental declaration as specified in section 9 of this Order.

- When the maximum value (or maximum at 95% confidence, if a statistical approach is used) of the range of variation of each environmental impact control indicator is less than or equal to the mean value of the indicator multiplied by a limit value (specified in Article 9 of this Order), then the values of the declared environmental impact indicators shall be calculated with the mean values of the sensitive parameters.

- If not, i.e. if at least one of the environmental impact control indicators has a maximum value greater than the average value of the indicator multiplied by a limit value (specified in Article 9 of this Order), then the upper limit must be declared. This upper limit corresponds to the value of the indicators obtained when the maximum values (or maximum with a probability of 95%, if a statistical approach is used) of the sensitive parameters are used.

- 4. Precise identification of products covered by a declaration

To determine whether a product is likely to be covered by a collective environmental declaration, it needs to be established whether the product is similar to the standard covered product and whether the registrant of that product is authorised to use this collective environmental declaration.

Identification of the standard product

The standard product must be correctly defined and described in order to facilitate reconciliation between the description of a product and that of the standard product. The description of the standard product must include the following as a minimum:

a) A list of the main components or predominant materials.

b) Information on the functionality or level of performance.

This identification must be sufficient to find out quickly and unequivocally whether a particular product may be covered by the collective declaration.

Identification of registrants who may use the collective environmental declaration

Since the collective environmental declaration is the property and responsibility of a community, that community may decide that only some registrants can use this collective environmental declaration.

A list of registrants authorised to use a collective environmental declaration must be provided:

- either in the form of an exhaustive list of names;

- or in the form of a condition of membership of a collective entity (association, trade union, signatories to a charter of good practice, etc.). In this case, the list of members of that community must be publicly available.

5. Content of the framework of validity

The framework of validity contains as a minimum:

- Identification of the standard product (mandatory);

- Products covered by the collective environmental declaration;

- Identification of registrants who can use the collective environmental declaration;

- The sensitivity study report including the range of validity and providing evidence that the declared values of the indicators are homogeneous.

6. Use of the framework of validity

In order for a registrant to use a collective environmental declaration for his or her product, he or she must provide evidence:

- That their product complies with the ‘standard product’ covered by the collective environmental declaration;

- It is on the list of registrants authorised for this collective environmental declaration;

- That it complies with the range of validity of the collective environmental declaration in question.